3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213019

# Electronic Meeting, August 15- August 26, 2022

Agenda item:	11.16.3

Source: 	vivo

Title: 	Discussion on the potential impact on further NR mobility enhancements for MR-DC/CA

Document for:	Discussion

- Introduction

According to the latest discussions from RAN#96e meeting, the objectives in new WID on further NR mobility enhancements is agreed [1]. For the MR-DC/CA, it mainly includes CHO enhancement and SCG selective activation. For reference, we provide the related objectives below.

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2. - To specify CHO including target MCG and target SCG [RAN3, RAN2]. Note 5: This is already being targeted for Rel-17, so this objective will be reviewed at RAN#97-e. - To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline - To specify RRM core requirements for the following, as necessary [RAN4]: - L1/L2-based inter-cell mobility - Enhanced CHO configurations addressed by this WI

Based on the above highlight bullets, we would like to provide general analysis of the potential RRM impact on further NR mobility enhancements for MR-DC/CA.

- Discussion

2.1 Enhanced Conditional handover

Conditional handover and Conditional PSCell addition/change have been fully discussed in R16/R17 and their respective corresponding requirements also have been specified in the earlier meetings. In release 18, RAN2 will focus on specifying mechanisms for CHO and MR-DC to be configured simultaneously.

From RAN4 RRM perspective, we should consider the RRM impact including specifying delay requirement according to the scenarios and procedures to be defined from RAN2. Due to the parallel discussion in RAN2 and RAN4 on mobility enhancement in the first meeting, RRM is lack of the latest solutions and details for CHO and CPAC from RAN2. Therefore, we would like only to list some potential cases which are possible to impact RRM requirements:

- Case 1: CHO and CPAC are triggered within a time gap (consider parallel processing for PSCell and PCell during Tprocessing)

Figure 1-a                                        Figure 1-b

This scenario is considered to support CHO and CPAC are triggered within a time gap. After receiving and decoding RRC command, UE begin to evaluate CHO and CPAC. Due to the different delay uncertainty and different measurement delay between CHO and CPAC, there may have little possibility to triggering CHO and CPAC at the same time. Therefore, the further extension of the assumption would be considered here. We assume that PCell/PSCell meet their corresponding conditions and execute CHO/CPAC within a time gap. As we can see in the Figure 1-a and Figure 1-b above, whichever MCG or SCG satisfy its condition first, this cell group needs to wait until another cell group satisfy its condition of execution as well. The reason why needs a wait is making CHO and CPAC to do software processing and do RF retuning during the same time gap in Tprocessing. This procedure can refer to the current delay requirement on handover with PSCell for the parallel processing scenario. Besides, there may still have no candidate PSCell satisfy pre-configured conditions when up to the max length of the waiting time. Let PCell execute subsequent CHO procedures is an optional alternative. For this alternative, the current requirement on CHO can be reused directly.

- Case 2: CHO is executed first and CPAC is executed later (consider to perform CHO and CPAC separately)

In this case, it considers CPAC can be triggered at any time after CHO is executed without waiting to parallel processing for CHO and CPAC at the same time in Tprocessing. One thing in this case needs to be noted is, when PCell or PSCell is in Tprocessing, there maybe have an impact on another Cell which is executed CHO or CPAC procedure. Then the corresponding delay requirement may be different from the current delay requirement on CHO/CPAC.

- Case 3: Only CHO is executed if no PSCell satisfy its corresponding condition

In this case, it will evaluate whether PSCell satisfies condition when UE realizes the condition of PCell handover is met. Different from case 1, if there has no PSCell satisfies, only executing conditional handover is considered rather than waiting for a while until PSCell also meets its pre-configured conditions. For this case, the current delay requirement on conditional handover could be considered to be reused.

In the analysis above, we provide the potential impact for RRM in possible cases. From RRM perspective, the details need to wait RAN2’s progress on this issue. The RRM requirements of CHO and CPAC will be discussed until RAN2 has specified concrete scenarios and procedures.

Proposal 1: The RRM requirements of CHO and CPAC will be discussed until RAN2 has specified concrete scenarios and procedures.

2.2 SCG Selective activation

In the legacy conditional PSCell addition and change, the UE which is configured CPA/CPC should release the CPA/CPC configurations when acquiring the first available PRACH occasion in the target PSCell. This will increase the signalling overhead and the delay of PSCell change. Therefore, the subsequent CPA/CPC without re-configuration and re-initialization will be introduced in Release 18. From RAN4 perspective, RRM needs to analyse the potential impacts in supportive scenarios to be specified from RAN2. We would like to provide some initial analysis on the possible scenarios and their corresponding impacts on RRM.

For SCG selective activation, there are scenarios at least should be considered in RAN4：

- Scenario 1: if any CPAC configuration has been changed before performing subsequent CPAC

- Scenario 2: if CPAC configurations have not been changed before performing subsequent CPAC

Regarding to ‘whether CPAC configuration has been changed or not’, what the ‘CPAC configuration’ here means is whether the candidate PSCell and their corresponding execute conditions. When considering there have some CPAC configurations been changed before performing the next CPAC, the previous configurations could not be reused and CPC/CPA would be reconfigured and reinitialized. In this scenario, the current CPAC delay requirements can be reused. Whereas, for the scenario 2, there is no new CPAC configurations need to be renew and CPAC could be triggered without re-configuration and re-initialization on the preparation from network so that it may mean the whole CPAC delay would be shortened to some extent.

In the above, we provide some analysis on the two possible scenarios. As we can see the subsequent CPAC may has some effect on the current CPAC delay requirement. From RRM perspective, the specific impact depends on RAN2’ conclusions.

Proposal 2: For subsequent CPAC, whether there are concrete effects on the current CPAC delay requirements depend on RAN2’s conclusions.

- Conclusions

Proposal 1: The RRM requirements of CHO and CPAC will be discussed until RAN2 has specified concrete scenarios and procedures.

Proposal 2: For subsequent CPAC, whether there are concrete effects on the current CPAC delay requirements depend on RAN2’s conclusions.

- Reference

[1] RP-221799 Revised WID on Further NR mobility enhancements, MediaTek Inc