3GPP TSG RAN WG4 Meeting #104-e	R4-2213048

Electronic Meeting, August 15 - August 26, 2022

Agenda item:			9.21.1.1

Source:	vivo

Title:	Remaining issues for PDC enhancement

Document for:	Discussion

- Introduction

In the RAN4#103-e meeting, RRM core requirements for propagation delay compensation enhancements were further discussed. The agreements and remaining open issues were captured in WF [1].

Issue 1-2: RAN4 waits for RAN1 conclusion on the applicable capabilities for PRS based PDC? - RAN4 keep the impacted parameters in [] until RAN1 has reached agreement. RAN4 can additionally add an editor’s note: ‘Pending agreement on UE processing capability discussion’ Issue 1-3: The number of samples assumed for deriving the accuracy requirements [1 or 4]? - Option 1: samples assumed for deriving the accuracy requirements: 1 - Option 2: samples assumed for deriving the accuracy requirements: 4 Issue 1-4: Is there a need to introduce scaling factor Kgap when the PDC resources occasions collide with MG? - Option 1: Introduce scaling factor Kgap when the PDC resources occasions collide with legacy MG only - Option 3: No requirements defined for when PDC resources collide with gaps - Clarification on the requirements applicable conditions need to included into specification for this option Issue 1-5: Is there a need for Rx beam sweeping factor if no QCL information is provided for the RS? - If no QCL information is provided for the PDC RS it is assumed that UE Rx beam sweeping is needed for PDC measurements. - RAN4 only define requirement conditioned: - QCL information is provided - UE Rx sweeping is not needed Issue 1-6: Is there a need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH? - Option 1: Yes, there is there a need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH. - Option 2: Other Agreement in GTW May 18th: Option 1 agreed Issue 1-7: Is there a need for defining measurement restriction if PDC RS is not QCL-ed with L1 RS? - Option 1: Yes, there is a need for defining measurement restriction if PDC RS is not QCL-ed with L1 RS. - Option 2: Other Agreement in GTW May 18th: Option 1 agreed Issue 1-8: Is there a need to define DRX requirements for PDC measurement? - Option 1: Yes, define DRX requirements for PDC such that UE is assumed to take one sample per DRX cycle. - Option 2: Other Agreement in GTW may 18th: FFS whether define DRX requirements for PDC measurement

In this contribution, we further provide our views on the remaining open issues of core requirements for PDC enhancement.

- Discussion

UE capabilities

Issue 1-2: RAN4 waits for RAN1 conclusion on the applicable capabilities for PRS based PDC? - RAN4 keep the impacted parameters in [] until RAN1 has reached agreement. RAN4 can additionally add an editor’s note: ‘Pending agreement on UE processing capability discussion’

The updated UE capabilities for RTT-based PDC are as follows.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. |
| --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | [2-51, 2-53] | Yes | N/A |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | 1. Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS 2. Max number of DL PRS Resources in DL PRS Resource Set for PDC Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 3. Max number of DL PRS resources that UE can process in a slot. a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | [25-19, 2-53] | Yes | N/A |

There is no UE capability of {N, T}, which is introduced for PRS based positioning, for RTT-based PDC based on PRS and SRS. The RTT-based PDC is based on measurements on one serving cell and within active BWP. Thus, measurement period requirements for PRS based UE Rx-Tx time difference measurement should be similar as for TRS.

Proposal 1: Measurement period requirements for PRS based UE Rx-Tx time difference measurement is TUERxTx,PRS=Nsample *TPRS .

Number of samples for TRS based UE Rx-Tx time difference measurement

Issue 1-3: The number of samples assumed for deriving the accuracy requirements [1 or 4]? - Option 1: samples assumed for deriving the accuracy requirements: 1 - Option 2: samples assumed for deriving the accuracy requirements: 4

For RTT-based PDC with TRS, it can be seen from simulation results in [2] that measurement accuracy with 4 samples and 1 sample are almost the same under AWGN channel. Since PDC is measurement on serving cell and TRS should be in the active BWP, there is no AGC issue for TRS based UE Rx-Tx time difference measurement. Therefore, there is no need to define core and accuracy requirements under AWGN with 4 samples.

On the other hand, defining core and accuracy requirements based on 4 samples may provide robust and accuracy measurement results under fading channels. Though the requirements are specified under AWGN, in practical it could be fading channels in most of the scenarios.

Proposal 2: Accuracy requirements for RTT-based PDC with TRS are based on measurements with 4 samples under AWGN channel.

Scaling factor

Issue 1-4: Is there a need to introduce scaling factor Kgap when the PDC resources occasions collide with MG? - Option 1: Introduce scaling factor Kgap when the PDC resources occasions collide with legacy MG only - Option 3: No requirements defined for when PDC resources collide with gaps Clarification on the requirements applicable conditions need to included into specification for this option

With the requirements are specified based on measurements with 4 samples, there would be no need to specify requirements for the case when PDC resource occasions collide with MG. UE can take samples outside of measurement gap. It would be quite complicated if PDC measurement are coupled with other RRM measurements, and even with L1 measurements. There should be no requirements if PDC resources occasions are fully collided with measurement gaps, i.e., it is invalid case.

Proposal 3: No requirements are defined for the case when PDC resources are collided with gaps.

DRX

Issue 1-8: Is there a need to define DRX requirements for PDC measurement? - Option 1: Yes, define DRX requirements for PDC such that UE is assumed to take one sample per DRX cycle. - Option 2: Other Agreement in GTW may 18th: FFS whether define DRX requirements for PDC measurement

In our view, DRX can be considered to when defining requirements for PDC measurement for the purpose of power saving. However, it may also have impact to PDC functionality as propagation delay may not be compensated timely. Our preference is to define PDC measurement requirements by taking DRX into consideration.

Proposal 4: For PRS/TRS based UE Rx-Tx time difference measurement for TT-based PDC, DRX related requirements are specified with the assumption that UE measures once per DRX cycle.

- Summary

In this contribution, we further provided our views on the remaining open issues of core requirements for PDC enhancement. Based on analysis following observations and proposals are present.

Proposal 1: Measurement period requirements for PRS based UE Rx-Tx time difference measurement is TUERxTx,PRS=Nsample *TPRS .

Proposal 2: Accuracy requirements for RTT-based PDC with TRS are based on measurements with 4 samples under AWGN channel.

Proposal 3: No requirements are defined for the case when PDC resources are collided with gaps.

Proposal 4: For PRS/TRS based UE Rx-Tx time difference measurement for TT-based PDC, DRX related requirements are specified with the assumption that UE measures once per DRX cycle.

- References

- R4-2210618	WF on NR_IIOT_URLLC_enh, Nokia

- R4-2208821	Updated simulation results for TRS based PDC, vivo