3GPP TSG-RAN WG4 Meeting #104-e	R4-2213064

Electronic Meeting, August 15 – 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on remaining RRM issues for RedCap UEs

Agenda item:	9.18.3.1.1	General 	[NR_redcap-Core]

Document for:	Discussion and Decision

Introduction

In the last RAN4 #103-e meeting, the following items related to general RRM core requirements for RedCap UE were discussed without consensus [1].

8.1.1 SDT+ eDRX TA validation requirements when UE is configured with eDRX was discussed but no conclusions were reached. Interested companies may provide their view at next meeting. 8.1.1 Neighbour cell relaxation when UE has failed to meet cell suitability criterion Following highlighted text to capture in the IDLE mode CR was discussed but not agreed. Interested companies may provide their view at next meeting: “If the UE is not configured with eDRX_IDLE cycle and the UE has evaluated according to Table 4.2B.2.2-1 for 1 Rx RedCap or Table 4.2.2.2-1 for 2 Rx RedCap in Nserv_RedCap consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities. In this case the UE shall not relax measurements on any of the neighbour cells even if the UE is configured with any relaxed measurement criterion and has fulfilled that criterion.”

In this contribution we present Nokia’s views regarding:

- TA validation for SDT and eDRX,

- TA validation for SDT in FR2, and

- Neighbour cell relaxation when UE has failed to meet cell suitability criterion.

Discussion

TA validation requirements for small data transmission 	and eDRX

At RAN4#103-e, the discussion on TA validation requirements for small data transmission for RedCap UE was progressed in the endorsed draft CR [2]. The impact of eDRX was discussed but no specific requirements for eDRX were agreed to be defined.

In our view, TA validation requirements for SDT should not depend on whether the UE is configured with or without eDRX. Thus, the same timing requirements for ensuring the two RSRP measurements are appropriate related to the two instants, i.e., reception of last TA command and TA validation before transmitting on preconfigured uplink resources, should apply. Hence, we do not see any justification to specify different TA validation requirements when eDRX is configured.

The following proposal is made.

Do not specify different TA validation requirements when eDRX is configured and hence reuse the requirements specified for the configuration without eDRX.

TA validation requirements for small data transmission in 	FR2

At RAN4#103-e, the discussion on TA validation requirements for small data transmission for RedCap UE was progressed in the endorsed draft CR [2] which specifies the requirements for FR1 but leaves them open for FR2, see the extract of TS 38.133 in Figure 1.

Figure 1: Extract from clause 5.2B.2.1 in TS 38.133 V17.6.0

From the specification, the term X1 above is not yet specified. Thereagainst, TA validation requirements for SDT for legacy NR devices are depicted in the extract of TS 38.133 in Figure 2.

Figure 2: Extract from clause 5.5.3 in TS 38.133 V17.6.0

Comparing TA validation requirements for SDT for RedCap UE and for legacy NR devices, we observe that they are identical for FR1, but different for FR2. In our view, FR2 requirements for RedCap UE should also be aligned to legacy NR devices. Such timing requirements for RSRP measurements, in order to prepare for TA validation, should indeed be identical as the TA validation refers to mobility behaviour of the UE. Hence, we propose to reuse the FR2 requirements for legacy NR devices defined in clause 5.5.3 for RedCap UE in clause 5.2B.2.1.

The following proposal is made.

Reuse the FR2 requirements for SDT for legacy NR devices defined in clause 5.5.3 for RedCap UE in clause 5.2B.2.1.

Neighbour cell relaxation when UE has failed to meet cell 	suitability criterion

At RAN4#103-e, the discussion on idle mode requirements for RedCap UE was progressed in the endorsed draft CR [3].

The wording in clause 4.2B.2.2 according to agreed changes in [3] that in case the UE evaluates

“in Nserv_RedCap consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.”,

is in fact quite generic referring to measurement rules, rather than measurement configuration, and the clarification discussed at RAN4#103-e

“In this case the UE shall not relax measurements on any of the neighbour cells even if the UE is configured with any relaxed measurement criterion and has fulfilled that criterion.”,

is appropriate in our view referring to the case of neighbour cell measurement relaxation introduced in clauses 4.2B.2.9 to 4.2B.2.11. As the serving cell suitability criterion is not met, we prefer that the UE measures, at the earliest, configured neighbour cells in order to identify suitable candidates for cell reselection. Thus, any measurement relaxation based on one or more measurement relaxation criteria being satisfied, is inappropriate in such case.

The following proposal is made.

Add the phrase: “In this case the UE shall not relax measurements on any of the neighbour cells even if the UE is configured with any relaxed measurement criterion and has fulfilled that criterion.”, for the cases configured eDRX and non-configured eDRX in clause 4.2B.2.2 in TS 38.133.

Conclusion

Remaining issues related to general RRM core requirements for RedCap UE are discussed in this contribution.

Following proposals are made.

- Do not specify different TA validation requirements when eDRX is configured and hence reuse the requirements specified for the configuration without eDRX.

Reuse the FR2 requirements for SDT for legacy NR devices defined in clause 5.5.3 for RedCap UE in clause 5.2B.2.1.

Add the phrase: “In this case the UE shall not relax measurements on any of the neighbour cells even if the UE is configured with any relaxed measurement criterion and has fulfilled that criterion.”, for the cases with and without configured eDRX in clause 4.2B.2.2 in TS 38.133.

- References

- R4-2210592, “WF on RedCap RRM requirements”, RAN4#103-e, Ericsson

- R4-2211039, “Draft CR on SDT requirements for RedCap for TS 38.133”, RAN4#103-e, Ericsson

- R4-2211038, “Draft CR: Changes to RedCap IDLE mode requirements for TS 38.133”, RAN4#103-e, Ericsson