3GPP TSG-RAN WG4 Meeting # 104-e 		R4-2213066

# Electronic Meeting, August 15 – 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	On RedCap UE demodulation and CSI reporting requirements

Agenda item:	9.18.5

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF on UE demodulation and CSI requirements for RedCap [1] are captured in the summary [2] and WF [3].

In this contribution we will express our views on the open issues related to configurations for UE demodulation and CSI reporting requirements for RedCap, namely:

- RedCap operation in FR2 operating bands, and

- selection of MCS for 256QAM for RedCap 1Rx.

Discussion

RedCap operation in FR2 operating bands

At RAN4 #103-e, the frequency range FR2-2 was introduced in RF specifications TS 38.101 and TS 38.104 for Rel-17. Figure 1 depicts an extract of TS 38.101-2, V17.6.0 depicting the approved changes.

Figure 1: Extract from TS 38.101-2, V17.6.0

The FR2-2 frequency range contains the operating band n263 specified for unlicensed operation. As RedCap is for licensed operation, FR2-2 is not applicable to RedCap.

The following proposal is made:

Specify RedCap operation for FR2-1 only.

Selection of MCS for 256QAM for RedCap 1Rx

WF ‎[3] contains the agreement to introduce 256QAM requirement for RedCap 2Rx. Regarding RedCap 1Rx, three options were discussed without conclusion:

For 256QAM requirements for 1Rx, - Option 1: Define 256QAM demodulation requirements for 1Rx in FR1 - Option 1a: MCS20 - Option 1b: MCS is based on the operating SNR - Option 2: Not to define 256QAM demodulation requirements for 1Rx in FR1

Based on the fact, that 256QAM is optional feature for RedCap UE (both 1Rx and 2Rx), as stated during the RAN4 #103-e email discussion [2], we think it can be beneficial to support 256QAM to increase throughput in case of good SNR conditions. However, as 1 Rx UE will experience lower SNR compared to 2Rx, it is crucial to verify performance with robust MCS. Among the options discussed for the MCS at RAN4 #103-e were: MCS20, MCS22 and MCS24, the latter being used for 256QAM for legacy UE’s with 2Rx.

The summary on coding rate and spectral efficiency for the different MCS’s is listed in Table 5.1.3.1-2 in TS 38.214, clause 5.1.3.1:

Table 5.1.3.1-2: MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

According to Table 5.1.3.1-2, MCS20 is the most robust scheme for 256QAM support providing target code rate of 0.67 compared to 0.74 for MCS22 and 0.82 for MCS24. Hence, it is proposed to select MCS20 for 256QAM support for RedCap 1Rx.

The following proposal is made:

Select MCS20 for 256QAM support for RedCap 1Rx UE.

Conclusion

In this contribution we have provided our views on open issues related to configurations for UE demodulation and CSI reporting requirements for RedCap.

We have made the following proposals:

- Specify RedCap operation for FR2-1 only.

- Select MCS20 for 256QAM support for RedCap 1Rx UE.

- References

- R4-220966, "Revised WID on support of reduced capability NR devices", Ericsson, RAN #95e

- R4-2210532, "Email discussion summary for [103-e][329] NR_RedCap_Demod", Moderator (Ericsson)

- R4-2210672, "WF on RedCap demodulation and CQI reporting requirements", Ericsson