3GPP TSG-RAN WG4 Meeting # 104-e 		R4-2213070

# Electronic Meeting, August 15 – 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Discussion on RedCap CQI reporting issues under static condition

Agenda item:	9.18.5.2.1

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF on UE demodulation and CSI requirements for RedCap [1] are captured in the WF [2] and summary [3].

In this contribution we will express our views on the open issues related to CQI reporting requirements for RedCap, namely:

- definition of supported CSI-RS configurations for CQI reporting tests under static condition,

- selection of overhead for TBS determination for RMCs for CQI reporting under static condition,

- definition of RMCs for CQI reporting under static condition,

- selection of SNR intervals for CQI reporting tests under fading condition.

Discussion

Supported CSI-RS configurations for CQI reporting tests under static condition

TS 38.101-4 depicts the CSI-RS configurations for CQI reporting tests under static condition for legacy NR devices, namely:

- ZP CSI-RS configuration

- NZP CSI-RS for CSI acquisition

- CSI-IM configuration

In our view, the same CSI-RS configurations should be supported and hence tested for RedCap UE.

The correspondent parameters for the CSI-RS configurations are same for RedCap except the CSI-RS periodicity and offset, which were changed to 10/1 according to WF [2].

The following proposal is made:

Reuse existing CSI-RS configurations for CQI reporting tests for RedCap.

Overhead for TBS determination for RMCs for CQI reporting under static condition

Regarding the definition of Reference Measurement Channels (RMCs) for CQI reporting, existing CSI RMCs are based on the overhead parameter for TBS determination, corresponding to the higher layer parameter xOverhead in PDSCH-ServingCellconfig according to TS 38.214, which may be 6, 12 or 18 per RB, if configured. Existing RMCs for CSI reporting mostly contain the value 0 for the overhead, i.e. the higher layer parameter is not configured. This setting is reused for RedCap RMCs for CQI reporting under static condition.

The following proposal is made:

Reuse the value 0 for the overhead parameter for TBS determination from existing CSI RMCs for CQI reporting under static condition.

RMCs for CQI reporting under static condition

For CQI reporting under static condition, new RMCs are required in FR1 due to BW restrictions of 20 MHz for RedCap UE. CQI reporting requirements are based on CQI table 1 according to WF [2], as existing CSI RMC for CQI table 1 based on 66 RBs cannot be reused, as being too large.

For FDD and HD-FDD, existing BW of 10 MHz is reused, whilst for TDD BW of 20 MHz is defined. This yields following RMC’s for 1Rx and 2Rx:

- For FDD and HD-FDD, 52 RBs are transmitted in 10 MHz and hence TBS.1-X1 (single layer) and TBS.1-X2 (dual layers) are the new RMCs for FDD and HD-FDD in FR1 for RedCap, as shown below in blue.

- For TDD, 51 RBs are transmitted in 20 MHz and hence TBS.1-X3 (single layer) and TBS.1-X4 (dual layers) are the new RMCs for TDD in FR1 for RedCap as shown below in blue.

The following proposal is made:

Adopt new RMCs for CQI reporting under static condition in FR1 based on 52 and 51 RBs as depicted in the table below:

Table A.4-1: Mapping of CQI Index to Information Bit payload (CQI table 1)

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 | TBS.1-X1 | TBS.1-X2 | TBS.1-X3 | TBS.1-X4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 | 52 | 52 | 51 | 51 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 6 | 6 | 0 | 0 | 0 | 0 |
| Available RE-s |  |  |  | 7590 | 7590 | 6240 | 6240 | 6120 | 6120 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A | N/A | N/A |
| 1 | 0.2344 | 0 | QPSK | 1800 | 3624 | 1480 | 2976 | 1480 | 2856 |
| 2 | 0.2344 | 0 |  | 1800 | 3624 | 1480 | 2976 | 1480 | 2856 |
| 3 | 0.3770 | 2 |  | 2856 | 5640 | 2408 | 4744 | 2408 | 4616 |
| 4 | 0.6016 | 4 |  | 4480 | 8968 | 3752 | 7424 | 3752 | 7296 |
| 5 | 0.8770 | 6 |  | 6528 | 13064 | 5504 | 11016 | 5376 | 10760 |
| 6 | 1.1758 | 8 |  | 8712 | 17928 | 7296 | 14600 | 7168 | 14344 |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 | 9224 | 18432 | 8968 | 17928 |
| 8 | 1.9141 | 13 |  | 14344 | 28680 | 12040 | 24072 | 11784 | 23568 |
| 9 | 2.4063 | 15 |  | 17928 | 35856 | 15112 | 30216 | 14600 | 29192 |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 | 16896 | 33816 | 16896 | 33816 |
| 11 | 3.3223 | 20 |  | 25104 | 50184 | 20496 | 40976 | 20496 | 40976 |
| 12 | 3.9023 | 22 |  | 29192 | 58384 | 24576 | 49176 | 24072 | 48168 |
| 13 | 4.5234 | 24 |  | 33816 | 67584 | 28168 | 56368 | 27656 | 55304 |
| 14 | 5.1152 | 26 |  | 38936 | 77896 | 31752 | 63528 | 31240 | 62504 |
| 15 | 5.5547 | 28 |  | 42016 | 83976 | 34816 | 69672 | 33816 | 67584 |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-1 of TS 38.214 [12] Note 5: TBS.1-3, TBS.1-4, TBS.1-5 and TBS.1-6 are applicable to RedCap UE. |  |  |  |  |  |  |  |  |  |

Thereagainst, for FR2-1, where RedCap UE has 2Rx, legacy FR2 requirements based on the configured BW of 100 MHz in TS 38.101-4, clause 8.2.2.2.1, can be reused, and the corresponding CSI RMC specified in Table A.4-1, TBS.1-2, can be reused as well.

The following proposal is made:

Reuse existing test for CQI reporting under static condition in clause 8.2.2.2.1 and corresponding CSI RMC for RedCap UE in FR2-1.

SNR intervals for CQI reporting tests under fading condition

WF ‎[2] contains following agreements and open issues on SNR intervals for CQI reporting under fading condition:

Agreement: Test points for CQI reporting test in fading condition for 2Rx - Lower SNR test points - Option 1: SNR=5/6 dB - Option 2: SNR=6/7 dB - Higher SNR test points: - SNR=12/13dB Agreement: Test points for CQI reporting test in fading condition for 1Rx - Set SNR test points 3 dB higher than 2Rx case

For the lower SNR test points, two options were discussed for 2Rx: 5/6 dB and 6/7 dB. As we do not plan to submit simulation results for this meeting, we propose to proceed selecting lower SNR test points for fading condition as contributed by other companies to this meeting.

The following proposal is made:

For the lower SNR test points for CQI reporting under fading condition, select the test points according to the majority view of contributing companies.

Conclusion

In this contribution we have provided our views on open issues related to related to CQI reporting requirements for RedCap.

We have made the following proposals:

- Reuse existing CSI-RS configurations for CQI reporting tests for RedCap.

- Reuse value 0 for the overhead parameter for TBS determination from existing CSI RMCs for CQI reporting under static condition.

- Adopt new RMCs for CQI reporting under static condition in FR1 based on 52 and 51 RBs as depicted in the table below:

Table A.4-1: Mapping of CQI Index to Information Bit payload (CQI table 1)

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 | TBS.1-X1 | TBS.1-X2 | TBS.1-X3 | TBS.1-X4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 | 52 | 52 | 51 | 51 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 6 | 6 | 0 | 0 | 0 | 0 |
| Available RE-s |  |  |  | 7590 | 7590 | 6240 | 6240 | 6120 | 6120 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A | N/A | N/A |
| 1 | 0.2344 | 0 | QPSK | 1800 | 3624 | 1480 | 2976 | 1480 | 2856 |
| 2 | 0.2344 | 0 |  | 1800 | 3624 | 1480 | 2976 | 1480 | 2856 |
| 3 | 0.3770 | 2 |  | 2856 | 5640 | 2408 | 4744 | 2408 | 4616 |
| 4 | 0.6016 | 4 |  | 4480 | 8968 | 3752 | 7424 | 3752 | 7296 |
| 5 | 0.8770 | 6 |  | 6528 | 13064 | 5504 | 11016 | 5376 | 10760 |
| 6 | 1.1758 | 8 |  | 8712 | 17928 | 7296 | 14600 | 7168 | 14344 |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 | 9224 | 18432 | 8968 | 17928 |
| 8 | 1.9141 | 13 |  | 14344 | 28680 | 12040 | 24072 | 11784 | 23568 |
| 9 | 2.4063 | 15 |  | 17928 | 35856 | 15112 | 30216 | 14600 | 29192 |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 | 16896 | 33816 | 16896 | 33816 |
| 11 | 3.3223 | 20 |  | 25104 | 50184 | 20496 | 40976 | 20496 | 40976 |
| 12 | 3.9023 | 22 |  | 29192 | 58384 | 24576 | 49176 | 24072 | 48168 |
| 13 | 4.5234 | 24 |  | 33816 | 67584 | 28168 | 56368 | 27656 | 55304 |
| 14 | 5.1152 | 26 |  | 38936 | 77896 | 31752 | 63528 | 31240 | 62504 |
| 15 | 5.5547 | 28 |  | 42016 | 83976 | 34816 | 69672 | 33816 | 67584 |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-1 of TS 38.214 [12] Note 5: TBS.1-3, TBS.1-4, TBS.1-5 and TBS.1-6 are applicable to RedCap UE. |  |  |  |  |  |  |  |  |  |

- Reuse existing test for CQI reporting under static condition in clause 8.2.2.2.1 and corresponding CSI RMC for RedCap UE in FR2-1.

- For the lower SNR test points for CQI reporting under fading condition, select the test points according to the majority view of contributing companies.

The first four proposals are implemented in the companion Draft CR [4].

- References

- R4-220966, "Revised WID on support of reduced capability NR devices", Ericsson, RAN #95e

- R4-2210672, "WF on RedCap demodulation and CQI reporting requirements", Ericsson

- R4-2210532, "Email discussion summary for [103-e][329] NR_RedCap_Demod", Moderator (Ericsson)

- R4-2213072, "Draft CR 38.101-4 Channel quality reporting for RedCap under static condition", Nokia, Nokia Shanghai Bell, RAN4 #104-e