3GPP TSG-RAN WG4 Meeting # 104-e 		R4-2213071

# Electronic Meeting, August 15 – 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Simulation results for RedCap CQI reporting under static condition

Agenda item:	9.18.5.2.1

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF for UE demodulation and CSI requirements for RedCap [1] are captured in the summary [2] and WF [3].

In this contribution, we present a summary of our current CQI reporting simulation results based on the agreed configurations in [3].

CQI reporting simulation results summary

The agreed configurations for CQI reporting performance were revised in the last meeting and are depicted in [3].

It is noted that results presented in this section refer to static condition for FR1 only, are initial results and hence subject to change in the coming meetings.

CQI reporting performance under static condition

Simulation results for alignment are reported in this section for agreed simulation assumptions contained in the WF [3].

FDD 15kHz FR1

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse | SNR (dB) | CQI metric (%) | Median CQI | MCS (Median CQI) | BLER (Median CQI) | MCS (Median CQI+1) | BLER (Median CQI+1) | MCS (Median CQI-1) | BLER (Median CQI-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 with static channel (Annex B.1 in TS 38.101-4) | New | 5 dB | 99.75 | 7 | 11 | 0 | 13 | 1 |  |  |
|  |  |  |  |  |  |  |  | 6 dB | 100 | 8 | 13 | 0.987 |  |  | 11 | 0 |
|  |  |  |  |  |  |  |  | 11 dB | 99.0 | 10 | 18 | 0 | 20 | 1 |  |  |
|  |  |  |  |  |  |  |  | 12 dB | 96.95 | 11 | 20 | 0.055 | 22 | 1 |  |  |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 with static channel (Annex B.1 in TS 38.101-4) | New | 8 dB | 100 | 8 | 13 | 0 | 15 | 1 |  |  |
|  |  |  |  |  |  |  |  | 9 dB | 100 | 8 | 13 | 0 | 15 | 0.048 |  |  |
|  |  |  |  |  |  |  |  | 14 dB | 100 | 11 | 20 | 0.064 | 22 | 1 |  |  |
|  |  |  |  |  |  |  |  | 15 dB | 99.4 | 11 | 20 | 0.002 | 22 | 1 |  |  |

TDD 30kHz FR1

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse | SNR (dB) | CQI metric (%) | Median CQI | MCS (Median CQI) | BLER (Median CQI) | MCS (Median CQI+1) | BLER (Median CQI+1) | MCS (Median CQI-1) | BLER (Median CQI-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 1 | 2 x 1 with static channel (Annex B.1 in TS 38.101-4) | New | 5 dB | 99.55 | 7 | 11 | 0 | 13 | 1 |  |  |
|  |  |  |  |  |  |  |  | 6 dB | 99.95 | 8 | 13 | 0.9785 |  |  | 11 | 0 |
|  |  |  |  |  |  |  |  | 11 dB | 98.95 | 10 | 18 | 0 | 20 | 1 |  |  |
|  |  |  |  |  |  |  |  | 12 dB | 97.0 | 11 | 20 | 0.443 |  |  | 18 | 0 |
| 2Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 2 | 2 x 2 with static channel (Annex B.1 in TS 38.101-4) | New | 8 dB | 100 | 8 | 13 | 0.0005 | 15 | 1 |  |  |
|  |  |  |  |  |  |  |  | 9 dB | 100 | 8 | 13 | 0.0005 | 15 | 0.056 |  |  |
|  |  |  |  |  |  |  |  | 14 dB | 100 | 11 | 20 | 0.2865 |  |  | 18 | 0 |
|  |  |  |  |  |  |  |  | 15 dB | 99.4 | 11 | 20 | 0.001 | 22 | 1 |  |  |

Conclusion

In this contribution we have provided a summary of our initial CQI reporting simulation results under static condition in FR1 for agreed configurations in the WF [3] for alignment.

The following proposal is made.

Include CQI reporting results for alignment into the simulation summary.

- References

- R4-220966, "Revised WID on support of reduced capability NR devices", Ericsson, RAN #95e

- R4-2210532, "Email discussion summary for [103-e][329] NR_RedCap_Demod", Moderator (Ericsson)

- R4-2210672, "WF on RedCap demodulation and CQI reporting requirements", Ericsson