3GPP TSG-RAN WG4 Meeting # 104-e 		R4-2213073

# Electronic Meeting, August 15 – 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Discussion on RI requirements

Agenda item:	9.18.5.2.2

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF for UE demodulation and CSI requirements for RedCap [1] are captured in the summary [2] and WF [3].

In this contribution we will express our views on the open issues related to RI reporting requirements.

Discussion

Scope of RI reporting requirements

In WF ‎[3], two options were discussed:

Whether to define RI reporting requirements for RedCap 2Rx UEs - Option 1: Define RI reporting requirements - Option 2: Not define RI reporting requirements

As commented at RAN4 #102-e, not supporting MIMO operation is not one of the reduced capabilities of the RedCap UE.

MIMO operation is not one of the reduced capabilities of the RedCap UE.

Like PMI reporting, for which performance requirements and corresponding test cases have been agreed at RAN4 #102-e, RI is still part of Redcap UE mandatory capabilities, hence RI reporting needs to be tested. We potentially see a significant negative TP performance impact if no requirements for RI reporting are specified, or rather such functionality is not implemented to increase spectral efficiency.

In our view, it is important to verify 2 Rx RedCap UE reports the appropriate MIMO rank, i.e. it reports RI=2 in case of good channel conditions, as part of a viable CSI reporting functionality similar as a Rel-15/Rel-16 legacy NR device.

Thus, RI reporting requirements for 2 Rx RedCap UE need to be specified.

We can’t follow the argumentation from the last two meetings that 2 Rx should not be tested because 1 Rx is not tested, as RI reporting requirements for 1 Rx UEs do not make sense in the first place.

Thus, option 1 should be selected.

Define RI reporting requirements for 2 Rx RedCap UEs.

Specification impact

The impact of RI reporting to specification is discussed in this section.

Test cases

We note Moderator’s Recommended WF submitted during RAN4 #103-e [2]:

- Recommended WF (Moderator) [2]: - Check the candidate test case list below if RAN4 agree to define the RI reporting test according to the current TS38.101-4. - One compromised option is to apply Test 2 only to verify 2Rx UE reports rank 2. Freq range and duplex mode; Reference; CBW / SCS; CQI table; SNR test point; Metric; Antenna configuration; New / Reuse. FR1 FDD; Table 6.4.2.1-1 Test 1; 10MHz / 15KHz; Table 1; 0dB; Follow RI vs Fixed RI=2 ≥ 1.0; 2x2 ULA Low; New. Table 6.4.2.1-1 Test 2; 10MHz / 15KHz; Table 1; 20dB; Follow RI vs Fixed RI=1 ≥ 1.05; 2x2 ULA Low; New. Table 6.4.2.1-1 Test 3; 10MHz / 15KHz; Table 1; 20dB; Follow RI vs Fixed RI=1 ≥ 0.9; 2x2 ULA High; New. FR1 TDD; Table 6.4.2.2-1 Test 1; 20MHz / 30KHz; Table 1; 0dB; Follow RI vs Fixed RI=2 ≥ 1.0; 2x2 ULA Low; New. Table 6.4.2.2-1 Test 2; 20MHz / 30KHz; Table 1; 20dB; Follow RI vs Fixed RI=1 ≥ 1.05; 2x2 ULA Low; New. Table 6.4.2.2-1 Test 3; 20MHz / 30KHz; Table 1; 20dB; Follow RI vs Fixed RI=1 ≥ 0.9; 2x2 ULA High; New. FR2 TDD; Table 8.4.2.2-1 Test 1; 100MHz / 120KHz; Table 1; 0dB; Follow RI vs Fixed RI=2 ≥ 1.0; 2x2 ULA Low; Reuse. Table 8.4.2.2-1 Test 2; 100MHz / 120KHz; Table 1; 16dB; Follow RI vs Fixed RI=1 ≥ 1.05; 2x2 ULA Low; Reuse. Table 8.4.2.2-1 Test 3; 100MHz / 120KHz; Table 1; 16dB; Follow RI vs Fixed RI=1 ≥ 1.05; 2x2 XP High; Reuse.

We agree to the view, that RI reporting test effort should be reduced. Thus, we propose to limit RI reporting tests to Test 2 to verify rank 2 being reported.

Limit RI reporting tests to Test 2 for FR1 FDD/TDD and FR2 TDD.

This requires two new test cases for FR1, and the existing test case is reused for FR2.

Definition of RMC for RedCap RI reporting

Regarding the definition of RMC’s for RedCap RI reporting for FR1, the new defined RMC’s for RedCap CQI reporting under static condition in FR1 (FDD, TDD) based on CQI index from CQI table 1 can be applied [4].

Reuse the RMCs, defined for RedCap CQI reporting under static condition in FR1, for RI reporting in FR1.

Conclusion

In this contribution we have provided our views on RedCap UE RI reporting requirements.

We have made the following observation and proposals:

- MIMO operation is not one of the reduced capabilities of the RedCap UE.

- Define RI reporting requirements for 2 Rx RedCap UEs.

Limit RI reporting tests to Test 2 for FR1 FDD/TDD and FR2 TDD.

Reuse the RMCs, defined for RedCap CQI reporting under static condition in FR1, for RI reporting in FR1.

The companion Draft CR is provided in [5].

- References

- R4-220966, "Revised WID on support of reduced capability NR devices", Ericsson, RAN #95e

- R4-2210532, "Email discussion summary for [103-e][329] NR_RedCap_Demod", Moderator (Ericsson)

- R4-2210672, "WF on RedCap demodulation and CQI reporting requirements", Ericsson

- R4-2213072, "Draft CR 38.101-4, Channel quality reporting for RedCap under static condition", Nokia, Nokia Shanghai Bell

- R4-2213074, "Draft CR 38.101-4, Rank Indicator reporting for RedCap", Nokia, Nokia Shanghai Bell