3GPP TSG-RAN WG4 Meeting #104-e	R4-2213098

Electronic Meeting, 15 August – 26 August 2022

Source:	Ericsson, Telstra

Title:	TP for TR 37.718-21-11: Including band combinations DC_7A-26A_n78A

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-21-11 to add DC_7A-26A_n78A and DC_7C-26A_n78A.

# 2. Text Proposal

---Start of changes---

## 5.x	DC_7-26_n78

### 5.x.1	Operating bands for EN-DC

Table 5.x.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-26_n78 | CA_7-26 | n78 | No |

### 5.x.2	Configuration for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-26A_n78A DC_7C-26A_n78A | DC_7A-78A DC_26A-78A | CA_7A-26A | n78A |

### 5.x.3		∆TIB and ∆RIB values

For DC_7-26_n78, the TIB,c and RIB,c values are reused from DC_5-7_n78 and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-26_n78 | 7 | 0.6 |
|  | 26 | 0.6 |
|  | n78 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-26_n78 | 7 | 0.2 |
|  | 26 | 0.2 |
|  | n78 | 0.5 |

### 5.x.4	REFSENS requirements

There are IMD2 and IMD5 impact from UL 7_n78 affecting DL band 26. MSD values are reused from DC_5A-7A_n78A.

There are IMD2 impact from UL 26_n78 affecting DL band 7. MSD values are reused from DC_5A-7A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-26A_n78A DC_7C-26A_n78A | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | 26 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | 26 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | 26 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-221832, New WID: Rel-18 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon