3GPP TSG-RAN WG4 Meeting #104-e                              R4-2213132

Electronic Meeting, 15 August– 26 August, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on triple beat MSD of UL DC_3C_n28A

Agenda Item:	10.1.1

Document for:	Approval

Introduction

In last meeting, the UL configuration DC_3C_n28A was removed from specification since the 1st order triple beat MSD haven’t been studied yet. Referring to the WF [1], some criteria and assumptions were captured to encourage companies for further study. In this paper we’d like to provide our analysis on triple beat MSD of UL DC_3C_n28A.

Discussion

Test configuration

Referring to clause 7.3.3.2 from TR 38.862 [2], it’s known how to detect a triple beat issue in the FDD band DL or a third band DL.

●   Mainly consider the 1st order triple beat α (TX22TX1) of 3rd order non-linearity -> this is shown in the table as triple beat order 1.

-  Detection -> Duplex Offset<1∙ULCAAggBW+TXMBW2 +RXBW2.

For 20MHz+20MHz band 3C, the LTE nominal channel spacing is 19.8MHz and the UL CA aggregated channel bandwidth is 39.8MHz.

Since UL configuration of band n28 was restricted as 25RB for each DL bandwidth REFSENS requirement, the TxMBW is about 4.5MHz. And the duplex distance of band n28 is 55MHz. Thus, when RxBW > (duplex distance- UL CA aggregated channel bandwidth- TxMBW /2) *2, i.e. RxBW>25.9MHz, the 1st order triple beat can fall into the DL range of band n28.

Proposal 1: It’s recommended to adopt the following test configuration for triple beat MSD of DC_3C_n28A.

Table 2-1: Reference sensitivity exception due to triple beat intermodulation

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
|  | n28 | 718 | 30 | 25(RBSTART=135) | 773 | TBA | FDD | 1st order triple beat α (TX22TX1) |
| DC_3C_n28A | 3 | 1720 | 20 | 1 (RBSTART=0) | 2550 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 2645 |  |  |  |

RF architecture and parameters assumption

Referring to the WF [1], it’s proposed to assume the RF architecture of DC_3C_n28A as below. And the attenuation and isolation values are assumed in table 2-2.

Figure 1 RF architecture of DC_3C_n28A

Table 2-2 Attenuation and isolation values

| Attenuation and Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna ISO | 10 | Main antenna to diversity antenna |
| Diplexer ISO | 10 |  |
| PCB ISO | 60 |  |
| PA gain | 25 |  |
| PCB isolation Paout-Pain | 60 | PCB isolation (PA forward mixing) |
| Band n28 Duplexer Tx rejection at B3 Tx | 30 |  |
| Band n28 Duplexer Tx rejection at n28 Rx | 43 |  |
| Band B3 Duplexer Tx rejection at n28 Tx | 32 |  |
| Band B3 Duplexer Tx rejection at n28 Rx | 32 |  |
| Band n28 filter/Duplexer Rx rejection at n28 Tx | 50 |  |
| Band n28 filter/Duplexer Rx rejection at B3 Tx | 33 |  |

MSD analysis

In this case, the MSD requirements due to 1st order triple beat α (TX22TX1) of 3rd order non-linearity is dominated by band n28 LNA non-linearity performance. Table 2-3 summarize the MSD results for UL DC_3C_n28A.

Table 2-3 MSD calculation for victim Rx of band n28

| Component | 1st order triple beat α (TX22TX1) (dBm) |  |
| --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna |
| Band n28 PA Reverse mixing interference | -126.5 |  |
| Band n28 PA Forward mixing interference | -157.5 |  |
| Band B3 PA Reverse mixing interference | -95.0 |  |
| Band B3 PA Forward mixing interference | -164.5 |  |
| Band n28 LNA IMD interference via filter | -65.0 | -76.0 |
| Total | -65.0 | -76.0 |
| REFSENS specified @30MHz of band n28 | -78.5 |  |
| SNR | -1 |  |
| Noise (Thermal Noise + interference from n28 UL) | -77.5 |  |
| Noise in each path (Thermal Noise + interference from n28 UL) | -74.5 | -74.5 |
| Noise + triple beat IMD interference | -64.5 | -72.2 |
| MRC Rx Noise | -72.9 |  |
| MSD | 4.6dB |  |

Proposal 2: It’s recommended to specify triple beat MSD of DC_3C_n28A as 4.6dB.

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: It’s recommended to specify triple beat MSD of DC_3C_n28A as below.

Table 2-1: Reference sensitivity exception due to triple beat intermodulation

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3C_n28A | n28 | 718 | 30 | 25(RBSTART=135) | 773 | 4.6 | FDD | 1st order triple beat α (TX22TX1) |
|  | 3 | 1720 | 20 | 1 (RBSTART=0) | 2550 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 2645 |  |  |  |

- References

[1] R4-2210557, WF on criteria for triple beat MSD analysis, Qualcomm Incorporated, Skyworks Solutions, Inc., ZTE Corporation

[2] TR 38.862, Study on band combination handling in RAN4, v17.0.0