3GPP TSG-RAN WG4 Meeting #104-e                              R4-2213163

Electronic Meeting, 15 August– 26 August, 2022

Source: 	Huawei, HiSilicon

Title: 	General discussion on Simplification of band combination specification

Agenda Item:	11.3.3

Document for:	Approval

Introduction

In recent RAN plenary meetings, a SID [1] on simplification of band combination specification for NR and LTE was established. In order to reduce the test burden and optimize the specification structure, the following objectives were specified in this SID.

- Investigate the feasibility and optimize the specification structure and reduce the test burden

  - Study the methodology to simplify the test efforts for a UE supporting multiple features, e.g., NR-CA, EN-DC on the same band combination

    - Study of similarity and dependency of RF requirements for different features on the same band combination

  - Study the methodology to simplify RF requirement specifications for

    - MSD requirements in 38.101-1 and 38.101-3, e.g., reducing the test configurations with different bandwidth combinations

    - For Delta_TIB and Delta_RIB requirements, investigate and define the framework of the general principle or requirements with band-combination specific exceptions

    - For Delta_TC,c, investigate whether it can be removed in low boundary formula for Pcmax

In this paper, we’d like to study the similarity and dependency of RF requirements for different features on the same band combination.

Similarity and Dependency of Tx RF requirements for different features

Dual UL inter-band CA and dual UL inter-band EN-DC/NE-DC case

For some RF requirements which were specified as BC specific, we listed the corresponding requirements and analyse their similarity and dependency.

UE Maximum output power / Spurious emissions for UE co-existence:

For PC3/PC2, the UE maximum output power requirements for different features on the same band combination are same. For example, MOP requirements for CA_n1A-n3A/DC_1A_n80A/DC_1A_n3A/DC_3A_n1A/DC_3A_n84A/DC_n3A_1A are same and the same RF architecture can be reused to achieve the MOP requirements. Thus, it can be concluded that there is no need to test the MOP requirements for each band combination above. Once one of these UL configurations is verified, the other UL configurations can be considered as being capable of meeting this requirements.

Table 1 UE Maximum output power requirements

| UL configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n3A DC_1A_n80A DC_1A_n3A DC_3A_n1A DC_3A_n84A DC_n3A_1A | 23 | +2/-3 |

Similarly, UL band combinations CA_n1-n3 DC_1_n80 DC_1_n3 DC_3_n1 DC_3_n84 DC_n3_1 need to meet the same spurious emission requirements for UE to UE coexistence assuming the same protected frequency bands in same region. Once one of these UL configurations is verified, the other UL configurations can be considered as being capable of meeting these spurious emission requirements for UE to UE coexistence.

Proposal 1: Generally, if CA_nA-nB, DC_A_nB (DC_A_nD), DC_B_nA (DC_B_nC), DC_nB_A, DC_nA_B have same MOP requirements and spurious emission requirements for UE to UE coexistence, there is no need to test the MOP requirements and spurious emission requirements for UE to UE coexistence for each UL configuration again and again. Once one of these UL configurations is verified, the other UL configurations for different feature in same band combination can be considered as being capable of meeting these requirements.

NOTE: Band nC and nD are the corresponding SUL bands with same UL frequency range of band nA and nB, e.g. SUL band n80 has same UL frequency range of band n3.

Similarity and Dependency of Rx RF requirements for different features

MSD due to harmonic/harmonic mixing or cross band isolation for two bands

One of the most important Rx requirements is the REFSENS requirement. In Rel-17, the MSD requirements for DL NR CA have been simplified without considering different bandwidth combinations, referring to the following agreed CRs [2][3][4]. Thus, reference sensitivity exception due to harmonic/harmonic mixing or cross band isolation specified for ENDC band combinations can be further simplified without considering different bandwidth combinations.

Proposal 2: reference sensitivity exception due to harmonic/harmonic mixing or cross band isolation specified for ENDC band combinations can be further simplified without considering different bandwidth combinations as what we have done for R17 NR CA band combinations.

In addition, since band nA=> band nB, band A=> band nB, band nA=> band B, band nC=> band nB, band nC=> band B have the same mechanism on UE in-device interference due to harmonic/harmonic mixing or cross band isolation, RAN4 can consider to reduce the test burden instead of verifying every band combinations. It's noted that Band nC are the corresponding SUL bands with same UL frequency range of band nA, e.g. SUL band n80 has same UL frequency range of band n3.

Proposal 3: since band nA=> band nB, band A=> band nB, band nA=> band B, band nC=> band nB, band nC=> band B have the same mechanism on UE in-device interference due to harmonic/harmonic mixing or cross band isolation, it’s recommended to test one of them in order to reduce the test burden.

NOTE: Band nC are the corresponding SUL bands with same UL frequency range of band nA, e.g. SUL band n80 has same UL frequency range of band n3.

MSD due to IMD for two bands

If DL_nA-nB_UL_nA-nB has MSD requirements due to two bands IMD interference, then ENDC combos DL_A_nB_UL_A_nB and DL_B_nA_UL_B_nA can share the same mechanism and principles with NR CA band combinations.

Proposal 4: Since ENDC combos DL_A_nB_UL_A_nB and DL_B_nA_UL_B_nA can share the same mechanism and principles with NR CA band combination DL_nA-nB_UL_nA-nB which have MSD requirements due to two bands IMD interference, it’s recommended to test one of them in order to reduce the test burden.

MSD due to IMD for three DL bands

If DL_nA-nB-nC_UL_nA-nB has a sensitivity degradation on DL band nC due to three bands IMD interference, then ENDC combos DL_A-C_nB_UL_A_nB/ DL_A _nB-nC _UL_A_nB and DL_B-C_nA_UL_B_nA/ DL_B _nA-nC _UL_B_nA can share the same mechanism and principles with NR CA band combinations.

Proposal 5: Since DL_A-C_nB_UL_A_nB/ DL_A _nB-nC _UL_A_nB and DL_B-C_nA_UL_B_nA/ DL_B _nA-nC _UL_B_nA can share the same mechanism and principles with NR CA band combination DL_nA-nB-nC_UL_nA-nB which has a sensitivity degradation on DL band nC due to three bands IMD interference, it’s recommended to test one of them in order to reduce the test burden.

REFSENS without degradation for more than one DL band

If CA_nA-nB-nC-nD share the same RF architecture as ENDC combos DC_ A_nB-nC-nD / DC_ B_nA-nC-nD / DC_ C_nA-nB-nD / DC_ D_nA-nB-nC / DC_ A-B_nC-nD / DC_ A-C_nB-nD / DC_ A-D_nB-nC/ DC_ B-C_nA-nD/ DC_ B-D_nA-nC/ DC_ C-D_nA-nB / DC_ A-B-C_nD/ DC_ A-B-D_nC/ DC_ A-C-D_nB/ DC_ B-C-D_nA, the REFSENS requirements should be similar. Thus, RAN4 can consider to test one of them in order to reduce the test burden.

Proposal 6: Since ENDC combos DC_ A_nB-nC-nD / DC_ B_nA-nC-nD / DC_ C_nA-nB-nD / DC_ D_nA-nB-nC / DC_ A-B_nC-nD / DC_ A-C_nB-nD / DC_ A-D_nB-nC/ DC_ B-C_nA-nD/ DC_ B-D_nA-nC/ DC_ C-D_nA-nB / DC_ A-B-C_nD/ DC_ A-B-D_nC/ DC_ A-C-D_nB/ DC_ B-C-D_nA with NR CA band combination CA_nA-nB-nC-nD which has RESENS requirements without degradation, it’s recommended to test one of them in order to reduce the test burden.

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: Generally, if CA_nA-nB, DC_A_nB (DC_A_nD), DC_B_nA (DC_B_nC), DC_nB_A, DC_nA_B have same MOP requirements and spurious emission requirements for UE to UE coexistence, there is no need to test the MOP requirements and spurious emission requirements for UE to UE coexistence for each UL configuration again and again. Once one of these UL configurations is verified, the other UL configurations for different feature in same band combination can be considered as being capable of meeting these requirements.

NOTE: Band nC and nD are the corresponding SUL bands with same UL frequency range of band nA and nB, e.g. SUL band n80 has same UL frequency range of band n3.

Proposal 2: reference sensitivity exception due to harmonic/harmonic mixing or cross band isolation specified for ENDC band combinations can be further simplified without considering different bandwidth combinations as what we have done for R17 NR CA band combinations.

Proposal 3: since band nA=> band nB, band A=> band nB, band nA=> band B, band nC=> band nB, band nC=> band B have the same mechanism on UE in-device interference due to harmonic/harmonic mixing or cross band isolation, it’s recommended to test one of them in order to reduce the test burden.

NOTE: Band nC are the corresponding SUL bands with same UL frequency range of band nA, e.g. SUL band n80 has same UL frequency range of band n3.

Proposal 4: Since ENDC combos DL_A_nB_UL_A_nB and DL_B_nA_UL_B_nA can share the same mechanism and principles with NR CA band combination DL_nA-nB_UL_nA-nB which have MSD requirements due to two bands IMD interference, it’s recommended to test one of them in order to reduce the test burden.

Proposal 5: Since DL_A-C_nB_UL_A_nB/ DL_A _nB-nC _UL_A_nB and DL_B-C_nA_UL_B_nA/ DL_B _nA-nC _UL_B_nA can share the same mechanism and principles with NR CA band combination DL_nA-nB-nC_UL_nA-nB which has a sensitivity degradation on DL band nC due to three bands IMD interference, it’s recommended to test one of them in order to reduce the test burden.

Proposal 6: Since ENDC combos DC_ A_nB-nC-nD / DC_ B_nA-nC-nD / DC_ C_nA-nB-nD / DC_ D_nA-nB-nC / DC_ A-B_nC-nD / DC_ A-C_nB-nD / DC_ A-D_nB-nC/ DC_ B-C_nA-nD/ DC_ B-D_nA-nC/ DC_ C-D_nA-nB / DC_ A-B-C_nD/ DC_ A-B-D_nC/ DC_ A-C-D_nB/ DC_ B-C-D_nA with NR CA band combination CA_nA-nB-nC-nD which has RESENS requirements without degradation, it’s recommended to consider to test one of them in order to reduce the test burden.

- References

[1] RP-221790, Revised SID: Study on simplification of band combination specification for NR and LTE, ZTE Corporation

[2] R4-2208709, Big CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-1, ZTE Corporation.

[3] R4-2209262, Big CR on Introduction of completed SUL band combinations into TS 38.101-1, Huawei, HiSilicon

[4] R4-2208335, Big CR to 38.101-1 Introduce RF requirements for HPUE CA with 2 bands downlink and x bands uplink (x =1,2), China Telecom