3GPP TSG-RAN WG4 Meeting # 104-e		R4-2213184

# Electronic Meeting, August. 15-26, 2022

Agenda Item:	12.5.4

Source: 		Qualcomm Incorporated

Title: 	Discussion on IoT NTN UE RF requirements

Document for:	Approval

# 1 Introduction

The new WI on NB-IoT/eMTC core & performance requirements for NTN was approved in RAN#95e [1]. The RF related objectives are listed as below:

Specification of RF requirements for Satellite Access Node (SAN) and UE including the following [RAN4]: - Specification of a 200 kHz channel raster in bands where this is feasible. - In bands where it is not feasible to define a 200 kHz channel raster, the specification of a 100 kHz channel raster to be used in conjunction with signalling of the “part-of EARFCN” indication on MIB, with multiple EARFCN hypotheses. - Verification of co-existence of IoT NTN with TN, re-using or extrapolating from existing coexistence results (from NR NTN or other) where appropriate, and considering additional simulations as necessary. - Leveraging the studies and requirements (where applicable) of NTN NR bands n256 and n255 (and any relevant E-UTRA bands), specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation: - S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL) - L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

In this paper, we provide our views on the IoT NTN UE RF requirements.

# 2 Discussion

- General aspects for UE RF requirements

The UE RF requirements for NR NTN have been specified in TS 38.101-5. Similar as Rel-17 NR NTN, a new specification, i.e., TS 36.101-2 will be created to capture IoT NTN UE requirements. RAN4 shall follow the same principle as NR NTN to reuse the TN requirements as much as possible. Meanwhile, the architecture of TS 36.101 should be the baseline for IoT NTN UE RF specification. For the same requirements for TN and NTN, directly referring to TS 36.101 would be preferred.

Proposal 1: RAN4 shall follow the same principle as NR NTN to reuse the TN requirements as much as possible. The architecture of TS 36.101 should be the baseline for IoT NTN UE RF specification. For the same requirements for TN and NTN, directly referring to TS 36.101 would be preferred.

- Operating bands

NR NTN bands n255 and n256 were specified. For IoT NTN, the same band numbering without prefix ‘n’, i.e., 255 and 256, should be defined. In addition, the feasibility of indicating LTE band number up to 256 should be confirmed.

Table 1: Satellite operating bands for NB-IoT/M1

| NB-IoT/ M1 satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| 256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| 255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE: NB-IoT/M1 satellite bands are numbered in descending order from 256. |  |  |  |

Proposal 2: For IoT NTN, the same band numbering without prefix ‘n’, i.e., 255 and 256, should be defined.

Proposal 3: The feasibility of indicating LTE band number up to 256 should be confirmed.

- Transmitter characteristics

The preliminary analysis for transmitter characteristics is listed in Table 2.

Table 2: Transmitter characteristics

| Transmitter requirements | Note |
| --- | --- |
| UE maximum output power | TN requirements can be reused. |
| MPR | Depending on the SEM and ACLR requirements. |
| A-MPR | Depending on the regulatory requirements. Same as n256 and n255. |
| Configured transmitted power | TN requirements can be reused. |
| Minimum requirement | TN requirements can be reused. |
| Transmit OFF power | TN requirements can be reused. |
| ON/OFF time mask | TN requirements can be reused. |
| Power Control | TN requirements can be reused. |
| Frequency error | More discussion is needed. See paper [2] |
| Transmit modulation quality | TN requirements can be reused. |
| Out of band emission | SEM/ACLR depends on the co-ex study |
| Transmit intermodulation | TN requirements can be reused. |

Proposal 4: Take the analysis for transmitter characteristics listed in Table 2 into account to specify IoT NTN UE Tx requirements.

- Receiver characteristics

The preliminary analysis for receiver characteristics is listed in Table 3.

Table 3: Receiver characteristics

| Receiver requirements | Note |
| --- | --- |
| Diversity characteristics | TN requirements can be reused. |
| Reference sensitivity power level | For NB-IoT, TN requirements can be reused. For M1, REFSENSE specified for n256 and n255 can be reused. |
| Maximum input level | NR NTN requirements can be reused. |
| ACS | Depending on co-ex study |
| Blocking | For NB-IoT, TN requirements can be reused. For M1, need to check whether to reuse the NR NTN out of band blocking requirements of n256. |
| Spurious response | TN requirements can be reused. |
| Intermodulation characteristics | TN requirements can be reused. |
| Spurious emissions | TN requirements can be reused. |

Proposal 5: Take the analysis for receiver characteristics listed in Table 3 into account to specify IoT NTN UE Rx requirements.

# 3 Conclusions

This contribution provided the analysis on the IoT NTN UE RF requirements and the following proposals were made:

Proposal 1: RAN4 shall follow the same principle as NR NTN to reuse the TN requirements as much as possible. The architecture of TS 36.101 should be the baseline for IoT NTN UE RF specification. For the same requirements for TN and NTN, directly referring to TS 36.101 would be preferred.

Proposal 2: For IoT NTN, the same band numbering without prefix ‘n’, i.e., 255 and 256, should be defined.

Proposal 3: The feasibility of indicating LTE band number up to 256 should be confirmed.

Proposal 4: Take the analysis for transmitter characteristics listed in Table 2 into account to specify IoT NTN UE Tx requirements.

Proposal 5: Take the analysis for receiver characteristics listed in Table 3 into account to specify IoT NTN UE Rx requirements.

# 4 References

RP-220938, New WID on NB-IoT/eMTC core & performance requirements for NTN, MediaTek Inc.

R4-2214045, Doppler pre-compensation in NTN IoT, Qualcomm Incorporated