3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213185

# Electronic Meeting, August. 15-26, 2022

Agenda Item:	12.5.2

Source: 		Qualcomm Incorporated

Title: 	Coexistence simulation restuls for TN-NTN NB IoT

Document for:	Approval

# Introduction

In this paper, we provide our preliminary simulation results for TN with NTN NB IoT in all the cases as captured in [1] for the TN with NTN coexistence simulations.

# 2. Discussion

## 2.1 Simulation scenarios

In this document we present our preliminary simulation results between TN and NTN NB IoT for the 6 cases as captured [1]. The simulated cases are summarized in Table 1. All the results are to be considered preliminary since the assumptions still need further discussion.

Table 1. List of study cases

| No. | Aggressor | Victim |
| --- | --- | --- |
| S1 | TN DL | NB IoT NTN DL |
| S2 | TN UL | NB IoT NTN UL |
| S3 | NB IoT NTN DL | TN DL |
| S4 | NB IoT NTN UL | TN UL |
| S5 | NB IoT NTN UL | TN DL |
| S6 | TN DL | NB IoT NTN UL |

## 2.2. Simulation parameters

The simulation parameters and assumptions as captured in [1] are used in the coexistence analysis between TN and NTN. Some simulation parameters and assumptions are adopted for the NB IoT NTN. For example, the NB IoT NTN channel bandwidth used with 1RB (180 kHz) with 1 UE in the DL and 12 UEs with 15 kHz BW in the UL direction.

The ACIR model is adopted as summarized in Table 2 for the NB IoT simulations. Rest of assumptions and parameters didn’t change from [1].

Table 2. ACIR model for each case

| No. | Aggressor | Victim |  |
| --- | --- | --- | --- |
| S1 | TN DL (1 UE, 20 MHz) | NB IoT NTN DL (1 UE, 180 kHz) | Flat model with adjusting the ACIR according to TR 36.942 and adjusting the ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor), and assuming flat ACS |
| S2 | TN UL (3 UE, 20/3 MHz each) | NB IoT NTN UL (12 UE, 15 kHz each) | 3 steps ACLR model according to TR 36.942 with adjusting the ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor), and assuming flat ACS |
| S3 | NB IoT NTN DL (1 UE, 180 kHz) | TN DL (1 UE, 20 MHz) | Flat model with adjusting the ACIR according to TR 36.942, 3 steps ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor), and assuming flat ACS |
| S4 | NB IoT NTN UL (12 UE, 15 kHz each) | TN UL (3 UE, 20/3 MHz each) | 3 steps ACLR model according to TR 36.942 with adjusting the ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor), and assuming flat ACS |
| S5 | NB IoT NTN UL (12 UE, 15 kHz each) | TN DL (1 UE, 20 MHz) | 3 steps ACLR model according to TR 36.942 with adjusting the ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor), and assuming flat ACS |
| S6 | TN DL (1 UE, 20 MHz) | NB IoT NTN UL (12 UE, 15 kHz each) | Flat ACIR model without adjusting the ACLR, and assuming flat ACS (worst case) |
| Note 1: Step 1 is 30 + X, Step 2 is 43 + X and Step 3 is 50 + X, where X is the step size for simulations, X = … -10, -5, 0, 5, 10… dB |  |  |  |

## 2.2. Network layout model

The same network layout model is used as captured in [1].

## 2.3 Simulation results

The coexistence simulation results for the 6 cases which are mentioned in Section 2.1 are discussed in this section. In the following, the simulation results will show the throughput loss as a function of the ACIR as seen in Table 3 for the case TN is the victim as in [2]. In case the NTN NB IoT is the victim the SINR loss is used as in [2] in dB as a function of the ACIR for different CDF points e.g., 5%, 50% and 95%-ile as seen in Table 4.

Table 3: Summary of ACIR as a function of Throughput Loss

| No. |  | Average T-Loss ACIR [dB] |  |  |  |  |  | Cell edge T-Loss ACIR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Environment / Satellite |  | Rural |  |  | Urban |  |  | Rural |  |  | Urban |  |  |
|  |  | GEO @ EL 90 | LEO 600 | LEO 1200 | GEO @ EL 90 | LEO 600 | LEO 1200 | GEO @ EL 90 | LEO 600 | LEO 1200 | GEO @ EL 90 | LEO 600 | LEO 1200 |
| 3 | NTN DL to TN DL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 5 | 0 | 0 | 0 |
| 4 | NTN UL to TN UL | 4 | 4 | 4 | 29 | 29 | 29 | 18 | 18 | 18 | 35 | 35 | 35 |
| 5 | NTN UL to TN DL | 22 | 22 | 22 | 18 | 18 | 18 | 32 | 32 | 32 | 26 | 26 | 26 |

Table 4: Summary of ACIR as a function of SINR Loss

| No. |  | ACIR [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Environment / Satellite |  | Rural |  |  |  |  |  |  |  |  |  | Urban |  |  |  |  |  |  |  |  |  |
|  |  | GEO @ EL 90 |  |  | LEO 600 |  |  | LEO 1200 |  |  | GEO @ EL 90 |  |  |  | LEO 600 |  |  | LEO 1200 |  |  |
| SINR Loss @x%-ile <1dB |  | 5 | 50 | 95 | 5 | 50 | 95 | 5 | 50 | 95 | 5 |  | 50 | 95 | 5 | 50 | 95 | 5 | 50 | 95 |
| 1 | TN DL to NTN DL | 0 | 0 | 10 | 0 | 0 | 2 | 0 | 0 | 2 | 12 |  | 22 | 38 | 3 | 13 | 31 | 3 | 13 | 31 |
| 2 | TN UL to NTN UL | 2.3 | 5 | 9 | 0 | 0 | 1 | 0 | 0 | 5 | 6 |  | 12 | 18 | 0 | 0 | 5.7 | 0 | 4 | 11 |
| 6 | TN DL to NTN UL | 15 | 19 | 26 | 5 | 10 | 17 | 9 | 13 | 20 | 29 |  | 36 | 44 | 16 | 23 | 32 | 21 | 28 | 37 |

From the preliminary simulation results in Table 3 and Table 4, we have the following comments for each case as follows:

- Case 1 (TN DL to NTN DL)

The Urban environment shows high ACIR requirements around 38 dB for GEO. However, in these simulations, no isolation distance assumed as in [1]. From, that we recommend applying isolation distance for Urban scenario as used in [1]. That will relax the ACIR requirements.

- Case 2 (TN UL to NTN UL)

The simulation results for case 2 is showing no high ACIR requirements needed. Hence the requirements in [1] can be reused.

- Case 3 (NTN DL to TN DL)

The simulation results for case 3 is showing no high ACIR requirements needed similarly to case 2. Hence the requirements in [1] can be reused.

- Case 4 (NTN UL to TN UL)

The simulation results for case 4 is showing a bit higher ACIR requirements with 35 dB for GEO case in the urban scenario compared to the results in [1]. That’s because of adjusting the ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor) as explained in Table 2. Furthermore, in this scenario we assumed 12 NTN UEs which is higher than the number of UEs in [1] which was 9 UEs. However, if case 1 will apply isolation distance in urban scenario, then the ACLR requirements would be even relaxed.

- Case 5 (NTN UL to TN DL)

The simulation results for case 5 is showing similar trend to case 4 with a bit higher ACIR requirements with 32 dB for GEO in rural scenario compared to the results in [1]. That’s because of adjusting the ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor) as explained in Table 2. Furthermore, in this scenario we assumed 12 NTN UEs which is higher than the number of UEs in [1] which was 9 UEs.

- Case 6 (TN DL to NTN UL)

The simulation results for case 6 is showing high ACIR requirements around 44 dB for GEO and 37 dB for LEO 1200. However, based on the discussion in RAN4 meeting #102e, the SAN ACS requirement can apply without the need to tighten it.

# 3.	Conclusion

In this paper, we provided simulation results between TN and NB IoT NTN for 6 coexistences scenarios. The summary of the results is shown in Table 3 and Table 4.

Based on the results obtained so far, the following observations can be made:

Observation 1: Coexistence in case 1, i.e., TN DL to NTN DL, is challenging in urban scenario. This is due to the NTN DL being much weaker than TN DL.

Observation 2: In case 2 and case 3, the coexistence could be feasible between TN and NTN in UL and the requirements in [1] could be be reused.

Observation 3: In case 4, the ACIR requirements are a bit tighter compared to the results in [1]. That’s because the adjustment in the ACLR and increasing number of NTN UEs.

Observation 4: In case 5 and similarly to observation 3, the ACIR requirements are a bit tighter compared to the results in [1].

Observation 5: For case 6, the simulations results show same trend as in [1] with high ACIR requirements. However, the SAN ACS requirement was discussed in RAN4 #102e and it was agreed to use 38 dB.

Based on the above, we propose the following:

Proposal 1: To consider the isolation distance in case 1 similarly to the studies in [1].  Based on that the NTN UE ACS from [1] can be reused.

Proposal 2: To use SINR loss as metric when NTN NB IoT is the victim and use throughput loss as metric when TN is the victim as in [2].

Proposal 3: RAN4 to further discuss the ACLR and ACS requirements for IoT NTN UE and SAN based on the co-existence simulation results.

# 4.	References

- 3GPP TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects

- 3GPP TR 36.802, Evolved Universal Terrestrial Radio Access (E-UTRA); NB-IOT; Technical Report for BS and UE radio transmission and reception.