3GPP TSG-RAN WG4 Meeting #104-e	R4-2213186

Electronic Meeting, August. 15-26, 2022

Agenda Item:

Source: 		Qualcomm Incorporated

Title: 	Coexistence scenarios of Air-to-ground network for NR

Document for:	Approval

# 1. Introduction

In RAN #95e meeting, the new WID on Air-to-ground network for NR was approved in [1]. Air-to-ground (ATG) network refers to in-flight connectivity technique, and a direct radio link will be established between BS on the ground and CPE type of UE mounted in the aircraft. The coexistence requirements between ATG and terrestrial network are important part of this WID to define the RF requirements for ATG. Therefore, this paper provides our considerations on the coexistence scenarios.

# 2. Discussion

# 2.1 System Parameter

There are already some trials and commercial operation for ATG, while the ATG system characteristics haven’t been discussed in 3GPP, which are essential for coexistence studies. The ATG topology and the system characteristics should be firstly discussed. Table 1 and Table 2 provide the essential key parameters for coexistence studies and the specific values are TBD.

Table 1. ATG Base Station Characteristics

| Type | ATG BS |
| --- | --- |
| Channel bandwidth (MHz) | TBD |
| Cell radius | TBD |
| Sector number | TBD |
| Max transmission power | TBD |
| Antenna gain | TBD |
| Beamforming pattern | TBD |
| Transmission bandwidth | TBD |
| ATG BS height | TBD |
| Noise figure | TBD |

Table 2. Aircraft CPE Characteristics

| Type | Aircraft CPE |
| --- | --- |
| Max transmission power | TBD |
| Antenna gain | TBD |
| Transmission bandwidth | TBD |
| Number of aircraft per BS | TBD |
| Aircraft height | TBD |
| Noise figure | TBD |
| Power control mechanism | TBD |

Observation 1: The ATG system characteristics haven’t been discussed in 3GPP, therefore, the ATG topology and the system characteristics should be firstly discussed for the coexistence.

## 2.2 Coexistence Scenario

The example bands mentioned in [1] are band n1, n78 and n79. Normally, only adjacent channel coexistence will be considered in RAN4 coexistence studies.  The band analysis is descripted as below.

- n1 (1920-1980MHz UL，2110-2170MHz DL)

  - Adjacent with NTN band n256 1980-2010MHz UL, 2170-2200MHz DL

  - The UL is adjacent to TDD band n39 1880-1920MHz

- n78 (3300-3800MHz)

- n79 (4400-5000MHz)

The band n1 is FDD band and is adjacent with NTN band n256.  The ATG UL is the link from aircraft to the ground-based BS, while the NTN UL is the link from UE on the ground to the satellite. The transmission signal direction is different for both UL and DL, so the antenna beamforming direction will be different, and the antenna gain for receiving the interference signal will be extremely low. Furthermore, considering the high altitude for satellite, e.g. 600km for LEO, the interference between ATG and NTN can be neglected with the high propagation loss and the extreme low antenna gain. Therefore, the coexistence study between ATG and NTN can be considered as low priority.

The band n1 UL is also adjacent to TDD band n39. Therefore, cross-link-interference should be considered for the ATG coexistence with terrestrial network.

The coexistence scenarios are summarized in Table 3.

Table 3. The coexistence scenarios considered for ATG and conventional terrestrial network

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | ATG with conventional BS | ATG DL | Conventional DL | Phase 1: Applicable for ATG operating in band n1, n78,n79 |
| 2 | ATG with conventional BS | ATG UL | Conventional UL |  |
| 3 | ATG with conventional BS | Conventional DL | ATG DL |  |
| 4 | ATG with conventional BS | Conventional UL | ATG UL |  |
| 5 | ATG with conventional BS | ATG UL | Conventional DL | Phase 1: Applicable for ATG operating in band n1, for e.g. coexistence with n39 |
| 6 | ATG with conventional BS | Conventional DL | ATG UL |  |
|  | ATG with ATG | ATG DL | ATG DL | Phase 2: Applicable for ATG operating in band n1, n78,n79 |
|  | ATG with ATG | ATG UL | ATG UL |  |

Observation 2: the example band n1 is adjacent to the NTN band n256. The interference between ATG and NTN can be neglected considering the high propagation loss and the low antenna gain on the interference signal direction.

Proposal 1: No need to consider the coexistence study between ATG and NTN in this WI.

Proposal 2: To consider the coexistence scenarios between ATG and conventional terrestrial network summarized in Table 3.

## 2.3 Coexistence Topology

The ISD for ATG BS is usually very large, e.g. about 100km to 200km as in [1]. Considering the big difference between ATG and conventional terrestrial network, one ATG BS is enough for the coexistence topology. Whether wrap-around is considered for ATG network needs to be discussed.

The coexistence topology can be separated into two parts: 1) the ATG interferes to conventional TN and 2) the conventional TN interferes to ATG as shown in Figure 1 and Figure 2.

For Figure 1, the ATG DL to conventional TN DL case can be simple since the locations of BSs are fixed. The beamforming pattern will influence the interference power to conventional UE receiving. The case that the ATG UL interfering to conventional TN UL will depend on whether CPE on aircraft is beamformed, and the aircraft locations.

ATG BS

Conventional BS

UL

DL

Figure 1. Coexistence topology for ATG interferes to conventional TN

For Figure 2, the interference from conventional TN to ATG are aggregated interference. Therefore, the topology needs further discussion, e.g. how many conventional BSs are counted, how the interference is aggregated, what is networking loading, the difference for rural and urban, etc. Many factors will impact the coexistence topology and the coexistence results.

Conventional BS

ATG BS

UL

DL

Conventional BS

ATG BS

UL

DL

Figure 2. Coexistence topology for conventional TN interferes to ATG

Proposal 3: The coexistence topology illustrated in Figure 1 and Figure 2 are proposed for initial discussion. The methodology for aggregated interference calculation should be further discussed.

# 3. Conclusion

In this paper, we discussed the coexistence studies for the ATG network, which is important part to define the RF requirements of ATG. We have the following observations and proposals:

Observation 1: The ATG system characteristics haven’t been discussed in 3GPP, therefore, the ATG topology and the system characteristics should be firstly discussed for the coexistence.

Observation 2: the example band n1 is adjacent to the NTN band n256. The interference between ATG and NTN can be neglected considering the high propagation loss and the low antenna gain on the interference signal direction.

Proposal 1: No need to consider the coexistence study between ATG and NTN in this WI.

Proposal 2: To consider the coexistence scenarios between ATG and conventional terrestrial network summarized in Table 3.

Proposal 3: The coexistence topology illustrated in Figure 1 and Figure 2 are proposed for initial discussion. The methodology for aggregated interference calculation should be further discussed.

# 4. References

[1] RP-220962, New WID: Air-to-ground network for NR, CMCC