3GPP TSG-RAN WG4 Meeting #104-e                                                    	R4-2213191

Electronic Meeting, 15– 26 August, 2022

Source:	Huawei, Hisilicon

Title:	TP for TR 37.718-21-11 DC_1A-8A_n7A

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

This contribution discusses the MSD requirement for 3DL/2UL DC combinations DC_1A-8A_n7A. A text proposal for TR 37.717-21-11 is provided to include DC_1A-8A_n7A.There are two uplink configurations,among them DC_1A-8A_n7A with UL DC_8A_n7A  can reuse the DC_1A-7A_n8A with UL DC_8A_n7A  which is completed in 38.101-3.This paper focuses on the DC_1A-8A_n7A with uplink configuration of DC_1A_n7A.

2.Discussion

For the 3DL/2UL MSD analysis, the cases that harmonic issue and IMD interference fall into the own Rx frequency of interfering band have been discussed and specified in the constituent fallback mode, thus we mainly focus on the cases that the IMD falls in the 3rd band Rx frequency. The details for DC combination have self-desense problems into the own 3rd Rx frequency band are listed as below:

3DL_DC_1A-8A_n7A with 2UL_1A_n7A   |2*fn7-3*fB1|   5th IMD into B8

Considering the minimum bandwidth is 5MHz, there is  only one combination that  can hit B8,that is 3*1977.5-2*2502.5=927.5MHz.

We use the Triplexer based RF front-end architecture to analyze the amount of IMD using the following component linearity assumptions:

Table 2 General linearity parameters

| Component | IP5 (dBm) |
| --- | --- |
| Antenna switch | 53 |
| Diplexer | 53 |
| Triplexer | 53 |
| PA forward mixing | 28 |
| PA reverse mixing | 30 |
| LNA | -10 |

Table 2 Attenuation and isolation

| Parameters | Values (dB) |
| --- | --- |
| Antenna isolation | 10 |
| PCB isolation PA-PA | 60 |
| Diplexer isolation | 15 |
| Tx duplexer rejection at Rx Band | 50 |
| LNA (in) to PA (out) (L-H/H-L cross-band) | 60 |
| PA (out) to PA (out) (quadplexer) | 50 |

Based on these parameters we get the following MSD.

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-8A_n7A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 5 | FDD | IMD5，¦2*fn7-3*fB1¦ |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |

3.Conclusion

This contribution discussed the MSD requirements for 3DL/2UL DC combinations including DC_1A-8A_n7A.

4.References

[1] 3GPP TS 36.101 V17.6.0 (2022-06), User Equipment (UE) radio transmission and reception (Release 17)

5. Text Proposal

<Start of Text Proposal>

5.X	DC_1A-8A_n7A

### 5.X.1	Configurations for DC

Table 5.239.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-8A_n7A | DC_8A_n7A DC_1A_n7A |

### 5.X.2	Co-existence studies

For 2UL/3DL(DC_1A-8A_n7A  with UL DC_1A_n7A) UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.239.2-1.

Table 5.239.2-1: Harmonic and IMD analysis for DC_1A_n7A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

As we can see from the above table，for 3DL_DC_1A-8A_n7A with 2UL_1A_n7A，Two-tone 5th order IMD products |2*fy -3*fx |may fall into DL reception frequency of Band 8.

### 5.X.3	∆TIB and ∆RIB values

The requirements of ∆TIB values in Table 6.2.5-3: ΔTIB,c (three bands)  “CA_1-7-8 , CA_1-7-7-8”from TS36.101 [2] can be applied for DC_1-8_n7.

The requirements of ∆RIB values in Table 7.3.1-1B: ΔRIB,c (three bands) “CA_1-7-8 , CA_1-7-7-8” from TS36.101 [2] can be applied for DC_1-8_n7.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n7 | 1 | 0.5 |
|  | 8 | 0.6 |
|  | n7 | 0.6 |

Table 5.X.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n7 | 1 | 0 |
|  | 8 | 0.2 |
|  | n7 | 0 |

### 5.X.4	Reference sensitivity exceptions

Table 5.239.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from DC_1A-8A_n7A.

Table 5.239.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-8A_n7A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
|  | 8 | 882.5 | 5 | 25 | 927.5 | 5.0 | IMD5 |

<End of Text Proposal>