3GPP TSG-RAN WG4 Meeting # 104 -e	R4-2213221

Electronic Meeting, August 25th – 26th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE Rx RF aspects for FR2-2

Agenda item:	9.14.3.2

Document for:	Approval

# Introduction

The latest WID on extending current NR operation to 71GHz [1] was approved at RAN#94. Before that, 3GPP RAN studied requirements for NR beyond 52.6GHz up to 114.25GHz, potential use cases and deployment scenarios, and NR system design requirements and considerations on top of regulatory requirements as captured in [2].

RF core requirements in 38.101-2

- Core requirements including MPR, A-MPR, Spurious, Blocking, ACS, Max input power, FRCs

RF core requirements in 38.101-2 and 38.104

- Core requirements including CA

Requirements in 38.101-3

- Inter-band CA configurations: down-scope intra-band non-contiguous CA within FR2-2 from Rel-17.

Note: The down-scoping of intra-band non-contiguous CA within FR2-2 in Rel-17 pertains to completion of the RF core requirements and is not intended to change how channelization for FR2-2 is developed.

The work has been extended at RAN#96 via the exceptions captured in RP-221783 as shown below:

In this contribution UE Rx related aspects are discussed.

# Discussion

During RAN4#103 agreements were made for most remaining FR2-2 Rx requirements. However, ACS and IBB were not resolved.

#### ACS Requirements

For ACS the WF captures the values presented in the table below for further consideration [3].

| ACS (Table 7.5-1) | Case 1 (Table 7.5-2) |  | Case 2 (Table 7.5-3) |  |
| --- | --- | --- | --- | --- |
| dB | Power in Transmission Bandwidth | PInterferer | Power in Transmission Bandwidth | PInterferer |
| 21 | REFSENS + 14 | REFSENS + 33.5 | -44.5 dBm | -25 dBm |

This ACS proposal is a slight relaxation as compared to FR2-1 but given the additional larger channel bandwidths it can be accepted. It can also be noted that the ACS of 21 dB is aligned to the TS. 38.803.

Proposal 1: Agree ACS requirements as given in TP#1 and TP#2

******* Start of TP #1 to 38.102-2 *******

Table 7.5-1: Adjacent channel selectivity

| Operating band |  | Units |  | Adjacent channel selectivity / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz |  | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257, n258, n261 | dB |  | 23 |  | 23 | 23 | 23 | N/A | N/A | N/A |
| n259, n260, n262 | dB |  | 22 |  | 22 | 22 | 22 | N/A | N/A | N/A |
| n263 | dB |  | 21 |  | 21 | 21 | 21 | 21 | 21 | 21 |

Table 7.5-2: Adjacent channel selectivity test parameters, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz |  | 100 MHz |  | 200 MHz |  | 400 MHz |  | 800 MHz |  | 1600 MHz |  | 2000 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |
| PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB |  | REFSENS +35.5 dB |  | REFSENS +35.5 dB |  | REFSENS +35.5 dB |  | N/A |  | N/A |  | N/A |
| PInterferer for band n259, n260, n262 | dBm | REFSENS + 34.5 dB |  | REFSENS +34.5 dB |  | REFSENS +34.5 dB |  | REFSENS +34.5 dB |  | N/A |  | N/A |  | N/A |
| PInterferer for band n263 | dBm | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB |
| BWInterferer | MHz | 50 |  | 100 |  | 200 |  | 400 |  | 800 |  | 1600 |  | 2000 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 |  | 100 / -100 NOTE 3 |  | 200 / -200 NOTE 3 |  | 400 / -400 NOTE 3 |  | 800 / -800 NOTE 3 |  | 1600 / -1600 NOTE 3 |  | 2000 / -2000 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern as described in Annex A.3.2 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Clause 7.3.2, which are applicable to different UE power classes. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. |  |  |  |  |  |  |  |  |

Table 7.5-3: Adjacent channel selectivity test parameters, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz |  |  | 100 MHz |  | 200 MHz |  | 400 MHz |  | 800 MHz |  | 1600 MHz |  | 2000 MHz |  |
| Power in Transmission Bandwidth Configuration for band n257, n258, n261 |  | dBm |  | -46.5 |  |  | -46.5 |  | -46.5 |  | -46.5 |  | N/A |  | N/A |  | N/A |  |
| Power in Transmission Bandwidth Configuration for band n259, n260, n262 |  | dBm |  | -45.5 |  |  | -45.5 |  | -45.5 |  | -45.5 |  | N/A |  | N/A |  | N/A |  |
| Power in Transmission Bandwidth Configuration for band n263 | dBm | -44,5 | -44,5 | -44,5 | -44,5 | -44,5 | -44,5 | -44,5 |
| PInterferer | dBm | -25 |  |  |  |  |  |  |
| BWInterferer |  | MHz |  | 50 |  |  | 100 |  | 200 |  | 400 |  | 800 |  | 1600 |  | 2000 |  |
| FInterferer (offset) |  | MHz |  | 50 / -50 NOTE 2 |  |  | 100 / -100 NOTE 2 |  | 200 / -200 NOTE 2 |  | 400 / -400 NOTE 2 |  | 800 / -800 NOTE 2 |  | 1600 / -1600 NOTE 2 |  | 2000 / -2000 NOTE 2 |  |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern TDD as described in Annex A and set-up according to Annex C. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 3: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. |  |  |  |  |  |  |  |  |

******* End of TP #1 to 38.102-2 *******

******* Start of TP #2 to 38.102-2 *******

Table 7.5A.1-1: Adjacent channel selectivity for intra-band contiguous CA

| Operating band | Units | Adjacent channel selectivity / CA bandwidth class |
| --- | --- | --- |
|  |  | All CA bandwidth class |
| n257, n258, n261 | dB | 23 |
| n259, n260, n262 | dB | 22 |
| n263 | dB | 21 |

Table 7.5A.1-2: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 1

| Rx Parameter | Units | All CA bandwidth Classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| PInterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| PInterferer for band n259, n260, n262 | dBm | Aggregated power + 20.5 |
| PInterferer for band n263 | dBm | Aggregated power + 19.5 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | + BWchannel CA / - BWchannel CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern as described in Annex A and set-up according to Annex C. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interfering signal has the same SCS as that of the closest carrier. NOTE 4: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. |  |  |

Table 7.5A.1-3: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 2

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n257, n258, n261 | dBm | - 46.5 |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n259, n260, n262 | dBm | - 45.5 |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n263 | dBm | - 44.5 |
| Pinterferer | dBm | - 25 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | + BWchannel CA / - BWchannel CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interfering signal has the same SCS as that of the closest carrier. NOTE 4: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. |  |  |

******* End of TP #2 to 38.102-2 *******

#### IBB Requirements

For IBB the WF [3] captures that agreement seem to be possible for channel bandwidths up to and including 400 MHz by reusing FR2-1 requirements. Further, it is proposed that if the ACS discussed above is agreed then the following should be agreed for IBB:

- The IBB of band n263 is specified for all supported bandwidths

- The IBB requirement of band n263 is the same as ACS requirement.

- Single CC Similar to FR2-1 with changed interference level to aligned to the ACS

- Intra contiguous DL CA same as FR2-1 with interference level changed to aligned to the ACS

From our perspective the FR2-1 requirements could be reused but given the adapted ACS for n263 we are fine with agreeing IBB as describe above. Therefor we have

Proposal 2: Agree IBB requirements as given in TP#3 and TP#4.

******* Start of TP #3 to 38.102-2 *******

Table 7.6.2-1: In band blocking requirements

| Rx parameter |  |  |  | Units |  | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz |  |  |  | 100 MHz |  | 200 MHz |  | 400 MHz |  | 800 MHz |  | 1600 MHz | 2000 MHz |  |  |  |
| Power in Transmission Bandwidth Configuration |  |  |  | dBm |  | REFSENS + 14 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWInterferer | MHz |  | 50 |  |  |  | 100 |  | 200 |  | 400 |  | 800 |  | 1600 | 2000 |  |  |  |
| PInterferer for bands n257, n258, n261 | dBm |  | REFSENS + 35.5 dB |  |  |  | REFSENS + 35.5 dB |  | REFSENS + 35.5 dB |  | REFSENS + 35.5 dB |  | N/A |  | N/A | N/A |  |  |  |
| PInterferer for band n259, n260, n262 | dBm |  | REFSENS + 34.5 dB |  |  |  | REFSENS + 34.5 dB |  | REFSENS + 34.5 dB |  | REFSENS + 34.5 dB |  | N/A |  | N/A | N/A |  |  |  |
| PInterferer for band n263 |  | dBm |  |  | REFSENS + 33.5 dB |  |  | REFSENS + 33.5 dB |  | REFSENS + 33.5 dB |  | REFSENS + 33.5 dB |  | REFSENS + 33.5 dB |  |  | REFSENS + 33.5 dB | REFSENS + 33.5 dB |
| FIoffset | MHz |  | ≤ -100 & ≥ 100 NOTE 5 |  |  |  | ≤ -200 & ≥ 200 NOTE 5 |  | ≤ -400 & ≥ 400 NOTE 5 |  | ≤ -800 & ≥ 800 NOTE 5 |  | ≤ -1600 & ≥ 1600 NOTE 5 |  | ≤ -3200 & ≥ 3200 NOTE 5 | ≤ -4000 & ≥ 4000 NOTE 5 |  |  |  |
| FInterferer | MHz |  | FDL_low + 25 to FDL_high - 25 |  |  |  | FDL_low + 50 to FDL_high - 50 |  | FDL_low + 100 to FDL_high - 100 |  | FDL_low + 200 to FDL_high - 200 |  | FDL_low + 400 to FDL_high - 400 |  | FDL_low + 800 to FDL_high - 800 | FDL_low + 1000 to FDL_high - 1000 |  |  |  |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1. TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE2: The REFSENS power level is specified in Clause 7.3.2, which are applicable according to different UE power classes. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG pattern OP.1 TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 4: FIoffset is the frequency separation between the center of the channel bandwidth and the center frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FIoffset shall be further adjusted (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. NOTE 7: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

******* End of TP #3 to 38.102-2 *******

******* Start of TP #4 to 38.102-2 *******

Table 7.6A.2.1-1: In band blocking minimum requirements for intra-band contiguous CA

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Power in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| Pinterferer for band n260, n262 | dBm | Aggregated power + 20.5 |
| Pinterferer for band n263 | dBm | Aggregated power + 19.5 |
| BWInterferer | MHz | BWChannel_CA |
| FIoffset | MHz | +2*BWChannel_CA / -2*BWChannel_CA NOTE 5 |
| FInterferer | MHz | FDL_low + 0.5*BWChannel_CA To FDL_high - 0.5*BWChannel_CA |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Table 7.3.2-1. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.3.2 QPSK, R=1/3 with one sided dynamic OCNG pattern OP.1 TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 4: The FInterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interfering signal has the same SCS as that of the closest carrier. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. NOTE 7: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. |  |  |

******* End of TP #4 to 38.102-2 *******

# Conclusion

In this contribution, we discuss ACS and IBB for UE for a NR band in the range 52.6GHz – 71GHz. We have made following observations and proposals:

Proposal 1: Agree ACS requirements as given in TP#1 and TP#2

Proposal 2: Agree IBB requirements as given in TP#3 and TP#4.

# References

- RP-213540, Revised WID:  Extending current NR operation to 71 GHz

- TR 38.808, “Study on supporting NR from 52.6 GHz to 71 GHz”, 3GPP

- R4-2210578, “WF on 60 GHz UE RF” , Qualcomm