3GPP TSG-RAN4 Meeting #104-e	R4-2213222

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	5.1.8

Source: 	Nokia

Title: 	Motivation for correction of CA_n40-n41 synchronous RX_TX

Document for:	Approval

# Introduction

The specification of TDD CA operation has been considered recently in RAN4 – 103 [1] with the introduction of CA_n38-n40 into 38.101-1. As flagged by other companies there is a history of this type of TDD bands that must be considered when expanding the lists of supported carrier aggregation combinations. This paper is capturing the information provided in relation to the flag of [1] and presents a correction to 38.101-1 that will prevent misinterpretation of the defined bands and their modes of operation.

# Discussion

As highlighted by other companies the introduction of CA_n38-n40 would require considerations of the mode of operation. 38.101-1 captures the TDD operation in two different notes:

NOTE 1:	Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability.

NOTE 9:	Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx.

Due to legacy of low loss TDD filtering for the mode of operation of the combination CA_n38-n40 the NOTE 9 was added in Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands) of 38.101-1. Making CA_n38-n40 applicable only for UE supporting Inter-band CA without simultaneous Rx/Tx.

Another table of 38.101-1 supports the decision surrounding the CA combination in terms of mode of operation. In Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands) of 38.101-1 the ΔTIB,c value of the CA combination CA_n40-n41 has a NOTE 3 at each band.

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n40-n41 | n40 | 0.53 |
|  | n41 | 0.53 |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

This NOTE 3 clarifies that CA_n40-n41 is considered without simultaneous RX/TX.

The motivation for adding CA_n38-n40 without any NOTE in 38.101-1 Table 5.2A.2.1-1 would be found a few lines further down Table 5.2A.2.1-1, which contains CA_n40-n41. This combination has no NOTE and is ideal for comparison to CA_n38-n40, since n38 is contained within the operation band of n41, both being TDD bands as well.

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n38-n409 | n38, n40 |  |
| CA_n40-n41 | n40, n41 |  |

The lack of NOTE at CA_n40-n41 may lead to confusion for readers of 38.101-1 since it is missing a reference to NOTE 9.

Observation 1: The lack of a NOTE at CA_n40-n41 may lead to confusion for readers of 38.101-1 since it is missing a reference to NOTE 9.

Proposal 1: It is proposed to correct CA_n40-n41 of 38.101-1 Table 5.2A.2.1-1 to include the reference to NOTE 9, same as asked for CA_n38-n40

Note a CR introducing the proposed correction have been provided as R4-2213223

# Conclusion

Observation 1: The lack of a NOTE at CA_n40-n41 may lead to confusion for readers of 38.101-1 since it is missing a reference to NOTE 9.

Proposal 1: It is proposed to correct CA_n40-n41 of 38.101-1 Table 5.2A.2.1-1 to include the reference to NOTE 9, same as asked for CA_n38-n40

# Reference

- R4-2209929, TP to TR 38.717-02-01: Addition of CA_n38-n40, TSG RAN WG4 Meeting #103-e