3GPP RAN WG4 Meeting #104-e		R4-2213232

Electronic meeting, 15th – 26th August, 2022

Agenda item:	9.14.1

Source:	Apple Inc.

Title:	SSB side conditions for band n263

WI/SI:	NR_ext_to_71GHz-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

In this contribution we provide the side conditions for beam correspondence for band n263. Similar as in FR2-1, we have used the defined equation from [1] for the derivation of minimum SSB for FR2-2. Our calculation for the SSB side condition for band n263 is explained in this contribution.

# 2	Discussion

According to the specification [1], the following equation is used for the derivation of minimum SSB_RP value. The aim of this contribution is to estimate the SSB side condition for band n263. In the example below we consider band n263 and its parameters. In RRM the measurement accuracy requirement is defined under “coarse beam” assumption, whereas in RF the assumption for beam correspondence is “fine beam”.

Minimum SSBRP=EIS spherical coveragePC3,n263,100 MHz+Z-10*log10PRBREFSENSx12- SNRREFSENS+SSBÊsIot + ∆MBs,n

Unlike the bands in FR2-1, band n263 single carrier channel BW starts at 100 MHz. Therefore, we have used a CBW of 100 MHz for the calculation in the equation above.

EIS spherical coverage PC3, n263, 100 MHz is the EIS spherical coverage value in dBm specified for power class 3 in Band n263 for 100MHz channel bandwidth as defined in [2]. Z is the gain difference between fine and rough beams. It is worth noticing that for the calculation of min SSB_RP in the the RF requirements for beam correspondence the assumption is fine beam, therefore, to derive the SNR condition for minimum SSBRP the 7 dB relaxation (Z) is not required.

The number of subcarriers in a PRB is 12 and RBRefsens is NRB associated with subcarrier spacing 120 kHz for 100MHz in [2]. SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage. SSB Ês/Iot is the minimum value required by the UE to perform measurements and is defined as -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. ∆MBS,n is the UE multi-band relaxation factor value in dB specified in [2].

For band n263 we have applied the following parameters as specified in [2]:

- EIS spherical coverage = -66.2 dBm/100 MHz,

- PRB_REFSENS = 66

- SNR_REFSENS = -1 dB

- SSB Ês/Iot = 6 dB

According to the Note in [2] the UEs that support multiple FR2 bands, the Minimum SSBRP values for all angles are increased by ΣMBS. Taking into account the list of parameters the minimum SSB for n263 should be defined as -88.2 dBm.

Table 1and Table 2 includes band n263 for the side conditions for SSB and for CSI-RS, respectively.

Table 1: Conditions for SSB based L1-RSRP measurements for beam correspondence

| Band | Minimum SSB (dBm/SCSSBB) |
| --- | --- |
| n257 | -96.2 |
| n258 | -96.2 |
| n259 | -90.7 |
| n260 | -91.9 |
| n261 | -96.2 |
| n262 | -88.5 |
| n263 | -88.2 |

Table 2: Conditions for CSI-RS based L1-RSRP measurements for beam correspondence

| Band | Minimum CSI-RS (dBm/SCSSBB) |
| --- | --- |
| n257 | -96.2 |
| n258 | -96.2 |
| n259 | -90.7 |
| n260 | -91.9 |
| n261 | -96.2 |
| n262 | -88.5 |
| n263 | -88.2 |

Our proposal is to consider the values for band n263 from Table 1 and Table 2 in the specification for the conditions for SSB based L1-RSRP measurements for beam correspondence and the conditions for CSI-RS based L1-RSRP measurements for beam correspondence, respectively.

Proposal 1:	RAN4 shall apply the minimum SSB and minimum CSI-RS as provided in Table 1 and Table 2 for band n263.

# 3	Conclusions

This contribution provides the corrected values for the conditions for SSB based L1-RSRP measurements for beam correspondence and the conditions for CSI-RS based L1-RSRP measurements for beam correspondence. In summary, we have made the following proposal.

Proposal 1:	RAN4 shall apply the minimum SSB and minimum CSI-RS as provided in Table 1 and Table 2 for band n263.

# 4	References

3GPP TS 38.133 V17.5.0

3GPP TS 38.101-2 V17.5.0