3GPP TSG-RAN4 Meeting # 104-e	R4-2213241

# Electronic Meeting,  15th  – 26th  August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR | <CR#> | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Test applicability for eIAB conformance testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh- Perf |  |  |  |  |  | Date: |  |  | 2022-8-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RF requriements are introduced in Rel-17 for IAB simultaneous operation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the test applicability clause for for IAB simultaneous operation test |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no test applicability for IAB simultaneous operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.3, 4.8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < start of changes >

### 4.8.3	Applicability of test configurations for single-band operation

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to clause 4.6. The generation and power allocation for each test configuration is defined in clause 4.7. This clause contains the test configurations for an IAB node capable of single carrier, multi-carrier and/or CA operation in both contiguous and non-contiguous spectrum in single band.

For an IAB node declared to be capable of single carrier operation only (D.16), a single carrier (SC) shall be used for testing.

For an IAB node declared to support multi-carrier and/or CA operation in contiguous spectrum within a single band (D.15-D.16), the test configurations in the second column of table 4.8.3-1 shall be used for testing.

For an IAB node declared to support multi-carrier and/or CA operation in contiguous and non-contiguous spectrum within a single band (D.15-D.16) and where the parameters in the manufacture's declaration according to clause 4.6 are identical for contiguous (C) and non-contiguous (NC) spectrum operation (D.9), the test configurations in the third column of table 4.8.3-1 shall be used for testing.

For an IAB node declared to support multi-carrier and/or CA in operation contiguous and non-contiguous spectrum within a single band (D.15-D.16) and where the parameters in the manufacture's declaration according to clause 4.6 are not identical for contiguous (C) and non-contiguous (NC) spectrum operation (D.9), the test configurations in the fourth column of table 4.8.3-1 shall be used for testing.

For an IAB node declared to support IAB simultaneous operation (D.x) in contiguous and non-contiguous spectrum within a single band (D.15-D.16) and where the parameters in the manufacture's declaration according to clause 4.6 are identical for contiguous (C) and non-contiguous (NC) spectrum operation (D.9), the test configurations in the third column of table 4.8.3-2 shall be used for testing of simultaneous operation.

For an IAB node declared to support IAB simultaneous operation (D.x) in contiguous and non-contiguous spectrum within a single band (D.15-D.16) and where the parameters in the manufacture's declaration according to clause 4.6 are not identical for contiguous (C) and non-contiguous (NC) spectrum operation (D.9), the test configurations in the fourth column of table 4.8.3-2 shall be used for testing of simultaneous operation.

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported channel bandwidth and the smallest supported sub-carrier spacing.

Table 4.8.3-2: Test configurations for a IAB capable of simultaneous operation in a single band

| IAB test case | Contiguous spectrum capable IAB | C and NC capable IAB with identical parameters | C and NC capable IAB with different parameters |
| --- | --- | --- | --- |
| Output power | IABTC1 | IABTC1 | IABTC1, IABTC3 |
| Transmit ON/OFF power (only applied to NR TDD IAB) | IABTC1 | IABTC1 | IABTC1, IABTC3 |
| Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Error Vector Magnitude | IABTC1 | IABTC1 | IABTC1, IABTC3 |
| Time alignment error | IABTC1 | IABTC1 | IABTC1, IABTC3 |
| Adjacent Channel Leakage power Ratio (ACLR) | IABTC1 | IABTC3 | IABTC1, IABTC3 |
| Cumulative ACLR requirement in non-contiguous spectrum | - | IABTC3 | IABTC3 |
| Operating band unwanted emissions | IABTC1 (Note 2) | IABTC1, IABTC3 (Note 2) | IABTC1, IABTC3 (Note 2) |
| Transmitter spurious emissions | IABTC1 | IABTC3 | IABTC1, IABTC3 |
| Transmitter intermodulation | IABTC1 | IABTC1, IABTC3 | IABTC1, IABTC3 |
| Adjacent Channel Selectivity (ACS) | IABTC1 | IABTC3 | IABTC1, IABTC3 |
| In-band blocking | IABTC1 | IABTC3 | IABTC1, IABTC3 |
| Out-of-band blocking | IABTC1 | IABTC3 | IABTC1, IABTC3 |
| Receiver spurious emissions | IABTC1 | IABTC3 | IABTC1, IABTC3 |
| Receiver intermodulation | IABTC1 | IABTC3 | IABTC1, IABTC3 |
| Note 1: IABTC2 is only applicable when contiguous CA is supported. Note 2: OBUE SC shall be tested using the widest supported channel bandwidth and the highest supported sub-carrier spacing. |  |  |  |

### 4.8.4	Applicability of test configurations for multi-band operation

For an IAB node declared to be capable of multi-band operation, the test configuration in table 4.8.4-1 and/or table 4.8.3-1 shall be used for testing. In the case where multiple bands are mapped on common multi-band connector, the test configuration in the second column of table 4.8.4-1 shall be used. In the case where multiple bands are mapped on common single-band connector, the test configuration in table 4.8.3-1 shall be used. In the case where multiple bands are mapped on separate single-band connector or multi-band connector, the test configuration in the third column of table 4.8.4-1 shall be used.

For an IAB node declared to be capable of IAB simultaneous operation of multi-band operation, the test configuration in table 4.8.4-1 and/or table 4.8.3-1 shall be used for testing. In the case where multiple bands are mapped on common multi-band connector, the test configuration in the second column of table 4.8.4-1 shall be used. In the case where multiple bands are mapped on common single-band connector, the test configuration in table 4.8.3-1 shall be used. In the case where multiple bands are mapped on separate single-band connector or multi-band connector, the test configuration in the third column of table 4.8.4-2 shall be used.

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported channel bandwidth and the smallest supported sub-carrier spacing.

Table 4.8.4-2: Test configuration for a IAB capable of simultaneous operation in multi-band operation

| IAB test case | Test configuration |  |
| --- | --- | --- |
|  | Common connector | Separate connectors |
| Output power | IABTC1/3 (Note 1), IABTC4 | IABTC1/3 (Note 1), IABTC4 |
| RE Power control dynamic range (only applied to IAB-DU) | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Transmit ON/OFF power (only applied to NR TDD IAB) | IABTC4 | IABTC4 |
| Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Error Vector Magnitude | IABTC1/3 (Note 1), IABTC4 | IABTC1/3 (Note 1), IABTC4 |
| Time alignment error (only applied to IAB-DU) | IABTC1/3 (Note 1), IABTC5 (Note 2) | IABTC1/3 (Note 1), IABTC5 (Note 2) |
| Adjacent Channel Leakage power Ratio (ACLR) | IABTC1/3 (Note 1), IABTC5 (Note 4) | IABTC1/3 (Note 1, 5), IABTC5 (Note 4, 5) |
| Cumulative ACLR requirement in non-contiguous spectrum | IABTC3 (Note 1), IABTC5 (Note 4) | IABTC3 (Note 1, 5) |
| Operating band unwanted emissions | IABTC1/3 (Note 1), IABTC5, SC (Note 7) | IABTC1/3 (Note 1, 5), IABTC5 (Note 5) |
| Transmitter spurious emissions | IABTC1/3 (Note 1), IABTC5 | IABTC1/3 (Note 1, 5), IABTC5 (Note 5) |
| Transmitter intermodulation | IABTC1/3 (Note 1) | IABTC1/3 (Note 1, 5) |
| Adjacent Channel Selectivity (ACS) | IABTC5 | IABTC1/3 (Note 1), IABTC5 (Note 6) |
| In-band blocking | IABTC5 | IABTC1/3 (Note 1), IABTC5 (Note 6) |
| Out-of-band blocking | IABTC5 | IABTC1/3 (Note 1), IABTC5 (Note 6) |
| Receiver spurious emissions | IABTC1/3 (Note 1), IABTC5 | IABTC1/3 (Note 1, 5), IABTC5 (Note 5) |
| Receiver intermodulation | IABTC5 | IABTC1/3 (Note 1), IABTC5 (Note 6) |
| Note 1: IABTC1 and/or IABTC3 shall be applied in each supported operating band. Note 2: IABTC5 is only applicable when inter-band CA is supported. Note 3: Void. Note 4: IABTC5 may be applied for Inter RF Bandwidth gap only. Note 5: For single-band operation test, other TAB connector(s) is (are) terminated. Note 6: IABTC5 is only applicable for multi-band receiver. |  |  |

## < end of changes >