# 3GPP TSG-RAN WG4 Meeting # 104-e						R4-2213243

# Electronic Meeting, 15th  –26th August, 2022

Source:		Ericsson

Title:			On IoT NTN raster

Agenda item:		12.5.2

Document for:		Approval

# Introduction

In [1], the RF part objective is stated as below:

- Specification of RF requirements for Satellite Access Node (SAN) and UE including the following [RAN4]:

- Specification of a 200 kHz channel raster in bands where this is feasible.

- In bands where it is not feasible to define a 200 kHz channel raster, the specification of a 100 kHz channel raster to be used in conjunction with signalling of the “part-of EARFCN” indication on MIB, with multiple EARFCN hypotheses.

- Verification of co-existence of IoT NTN with TN, re-using or extrapolating from existing coexistence results (from NR NTN or other) where appropriate, and considering additional simulations as necessary.

- Leveraging the studies and requirements (where applicable) of NTN NR bands n256 and n255 (and any relevant E-UTRA bands), specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL)

  - L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

In this paper, we present our view on the IoT NTN raster design.

# Discussion

In WID[1], the verification of co-existence of IoT NTN and TN is stated as one investigation:

- Specification of a 200 kHz channel raster in bands where this is feasible.

- In bands where it is not feasible to define a 200 kHz channel raster, the specification of a 100 kHz channel raster to be used in conjunction with signalling of the “part-of EARFCN” indication on MIB, with multiple EARFCN hypotheses.

According to WID, the framework and requirement of the legacy NB-IoT/eMTC shall be reused, this should be including the raster definition.

The specification work of this WI shall:

- Be limited to FDD requirements applicable for NB1/NB2 and Cat-M1 UE categories (HD-FDD and FD-FDD for Cat-M1).

- Re-use the framework and requirements from NB-IoT/eMTC, as well as NR NTN requirements, where applicable.

As the WID does not give any motivation of the channel raster change to 200kHz or 100kHz with signalling of “part-of EARFCN”, it is necessary to have RAN4 understanding on it.

Figure 1:  Ambiguity of channel assignment from channels at 100 (n+1) or 100 (n+2).

When the UE receive the carrier with doppler shift , it may be difficult to know where the carrier is from raster point of 100 (n+1) or 100 (n+2) as shown in Figure 1, the centre frequency distance of the two channels will be 100kHz. This generates the channel assignment ambiguity and either increasing channel raster size to 200kHz or with a “part-EARFCN” signalling could solve the ambiguity issue.

The 200kHz channel raster or 100kHz raster with signalling is to solve the channel assignment ambiguity.

Align the RAN4 understanding on introducing the 200kHz or reusing 100kHz raster with EARFCN signalling.

For NB-IoT, introducing the 200kHz channel raster needs some discussion considering Foffset definition for NB-IoT. Currently it is defined as 200kHz in Table 5.6-3A in 36.104.  The Foffset is the defined as the separation between the channel edge frequency and the centre of the measuring filter and visualized in Figure 7.1.6.2-1 [7].

Table 5.6-3A: Foffset for NB-IoT standalone operation

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | 200 kHz |

Figure 7.1.6.2-1[7]: ACLR for NB-IoT standalone

In 36.104the Base Station ACLR for standalone NB-IoT operation in paired spectrum is described as below.

Table 6.6.2.1-2b: Base Station ACLR for standalone NB-IoT operation in paired spectrum

| Channel bandwidth of NB-IoT lowest/highest carrier transmitted BWChannel [kHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 200 | 300 kHz | Standalone NB-IoT | Square (180 kHz) | 40 dB |
|  | 500 kHz | Standalone NB-IoT | Square (180 kHz) | 50 dB |

The different operators need to respect the Foffset when deploying the standalone mode carrier for NB-IoT and thus the 100kHz guard band between two NB-IoT carriers from different operator would be needed. The guard band between two NB-IoT with 200kHz raster will be 200kHz which is not spectrum efficient from this aspect.

Guard band between NB-IoT carriers for different operator would increase from 100kHz to 200kHz to respect Foffset and accommodate the raster design.

- Therefore, we think the 100kHz raster design with signaling option would be better choice.

- If the 100kHz raster is kept the same as NB-IoT TN UE, the clause in TS 36.101 for carrier frequency and EARFCN for NB1 and NB2 TN UE could be reused for IoT NTN. The carrier frequency of category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143 and the Offset of category NB1/NB2 Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD. As the standalone should be prioritized, the MDL (-0.5) could be reused according to note 4 inTS 36.101.

NOTE 4:	For the carrier including NPSS/NSSS for stand-alone operation, MDL = -0.5.

- The MDL definition in NOTE 4 is for anchor carrier for legacy NB-IoT, for non-anchor carrier, the MDL is selected from {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD according to legacy NB-IoT requirement. The legacy NB-IoT raster DL offset may be defined using the reference to the in-band and guard-band operation and orthogonality between any NB carrier is guaranteed by placing it in LTE RB grid. For standalone deployment and keeping orthogonality between the NB carriers,  it is only possible with MDL =-0.5 as shown in Figure 1. Nonetheless, there will be 8 unused subcarriers between anchor and non-anchor carriers and this will fragment the spectrum usage. To avoid this, a different MDL needs to be defined.

In the legacy NB-IoT specification, MDL is selected from {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD, which results in unutilized spectrum resources for orthogonal arrangement between anchor and non-anchor carrier since between anchor and non-anchor carriers there are 8 unused subcarriers as the closest possible selection.

- Figure 1: Example of closest standalone anchor and non-anchor carriers deployment

Table 1 lists the MDL value with different number of empty subcarriers between anchor and non-anchor carrier, the offset from the anchor carrier, and the offset from 100KHz raster for non-anchor carrier (MDL) for stand-alone deployment.

Table 1: MDL with the number of empty subcarriers between anchor and non-anchor, offset from anchor carrier, and offset from 100KHz channel raster.

| Empty subcarriers between Anchor and non-anchor | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offset from Anchor(KHz) | 180 | 195 | 210 | 225 | 240 | 255 | 270 | 285 | 300 |
| Offset from 100KHz raster(KHz) | -20 | -5 | 10 | 25 | 40 | -45 | -30 | -15 | 0 |
| MDL | -4.5 | -1.5 | 1.5 | 4.5 | 7.5 | -9.5 | -6.5 | -3.5 | -0.5 |

| Empty subcarriers between Anchor and non-anchor | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offset from Anchor(KHz) | -180 | -195 | -210 | -225 | -240 | -255 | -270 | -285 | -300 |
| Offset from 100KHz raster(KHz) | 20 | 5 | -10 | -25 | -40 | 45 | 30 | 15 | 0 |
| MDL | 3.5 | 0.5 | -2.5 | -5.5 | -8.5 | 8.5 | 5.5 | 2.5 | -0.5 |

- So, from the Table 1, it can be observed that a new set of MDL needs to be introduced to have the integer of subcarrier (0 to 8) between two NB-IoT carriers, and the set of MDL is {-9.5, -8.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 7.5, 8.5}.

In Rel-18 for NB-IoT over NTN, MDL is selected from {-9.5, -8.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 7.5, 8.5} for FDD, which opens the possibility of having no unused subcarriers (i.e., no unutilized resources) between anchor and non-anchor carriers with orthogonality design.

- Compared with the MDL set currently in the specification {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD, the above new MDL set that allows having 0 to 8 subcarriers between anchor and non-anchor carrier is offset with 0.5 and thus the new M-DL value could be derived from the formula:

- MDL = MDL + 0.5The above formula means that the UE could derive a new value (i.e., 0 to 8 subcarriers between anchor and non-anchor carrier) when the legacy offset value is signalled from network. In specification, this could be done by adding new note below:

- NOTE 5: For the carrier not including NPSS/NSSS for stand-alone operation, MDL = MDL + 0.5.

Introducing a new set value of MDL {-9.5, -8.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 7.5, 8.5}. Alternatively, re-using the existing set introducing a new note for deriving a new MDL value defining MDL = MDL + 0.5.

# Conclusions

In this contribution, we present our view on the raster design for IoT NTN UE with below observations and proposal:

- The 200kHz channel raster or 100kHz raster with signalling is to solve the channel assignment ambiguity.

Guard band between NB-IoT carriers for different operator would increase from 100kHz to 200kHz to respect Foffset and accommodate the raster design.

In the legacy NB-IoT specification, MDL is selected from {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD, which results in unutilized spectrum resources for orthogonal arrangement between anchor and non-anchor carrier since between anchor and non-anchor carriers there are 8 unused subcarriers as the closest possible selection.

In Rel-18 for NB-IoT over NTN, MDL is selected from {-9.5, -8.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 7.5, 8.5} for FDD, which opens the possibility of having no unused subcarriers (i.e., no unutilized resources) between anchor and non-anchor carriers with orthogonality design.

- Align the RAN4 understanding on introducing the 200kHz or reusing 100kHz raster with EARFCN signalling.

- Introducing a new set value of MDL {-9.5, -8.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 7.5, 8.5}. Alternatively, re-using the existing set introducing a new note for deriving a new MDL value defining MDL = MDL + 0.5.

# References

- RP-220938, New WID on NB-IoT/eMTC core & performance requirements for NTN, MediaTek Inc

- 36.763, Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)

- TS 38.101-5, NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

- TS 36.101,Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception

- TR 38.863, Non-terrestrial networks (NTN) related RF and co-existence aspects

- R1-2103962, Summary #3 of AI 8.15.1 Scenarios applicable to NB-IoT/eMTC

- TR 38.802, 	Evolved Universal Terrestrial Radio Access (E-UTRA); NB-IOT; Technical Report for BS and UE radio transmission and reception