# 3GPP TSG-RAN WG4 Meeting # 104-e						R4-2213245

# Electronic Meeting, 15th  –26th August, 2022

Source:		Ericsson

Title:			SAN RF impact overview

Agenda item:		12.5.3

Document for:		Approval

# Introduction

In [1], the RF part objective is stated as below:

- Specification of RF requirements for Satellite Access Node (SAN) and UE including the following [RAN4]:

- Specification of a 200 kHz channel raster in bands where this is feasible.

- In bands where it is not feasible to define a 200 kHz channel raster, the specification of a 100 kHz channel raster to be used in conjunction with signalling of the “part-of EARFCN” indication on MIB, with multiple EARFCN hypotheses.

- Verification of co-existence of IoT NTN with TN, re-using or extrapolating from existing coexistence results (from NR NTN or other) where appropriate, and considering additional simulations as necessary.

- Leveraging the studies and requirements (where applicable) of NTN NR bands n256 and n255 (and any relevant E-UTRA bands), specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL)

  - L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

In this paper, we present our view on the SAN RF requirement overview.

# Discussion

In WID[1], there is additional description as below:

All requirements specified as part of this WI shall be Release-independent from Rel-17, and generic requirements defined for the above frequency bands shall be used as a basis for definition (in a later Release-independent work item) of requirements for future E-UTRA bands for IoT NTN operation.

The specification work of this WI shall:

- Be limited to FDD requirements applicable for NB1/NB2 and Cat-M1 UE categories (HD-FDD and FD-FDD for Cat-M1).

- Re-use the framework and requirements from NB-IoT/eMTC, as well as NR NTN requirements, where applicable.

NOTE: Rel-17 IoT NTN specifications do not cover non-NTN NB-IoT/eMTC functionality defined later than Rel-16

- As the objective require the release -independent from Rel-17,the scope of the Rel-17 WID should be inherited in Rel-18 RAN4 WID. For example, the scope of WID[1] has relevant part in WID[4]:

- Standalone deployment for NB-IoT / eMTC (i.e. operating in carrier(s) used only for NB-IoT NTN (resp. eMTC NTN)) for support in Rel-17 timeframe will be prioritized.

Standalone deployment for NB-IoT / eMTC should be priorizted for IoT NTN to be release independent from Rel-17.

- In legacy NB-IoT and eMTC specification, the conducted requirement is specified in 36.104 and BS type 1-C is specified for NB-IoT In-band operation in 38.104. The NTN SAN specifies only type 1-H and type 1-O which means the 36.104 specification cannot be reused directly for IoT NTN.  For type 1-O, it is not specified for NB-IoT and eMTC before. NTN SAN specification in this case could be used as starting point for requirement specification.

NTN SAN specification could be based for specifying the IoT NTN.

- For other specification impact, the analysis is provided in below table.

- Table 1: NTN SAN Type 1-H specification updates to support IoT NTN

| - Requirement | - Requirement clause | - Updates to support SA NB-IoT | - Updates to support SA Cat-M1 |
| --- | --- | --- | --- |
| - Satellite Access Network output power | - 6.2 | - reuse | - reuse |
| - Output power dynamics | - 6.3 | - N/A | - Add 1.4MHz channel BW for total power dynamic range, remove other channel BW |
| - Transmit ON/OFF power | - NA | - N/A | - N/A |
| - Frequency error | - 6.5.1 | - reuse | - reuse |
| - Modulation quality | - 6.5.2 | - Update for NB-PDSCH, QPSK and 16QAM | - reuse |
| - Time alignment error | - NA | - N/A | - N/A |
| - DL RS power | - Not exist | - To be added acc. clause 6.5.4 in 36.104 | - To be added acc. clause 6.5.4 in 36.104 |
| - Occupied bandwidth | - 6.6.2 | - reuse | - reuse |
| - ACLR | - 6.6.3 | - Depend on co-existing discussion/simulation | - Depend on co-existing discussion/simulation |
| - Operating band unwanted emissions | - 6.6.4 | - Check if reuse if ACLR reused | - Check if reuse if ACLR reused |
| - Transmitter spurious emissions | - 6.6.5 | - Reuse | - reuse |
| - Transmitter intermodulation | - NA | - Follow the same with NTN SAN | - Follow the same with NTN SAN |
| - Reference sensitivity level | - 7.2 | - Add NB-IoT channel BW and REFSENS | - Add 1.4MHz channel BW and REFRENS, remove other channel BW |
| - Dynamic range | - 7.3 | - Add NB-IoT channel BW | - Add 1.4MHz channel BW |
| - ACS | - 7.4.1 | - Depend on co-existing simulation | - Depend on co-existing simulation |
| - In-band blocking | - NA | - Follow the same with NTN SAN | - Follow the same with NTN SAN |
| - Out-of-band blocking | - 7.5 | - reuse | - reuse |
| - Receiver spurious emissions | - 7.6 | - reuse | - reuse |
| - Receiver intermodulation | - NA | - Follow the same with NTN SAN | - Follow the same with NTN SAN |
| - In-channel selectivity | - 7.8 | - reuse | - reuse |

- Table 2: NTN SAN Type 1-O specification updates to support IoT NTN

| - Requirement | - Requirement clause | - Updates to support SA NB-IoT | - Updates to support Cat-M1 |
| --- | --- | --- | --- |
| - Radiated transmit power | - 9.2 | - Reuse | - reuse |
| - OTA Satellite Access Network output power | - 9.3 | - Reuse | - reuse |
| - OTA output power dynamics | - 9.4 | - Reuse | - reuse |
| - OTA transmit ON/OFF power | - NA | - N/A | - N/A |
| - OTA frequency error | - 9.6.1 | - Reuse | - reuse |
| - OTA modulation quality | - 9.6.2 | - Same as the type 1-H | - Same as the type 1-H |
| - OTA time alignment error | - NA | - N/A | - N/A |
| - OTA occupied bandwidth | - 9.7.2 | - Reuse | - reuse |
| - OTA ACLR | - 9.7.3 | - Depend on co-existing simulation | - Depend on co-existing simulation |
| - OTA out-of-band emission | - 9.7.4 | - Check if reuse with ACLR result | - Check if reuse with ACLR result |
| - OTA transmitter spurious emission | - 9.7.5 | - reuse | - reuse |
| - OTA transmitter intermodulation | - NA | - Follow the same with NTN SAN | - Follow the same with NTN SAN |
| - OTA sensitivity | - 10.2 | - Reuse | - reuse |
| - OTA reference sensitivity level | - 10.3 | - Add NB-IoT channel BW and REFSENS | - Add 1.4MHz channel BW and REFRENS, remove other channel BW |
| - OTA dynamic range | - 10.4 | - Add NB-IoT channel BW | - Add 1.4MHz channel BW |
| - OTA ACS | - 10.5.1 | - Depend on co-existing simulation | - Depend on co-existing simulation |
| - OTA in-band blocking | - NA | - N/A | - N/A |
| - OTA out-of-band blocking | - 10.6 | - Reuse | - reuse |
| - OTA receiver spurious emission | - 10.7 | - Reuse | - reuse |
| - OTA receiver intermodulation | - NA | - N/A | - N/A |
| - OTA in-channel selectivity | - 10.9 | - Reuse | - reuse |

- For the general clause clause 5, the channel raster needs to be discussed separately as in WID, there is one objective to design a 200kHz channel raster. The analysis will be provided in separated companion paper.

Consider the above potential updates for new SAN specification for IoT SAN based on NTN SAN TS 38.108.

# Conclusions

In this contribution, we present our view on the RF requirement overview for SAN node with below observations and proposal:

- Standalone deployment for NB-IoT / eMTC should be priorizted for IoT NTN to be release independent from Rel-17.

NTN SAN specification could be based for specifying the IoT NTN.

- Consider the above potential updates for new SAN specification for IoT SAN based on NTN SAN TS 38.108.

# References

- RP-220938, New WID on NB-IoT/eMTC core & performance requirements for NTN, MediaTek Inc

- 36.763, Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)

- TS 38.108, NR; Satellite Access Node radio transmission and reception

[4]	RP-211601, WID on NB-IoT/eMTC support for NTN, MediaTek Inc., Eutelsat S.A.