3GPP TSG-RAN WG4 Meeting # 104-e												R4-2213253

Electronic Meeting, 15 – 26 August, 2022

Agenda item:			9.19.1.1

Source:	Ericsson

Title:	Applicability of Rx/RxTx TEG (core part)

Document for:	Discussion

# Introduction

In this contribution applicability of Rx/RxTx TEG is discussed. The discussion is based on the agreement on Rx/RxTx TEG framework agreed in RAN4 and applicability of Rx/RxTx TEG agreed in RAN1. In addition issue related to UE Rx/RxTx TEG margin reporting raised by RAN1 in R1-2205382 reply LS on the UE/TRP TEG framework has been addressed.

# Discussion

## Applicability of Rx/RxTx TEG

RAN4#102-e agreed on the following Rx TEG framework [1]:

Framework of UE/TRP Rx TEG

Agreements:

- The framework of UE/TRP Rx TEG

  - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec.

    - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part.

  - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF.

  - For UE that supports multiple Rx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Rx TEG is M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M.

  - The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID.

  - The RRM accuracy requirements corresponding to the candidate timing error margin values {TE1, TE2, …, TEN} will be defined in Perf part.

- The framework of UE/TRP Rx TEG can be also applied for UE/TRP RxTx TEG.

  - Note: if additional issues are identified based on RAN1/2 progress, then this agreement can be revised.

Based on the agreement the applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided. The agreement further mandates that the UE reported TEG applies only to the measurements that are tagged with the corresponding TEG ID. It was also agreed to revise the agreement based on the issues identified based on RAN1/2 progress.

RAN1#109-e agreed on the reply LS on the UE/TRP TEG framework [2]. Issue #2 in the reply LS is relevant to the applicability of the reported Rx/RxTx TEG that specifies the RAN1 agreement on the issue. Based on RAN1 agreement, when the TEG feature is combined with the reporting of multiple measurement instances, the applicability of a reported Rx/RxTx TEG is limited to the measurements contained within the single measurement instance of a measurement report in which the RX/RxTx TEG information is provided, and only to measurements that are tagged with the corresponding Rx/RxTx TEG ID [3].

Observation 1: Based on the RAN1 agreement, applicability of RX/RxTx TEG needs to be updated.

Proposal 1: The applicability of reported UE Rx/RxTx TEG is limited to the measurements contained within the single measurement instance report in which the Rx/RxTx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID.

## Rx/RxTx TEG margin reporting

RAN1 has asked RAN4 to provide input on whether UE Rx/RxTx TEG margins are provided to LMF as UE capability, or as LPP signalling parameters outside of UE capability signaling.

RAN4#103-e discussed and made the following agreement on issues related to reported value for Tx/Rx/RxTx TEGs:

Whether the reported value for Tx TEGs, Rx TEGs and RxTx TEGs can be different?

Agreement:

- The reported value for Tx TEGs, Rx TEGs and RxTx TEGs can be different.

Whether the reported value for Tx/Rx/RxTx TEGs can be different at different times?

Agreement:

- The reported value for the timing error margin of Tx/Rx/RxTx TEGs can be different at different times.

Based on the agreement the reported value for Tx TEGs, Rx TEGs and RxTx TEGs can be different. And the reported value for the timing error margin of Tx/Rx/RxTx TEGs can be different at different times. Defining whether UE Rx/RxTx TEG margins are provided to LMF as UE capability, or as LPP signalling parameters outside of UE capability signaling is out of RAN4 scope. It is up to RAN2 to decide on the appropriate signalling for TEG margin value reporting.

Proposal 2: Respond to RAN1 LS (R1-2205382) as following:

Defining whether UE Rx/RxTx TEG margins are provided to LMF as UE capability, or as LPP signalling parameters outside of UE capability signaling is out of RAN4 scope. It is up to RAN2 to decide on the appropriate signalling for TEG margin value reporting.

# Summary

In this contribution, we presented our view on applicability margin reporting of Rx/RxTx TEG and proposed the following:

Proposal 1: The applicability of reported UE Rx/RxTx TEG is limited to the measurements contained within the single measurement instance report in which the Rx/RxTx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID.

Proposal 2: Respond to RAN1 LS as following:

Defining whether UE Rx/RxTx TEG margins are provided to LMF as UE capability, or as LPP signalling parameters outside of UE capability signaling is out of RAN4 scope. It is up to RAN2 to decide on the appropriate signalling for TEG margin value reporting.

# References

- R4-2206997, “WF on NR Positioning Enhancements (Part 2)”, CATT.

- R1-2205382, “Reply LS on the UE/TRP TEG framework”, CATT.

- R1-2202922, “LS on multiple measurement instances”, CATT.