3GPP TSG-RAN WG4 Meeting # 104-e												R4-2213254

Electronic Meeting, 15 – 26 August 2022

Agenda item:			9.19.1.4

Source:	Ericsson

Title:	Remaining issues on impact on RRM requirements

Document for:	Discussion

# Introduction

In RAN4#103-e issues related to PRS measurement period starting point and requirements for PRS measurement outside MG were discussed. As an output of the discussion agreements on some aspects of the issues were only reached. The remaining aspects of these issues were identified FFS. In this contribution we present our view on the FFS parts.

# Discussion

## Requirements for PRS measurement outside MG

RAN4#103-e made the following agreement on requirement for PRS measurement with POS MG [1]. The requirement for PRS measurement outside MG is FFS.

Agreements:

- For PRS measurement with POS MG:

  - the PRS requirements apply provided that only one POS MG is activated, and the RRM requirements do not apply for RRM measurement that requires MG.

FFS: For PRS measurement outside MG, the PRS requirements apply provided that no POS MG is activated during the measurement period.

For gapless PRS measurement a PPW is configured during which the UE performs measurement on PRS from the reference and the target TRPs for positioning. When a PPW is configured, network does not activate a POS MG to this UE. Therefore, from the RAN4 point of view, it is enough to define requirement assuming that no POS MG is activated during the gapless PRS measurement period.

# Observation 1: No POS MG is activated for UEs configured to measure PRS within PPW.

# Proposal 1: For PRS measurement outside MG, the PRS requirements apply provided that no POS MG is activated during the measurement period.

## PRS measurement period starting point

RAN4#103-e concluded the discussion on the starting point of positioning measurement period with the following agreement [2]:

Agreement:

- For the event of periodic location, FFS whether UE should start measurement after T or complete measurement before T, where T is the time that the event configured for deferred MT-LR occurs

  - If UE should complete measurement before T, FFS the start of measurement period and applicability condition

- For the event other than periodic location, UE should start measurement after T, where T is the time that the event configured for deferred MT-LR occurs.

TS 37.355 defines scheduled location time (T) irrespective of whether the event is periodic location or other than periodic location [3]. Therefore, the PRS measurement period starting point shall be defined as a general condition that applies when the UE is provided with ScheduledLocationTime T as a part of ScheduledLocationRequest.

# Proposal 2: UE shall start PRS measurement after T, provided that the UE has received ScheduledLocationTime T as a part of ScheduledLocationRequest.

# Summary

In this paper we presented out view on requirement for PRS measurement outside measurement gap and PRS measurement period starting point and made the following observation and proposals:

# Observation 1: No POS MG is activated for UEs configured to measure PRS within PPW.

# Proposal 1: For PRS measurement outside MG, the PRS requirements apply provided that no POS MG is activated during the measurement period.

# Proposal 2: UE shall start PRS measurement after T, provided that the UE has received ScheduledLocationTime T as a part of ScheduledLocationRequest.

# References

[1] R4-2210600, “WF on NR Positioning Enhancements (Part 1)”, Ericsson.

[2] R4-2210581, “WF on R15 and R16 RRM maintenance”, Huawei, Hisilicon.

[3] TS 37.355, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP) (Release 17).