3GPP TSG-RAN4 Meeting # 104-e	R4-2213257

# Electronic Meeting, 15-26, August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2516 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 measurement period requirement for gapless PRS measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Correction of applicable number of PFL during gapless PRS measurement in RRC_CONNECTED state. - Clarification on applicability of gapless PRS measurement period requirement in RRC_CONNECTED state. - Clarification on Rx beam sweeping factor for UEs supporting lower Rx beam sweeping factor. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Single PFL within active DL BWP is measured when UE is configured for gapless RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement. - Measeurement period requirement applicability for Cap 1A, Cap 1B, and Cap 2 UEs is defined. - Rx beam sweeping factor for gapless PRS measurement is configured by LFM to UEs that are capable of PRS measurement with lower Rx beam sweeping factor(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement period requirement for gapless RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.2.7, 9.9.3.6, and 9.9.4.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------START OF CHANGE 1---------------

#### 9.9.2.7	Measurements Period Requirements without Measurement Gaps

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], on a single positioning frequency layer within active DL BWP during the measurement period TRSTD_wo_gap,i defined as:

TRSTD_wo_gap,i=kmultiTEG,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

where:

NRxBeam is the scaling factor for Rx beam sweeping:

- NRxBeam,i = 1 if positioning frequency layer i is in FR1.

- NRxBeam,i = 8 if positioning frequency layer i is in FR2 and UE does not support [TBD, IE for support of lower Rx beam sweeping factor].

- NRxBeam,i = 8 if positioning frequency layer i is in FR2 and requested by LMF via lowerRxBeamSweepingThan8-FR2-r17 despite UE being capable of supporting [TBD, IE for support of lower Rx beam sweeping factor].

- NRxBeam,i = [TBD, IE for support of lower Rx beam sweeping factor] if positioning frequency layer i is in FR2, UE is capable of supporting [TBD, IE for support of lower Rx beam sweeping factor] and is requested by LMF via lowerRxBeamSweepingThan8-FR2-r17.

- NRxBeam,iNRxBeam,i NRxBeam,i NRxBeam,i.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

NPRS,islot is the maximum number of DL PRS resources in a single positioning frequency layer within active DL BWP configured in a slot.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with PPW are considered. For cap 1A and cap 1B UEs that report (N2, T2) in their capability signalling, Lavailable shall be calculated based on the unmuted PRS resource instances that fully or partially overlap with the first (T2-N2) ms of the PPW.

Nsample is the number of PRS RSTD samples and

if UE does not support supportedDL-PRS-ProcessingSamples, or UE is not indicated by LMF to perform measurement with reduced sample number, Nsample= 4;

if UE supports supportedDL-PRS-ProcessingSamples and is indicated by LMF to perform measurement with reduced sample number, Nsample= 1 if the following conditions are met; Nsample= 2 otherwise.

PRS bandwidth is within the active BWP, and

Difference between the serving cell SS-RSRP and neighbor cell/TRP PRS-RSRP is within [6] dB.

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer within the active DL BWP, including the sampling time and processing time, Tlast,i = FFS,

Teffect,i is the periodicity of the PRS RSTD measurement in thepositioning frequency layer within active DL BWP defined as:

Teffect,i = TiTTavailable_PRS,i*Tavailable_PRS,i

Where,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,PPWRPi, the least common multiple between TPRS,i and PPWRPi.

PPWRPi is the repetition periodicity of the PRS processing window applicable for measurements in positioning frequency layer within active DL BWP .

TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer within active DL BWP.

If more than one PRS periodicities are configured in the positioning frequency layer within active DL BWP, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity,

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note: For the purpose of calculating TPRS,i, only the PRS resources that meet the conditions for PRS measurement outside measurement gaps as defined in clause 9.9.1.2 are considered.

N is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].

The time TRSTD_wo_gap,i starts from the first instance of the activated PPW for measurement of positioning frequency layer within the active DL BWP aligned with a DL PRS resource(s) in the assistance data after both the NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34].

If during the measurement period, the PPW is re-configured or reactivated, the measurement period can be longer. When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

TRSTD,i

The measurement period requirement in this section applies provided UE indicates that PRS is higher priority than other signals within PRS processing window:

- For Cap 1A UEs: measurement period requirement in this clause applies when PRS has higher priority than all other DL signals/channels (per UE) present inside the PRS processing window instances comprising the measurement period. If higher-priority DL signals/channels (per UE) are present inside any of the PRS processing windows comprising the required measurement period, the measurement period can be extended.

- For Cap 1B UEs: measurement period requirement in this clause applies when PRS in FR1 has higher priority than all other DL signals/channels (in the same band) present inside the PRS processing window instances comprising the measurement period. If higher-priority DL signals/channels (in the same band) are present inside any of the PRS processing windows comprising the required measurement period, the measurement period can be extended.

- For Cap 2 UEs: measurement period requirement in this clause applies when PRS in FR1 has higher priority than all other DL signals/channels (in the same CC) that overlap in time with PRS symbols inside the PRS processing window instances comprising the measurement period. If higher-priority signals/channels (in the same CC) overlap in time with PRS symbols inside any of the PRS processing windows comprising the required measurement period, the measurement period can be extended.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements. The RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.

---------------END OF CHANGE 1---------------

---------------START OF CHANGE 2---------------

#### 9.9.3.6	Measurement Period Requirements without Measurement Gaps

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements as defined in TS 38.215 [4] without measurement gap, on configured positioning frequency layer, within TPRS-RSRP_wo_gap,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

Teffect,i

TPRS-RSRP,wo_gapi=NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

where

NRxBeam is the scaling factor for Rx beam sweeping:

- NRxBeam,i = 1 if positioning frequency layer i is in FR1.

- NRxBeam,i = 8 if positioning frequency layer i is in FR2 and UE does not support [TBD, IE for support of lower Rx beam sweeping factor].

- NRxBeam,i = 8 if positioning frequency layer i is in FR2 and requested by LMF via lowerRxBeamSweepingThan8-FR2-r17 despite UE being capable of supporting [TBD, IE for support of lower Rx beam sweeping factor].

- NRxBeam,i = [TBD, IE for support of lower Rx beam sweeping factor] if positioning frequency layer i is in FR2, UE is capable of supporting [TBD, IE for support of lower Rx beam sweeping factor] and is requested by LMF via lowerRxBeamSweepingThan8-FR2-r17.

- NRxBeam,i NRxBeam,iNRxBeam,iNRxBeam,i

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer within the active DL BWP to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resource instances that meet the applicability conditions and fully or partially overlapped with PRS processing window are considered.

For cap 1A and cap 1B UEs that report (N2, T2) in their capability signalling, Lavailable shall be calculated based on the unmuted PRS resource instances that fully or partially overlap with the first (T2-N2) ms of the PPW.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer within active DL BWP configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS-RSRP measurement samples and

if UE supports supportedDL-PRS-ProcessingSamples and is indicated by LMF to perform measurement with reduced sample number, Nsample= 1 if the following conditions are met; Nsample= 2 otherwise.

-	PRS bandwidth is within the active BWP, and

-	Difference between the serving cell SS-RSRP and neighbor cell/TRP PRS-RSRP is within [6] dB.

if UE does not support supportedDL-PRS-ProcessingSamples, or UE is not indicated by LMF to perform measurement with reduced sample number, Nsample= 4;

Tlast = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer within the active DL BWP,

TiT	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,PPWRPi is the least common multiple between TPRS,i and PPWRPi,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer within active DL BWP,

PPWRPi is the PRS processing window repetition period in the positioning frequency layer within active DL BWP.

If positioning frequency layer within the active DL BWP has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. If bitmap b1  for higher-layer parameter DL-Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS, only the PRS resources that meet the applicability conditions and fully or partially covered by the PRS processing window are considered.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS-RSRP_wo_gap,total starts from the first PRS processing window instance aligned with DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 9.9.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 9.9.4.

The measurement requirements do not apply for a PRS resource:

if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The measurement period requirement in this section applies provided UE indicates that PRS is higher priority than other signals within PRS processing window:

- For Cap 1A UEs: measurement period requirement in this clause applies when PRS has higher priority than all other DL signals/channels (per UE) present inside the PRS processing window instances comprising the measurement period. If higher-priority DL signals/channels (per UE) are present inside any of the PRS processing windows comprising the required measurement period, the measurement period can be extended.

- For Cap 1B UEs: measurement period requirement in this clause applies when PRS in FR1 has higher priority than all other DL signals/channels (in the same band) present inside the PRS processing window instances comprising the measurement period. If higher-priority DL signals/channels (in the same band) are present inside any of the PRS processing windows comprising the required measurement period, the measurement period can be extended.

- For Cap 2 UEs: measurement period requirement in this clause applies when PRS in FR1 has higher priority than all other DL signals/channels (in the same CC) that overlap in time with PRS symbols inside the PRS processing window instances comprising the measurement period. If higher-priority signals/channels (in the same CC) overlap in time with PRS symbols inside any of the PRS processing windows comprising the required measurement period, the measurement period can be extended.

The requirements in clause 9.9.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If during the measurement period of one or more positioning frequency layers, the PPW is reconfigured or reactivated, the PRS-RSRP measurement period can be longer.

---------------END OF CHANGE 2---------------

---------------START OF CHANGE 3---------------

#### 9.9.4.6	Measurement Period Requirements without Measurement Gaps

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause x.x.x) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layer within the measurement period TUERxTx_wo_gap,Total ms.

TUERxTx_wo_gap, Total=i=1LTUERxTx_wo_gap,i+L-1*maxTeffect,i

i

TUERxTx_wo_gap,i

Teffect,i

TUERxTx_wo_gap,i=kmultiTEG,i*CSSFi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i

Where

NRxBeam is the scaling factor for Rx beam sweeping:

- NRxBeam,i = 1 if positioning frequency layer i is in FR1.

- NRxBeam,i = 8 if positioning frequency layer i is in FR2 and UE does not support [TBD, IE for support of lower Rx beam sweeping factor].

- NRxBeam,i = 8 if positioning frequency layer i is in FR2 and requested by LMF via lowerRxBeamSweepingThan8-FR2-r17 despite UE being capable of supporting [TBD, IE for support of lower Rx beam sweeping factor].

- NRxBeam,i = [TBD, IE for support of lower Rx beam sweeping factor] if positioning frequency layer i is in FR2, UE is capable of supporting [TBD, IE for support of lower Rx beam sweeping factor] and is requested by LMF via lowerRxBeamSweepingThan8-FR2-r17.

- CSSFi

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul. 	NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if the measured positioning frequency layer is in FR1 and NRxBeam,i=8 if the measured positioning frequency layer  is in FR2,

Lavailable_PRS,i is the time duration of available PRS resources in the positioning frequency layer , to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].  For calculation of Lavailable_PRS,i, only the unmuted PRS resources that meet the applicability conditions and fully or partially overlapped with PRS processing window are considered. For cap 1A and cap 1B UEs that report (N2, T2) in their capability signalling, Lavailable shall be calculated based on the unmuted PRS resource instances that fully or partially overlap with the first (T2-N2) ms of the PPW.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer within the active DL BWP configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to [durationOfPRS-ProcessingSysmbols] in TS 37.355 [34] processed every T ms corresponding to [durationOfPRS-ProcessingSymbolsInEveryTms] in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to [supportedBandwidthPRS] in clause 4.2.7.2 of TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to [maxNumOfDL-PRS-ResProcessedPerSlot] as specified in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of UE Rx-Tx time difference measurement samples.

Nsample =4 for UE not supporting reduced number of PRS samples.

Nsample =1 for UE supporting reduced number of PRS samples and the difference between the serving cell SS-RSRP and target neighbour cell PRS-RSRP is within [6]dB.

Nsample =2 for UE supporting reduced number of PRS samples and the difference between the serving cell SS-RSRP and target neighbour cell PRS-RSRP equals to or larger than [6]dB.

Tlast,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the positioning layer within active DL BWP, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is periodicity of UE Rx-Tx time difference measurement in the positioning frequency layer within active DL BWP:

[Teffect,i= TiTavailablePRS,i*TavailablePRS,i]

where

TTi	corresponds to [durationOfPRS-ProcessingSymbolsInEveryTms] in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,PPWRPi, the least common multiple between TPRS,i and PPWRPi

PPWRPi is the PRS processing window repetition periodicity in the positioning frequency layer within active DL BWP.

TPRS,i is the PRS resource periodicity in positioning frequency layer . If the positioning frequency layer has more than one DL PRS resource sets with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of TperPRS with muting among DL PRS resource sets is used to derive TPRS,i, where

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1

Note:	For the purpose of calculating TPRS, only the PRS resources that meet the applicability conditions and fully or partially covered by the PRS processing window are considered.

The time TUERxTx_wo_gap,Total starts from the first PRS processing window instance aligned with DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period, the PRS processing window is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The measurement period requirement in this section applies provided UE indicates that PRS is higher priority than other signals within PRS processing window:

- For Cap 1A UEs: measurement period requirement in this clause applies when PRS has higher priority than all other DL signals/channels (per UE) present inside the PRS processing window instances comprising the measurement period. If higher-priority DL signals/channels (per UE) are present inside any of the PRS processing windows comprising the required measurement period, the measurement period can be extended.

- For Cap 1B UEs: measurement period requirement in this clause applies when PRS in FR1 has higher priority than all other DL signals/channels (in the same band) present inside the PRS processing window instances comprising the measurement period. If higher-priority DL signals/channels (in the same band) are present inside any of the PRS processing windows comprising the required measurement period, the measurement period can be extended.

- For Cap 2 UEs: measurement period requirement in this clause applies when PRS in FR1 has higher priority than all other DL signals/channels (in the same CC) that overlap in time with PRS symbols inside the PRS processing window instances comprising the measurement period. If higher-priority signals/channels (in the same CC) overlap in time with PRS symbols inside any of the PRS processing windows comprising the required measurement period, the measurement period can be extended.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 9.9.4.6 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by [NR-DL-PRS-ResourcesCapability] in [NR-Multi-RTT-ProvideCapabilities], and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE continues the UE Rx-Tx time difference measurement, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

Editor’s note: FFS when SRS is reconfigured without cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

---------------END OF CHANGE 3---------------