3GPP TSG-RAN WG4 Meeting # 104-e												R4-2213267

Electronic Meeting, 15 – 26 August 2022

Agenda item:			9.19.2.2.3

Source:	Ericsson

Title:	On DL PRS-RSRPP accuracy requirement

Document for:	Discussion

# Introduction

RAN4#103-e agreed on the need to define PRS-RSRPP measurement accuracy requirement [1]. RAN4#103- e also agreed on the simulation assumptions for PRS-RSRPP measurement performance evaluation [2]. In our companion paper simulation results for first path PRS-RSRP error has been reported [3]. In this paper we present our view on PRS-RSRPP accuracy requirement.

# Discussion

To evaluate the baseband performance of PRS-RSRPP measurement, two-tap channel model defined in 38.101-4 was agreed. The parameters of the two-tap channel model to be used for evaluation were: a = 1, τd=0.45 µs and fD=5 Hz. Based on the definition of propagation condition, the ideal value of PRS-RSRPP for the first path can reasonably be calculated as half of the ideal PRS-RSRP value. Based on this assumption simulation results for the first path PRS-RSRPP error reported in R4-2213266 are generated.

Observation 1: Ideal value of first path PRS-RSRP is half of the ideal PRS-RSRP value.

Based on the simulation results reported in our companion paper, the following observations are made on DL PRS-RSRPP accuracy requirement:

Observation 2: In most of the cases it was observed that the PRS-RSRPP error is less than 1dB for cell 1 and cell 2.

Observation 3: It was observed that the PRS-RSRPP measurement for cell 3 is less accurate.

Observation 4: Observation 2 and observation 3 hold true for all evaluated scenarios.

Observation 5: PRS-RSRPP accuracy for cell 3 is found to be within  3dB.

# Summary

In this paper we presented our view on DL PRS-RSRPP accuracy requirement based on the simulation results reported in R4-2213266.

Observation 1: Ideal value of first path PRS-RSRP is half of the ideal PRS-RSRP value.

Observation 2: In most of the cases it was observed that the PRS-RSRPP error is less than 1dB for cell 1 and cell 2.

Observation 3: It was observed that the PRS-RSRPP measurement for cell 3 is less accurate.

Observation 4: Observation 2 and observation 3 hold true for all evaluated scenarios.

Observation 5: PRS-RSRPP accuracy for cell 3 is found to be within  3dB.

# References

[1] R4-2210600, “WF on NR Positioning Enhancements (Part 1)”, Ericsson.

[2] R4-2211061,	“Simulation assumption for PRS-RSRPP”, Huawei.

[3] R4-2213266, “Link level simulation results for PRS-RSRPP accuracy requirement”, Ericsson.