3GPP TSG-RAN WG4 Meeting # 104-e												R4-2213277

Electronic Meeting, 15 – 26 August 2022

Agenda item:			11.14.2

Source:	Ericsson

Title:	On accuracy improvement based on PRS/SRS bandwidth aggregation

Document for:	Discussion

# Introduction

RAN4#94e approved the revised SID on study on expanded and improved NR positioning with the following objectives [1]:

- Improved accuracy, integrity, and power efficiency:

  - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]:

    - Identify the error sources, [RAN1, RAN2].

    - Study methodologies, procedures, signaling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2]

    - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

  - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4].

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary.

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1]

    - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study.

    - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state.

In this contribution, we present our views on different aspects of accuracy improvement based on bandwidth aggregation for intra-band carriers.

# Discussion

To elaborate on different aspects of PRS bandwidth aggregation a single path channel model in frequency domain is considered:

clj=PTXGnlexpi∅l-2πifSCSj-flδ+Nlj,                                                                     (1)

where j=-nl2,…,nl2-1 are subcarrier indices, l=1,2,…,m are PFL indices (each with different and unknown phase offset ∅l), fSCS is the subcarrier spacing, fl is the carrier frequency of PFL l, PTX is the Tx power, G is the (unknown) path gain, δ is the (unknown) delay, and Nlj is i.i.d. complex Gaussian noise with zero mean and variance σ2. For each PFL index l PRS is measured on nl consecutive subcarriers, e.g., by combining k consecutive symbols from a staggered comb-k PRS. The bandwidth allocated to PRS in PFL l is then nlfSCS.

The Cramér-Rao bound of an unbiased delay estimator (δ) takes the form:

varδ-δ≥3σ22π2PTXGfSCS2l=1mnl2-2                                                                                        (2)

From (3), the following observations can be made:

Observation 1: Cramér-Rao lower bound on the ToA error variance scales to the inverse square of the PRS bandwidth under the fixed Tx power assumption.

Observation 2: The ToA error variance is inversely proportional to the number of consecutive PRS resources (which is seen if we interpret the PFLs l=1,2,…,m as different PRS resources with the same bandwidth nl=n).

Observation 3: If the number of PRS resources is doubled (for example, from a 1-slot staggered comb-k PRS to a 2-slot staggered comb-k PRS at the same bandwidth), the ToA error variance is halved.

Observation 4: ToA error variance decreases faster with increase in bandwidth than increasing PRS resources in the time dimension.

Observation 5: Increased PRS bandwidth by aggregating PRS resources from multiple PFLs helps improve RSTD and UE Rx-Tx measurement accuracy.

Observation 6: The total PRS bandwidth after aggregation does not enhance achievable ToA accuracy if the aggregated PRS resources in different PFL that are combined are non-coherent in phase.

Observation 7: Maintaining phase consistency between the PRS resources in different PFLs is easier when PFLs are within the same band and are contiguous.

Observation 8: Timing alignment between PRS resources transmitted in different PFLs impact achievable ToA accuracy.

To support MIMO transmissions and carrier aggregation at UE, TS 38.104 has defined the following requirements on timing alignment at BS [2].

Minimum requirement for BS type 1-C and BS type 1-H 
For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3μs.

For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3μs.

Minimum requirement for BS type 2-O
For MIMO transmission, at each carrier frequency, OTA TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 130ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 260ns.

For inter-band carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3μs.

Observation 9: For intra-band contiguous carrier aggregation, TAE shall not exceed 260ns and 130ns for BS type 1-C/BS type 1-H and BS type 2-O, respectively.

Observation 10: For intra-band non-contiguous carrier aggregation, TAE shall not exceed 3μs for base station types 1-C and 1-H and TAE shall not exceed 260ns for BS type 2-O.

Observation 11: For inter-band carrier aggregation, TAE shall not exceed 3μs for BS type 1-C and BS type 1-H and TAE shall not exceed 3μs for BS type 2-O.

The TAE is higher for intra-band non-contiguous and inter-band carrier aggregation. Higher TAE does not help achieving the intended accuracy improvement. Also based on observations 6, 7, and 8 RAN4 shall study solutions for accuracy improvement based on PRS bandwidth aggregation only for intra-band contiguous carriers.

Proposal 1: RAN4 to study solutions for accuracy improvement based on PRS bandwidth aggregation only for intra-band contiguous carriers.

TS38.101-1 has defined the following timing alignment requirement for UL intra-band contiguous CA [3].

Time alignment error for intra-band UL contiguous CA for UL MIMO

For intra-band UL contiguous CA and UE(s) with multiple transmit antenna connectors supporting UL MIMO, this requirement applies as specified in 6.4D.3: The time alignment error (TAE) is defined as the average frame timing difference between any two transmissions on different transmit antenna connectors for each CC. For UE(s) with multiple transmit antenna connectors, the Time Alignment Error (TAE) shall not exceed 130 ns.

Proposal 2: RAN4 to study solutions for accuracy improvement based on SRS bandwidth aggregation only for intra-band contiguous carriers.

# Summary

In this contribution following observations and proposals are made:

Observation 1: Cramér-Rao lower bound on the ToA error variance scales to the inverse square of the PRS bandwidth under the fixed Tx power assumption.

Observation 2: The ToA error variance is inversely proportional to the number of consecutive PRS resources (which is seen if we interpret the PFLs l=1,2,…,m as different PRS resources with the same bandwidth nl=n).

Observation 3: If the number of PRS resources is doubled (for example, from a 1-slot staggered comb-k PRS to a 2-slot staggered comb-k PRS at the same bandwidth), the ToA error variance is halved.

Observation 4: ToA error variance decreases faster with increase in bandwidth than increasing PRS resources in the time dimension.

Observation 5: Increased PRS bandwidth by aggregating PRS resources from multiple PFLs helps improve RSTD and UE Rx-Tx measurement accuracy.

Observation 6: The total PRS bandwidth after aggregation does not enhance achievable ToA accuracy if the aggregated PRS resources in different PFL that are combined are non-coherent in phase.

Observation 7: Maintaining phase consistency between the PRS resources in different PFLs is easier when PFLs are within the same band and are contiguous.

Observation 8: Timing alignment between PRS resources transmitted in different PFLs impact achievable ToA accuracy.

Observation 9: For intra-band contiguous carrier aggregation, TAE shall not exceed 260ns and 130ns for BS type 1-C/BS type 1-H and BS type 2-O, respectively.

Observation 10: For intra-band non-contiguous carrier aggregation, TAE shall not exceed 3μs for base station types 1-C and 1-H and TAE shall not exceed 260ns for BS type 2-O.

Observation 11: For inter-band carrier aggregation, TAE shall not exceed 3μs for BS type 1-C and BS type 1-H and TAE shall not exceed 3μs for BS type 2-O.

Proposal 1: RAN4 to study solutions for accuracy improvement based on PRS bandwidth aggregation only for intra-band contiguous carriers.

Proposal 2: RAN4 to study solutions for accuracy improvement based on SRS bandwidth aggregation only for intra-band contiguous carriers.

# References

[1] RP-213588, “Revised SID on Study on expanded and improved NR positioning”, Intel.

[2] TS 8.104: “NR; Base Station (BS) radio transmission and reception”.

[3] TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.