3GPP TSG-RAN WG4 Meeting # 104-e												R4-2213278

Electronic Meeting, 15 – 26 August 2022

Agenda item:			11.14.3

Source:	Ericsson

Title:	On accuracy improvement based on NR carrier phase measurements

Document for:	Discussion

# Introduction

RAN4#94e approved the revised SID on study on expanded and improved NR positioning with the following objectives [1]:

- Improved accuracy, integrity, and power efficiency:

  - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]:

    - Identify the error sources, [RAN1, RAN2].

    - Study methodologies, procedures, signaling, etc. for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2]

    - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

  - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4].

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary.

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1]

    - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study.

    - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state.

In this contribution, we present our views on different aspects of accuracy improvement based on NR carrier phase measurements.

# Discussion

Carrier phase-based measurement allows GNSS based positioning solutions to offer centimeter level accuracy in an outdoor environment. Inspired from GNSS success, Rel. 18 SID intends to study NR carrier phase-based positioning solutions to meet requirements of use cases that demand stringent positioning accuracy. RAN1#109e discussed the different aspects of NR carrier phase-based positioning and agreed to [2]:

- Study NR carrier phase positioning with the carrier phase measurements of one carrier frequency or multiple frequencies.

- Study combination of NR carrier phase positioning with another standardized Rel. 17 positioning method, e.g., DL-TDOA, UL-TDOA, Multi-RTT, etc.

- Study impact of multipath on carrier phase measurement.

- Study impact of error sources such as:

  - Phase noise (FR2)

  - CFO/Doppler

  - Oscillator-drift

  - Transmitter/receiver antenna reference point location errors

  - Transmitter/receiver initial phase error

  - Phase centre offset

For the evaluation purposes RAN1 agreed to:

- Reuse simulation assumptions of NR Rel-16/17.

- Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary.

- InF-SH and InF-DH as baseline scenarios.

- FR1 and FR2 in terms of frequency ranges.

The agreements made in RAN1#109e shows that the RAN1 work on carrier phase-based positioning is at its early phase. For RAN4 to understand the impact of carrier phase-based positioning on RAN4 specifications, more concrete progress in RAN1 shall be made. In this sense, RAN4 shall wait for further progress in RAN1 to evaluate and assess the scope of solutions based on NR carrier phase measurements to be studied by RAN4.

Proposal 1: RAN4 to wait for further progress in RAN1 to evaluate and assess the scope of solutions based on NR carrier phase measurements to be studied by RAN4.

# Summary

In this contribution following is proposed:

Proposal 1: RAN4 to wait for further progress in RAN1 to evaluate and assess the scope of solutions based on NR carrier phase measurements to be studied by RAN4.

# References

[1] RP-213588, “Revised SID on Study on expanded and improved NR positioning”, Intel.

[2] R1-2205165, “FL Summary #2 for improved accuracy based on NR carrier phase measurement, CATT.