3GPP TSG-RAN WG4 Meeting #104-e 	       R4-2213336

# Electronic Meeting, 15 - 26 May 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results on PDSCH demodulation requirements for NTN

Agenda item: 	9.11.7.3.1

Document for:	Discussion

# Introduction

The NTN WI is presented in [1], where the following RAN4 objectives are defined:

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite Note 2: The spectrum usage on the service link for HAPS might be a different spectrum allocation than for Satellite.

In this contribution we present preliminary simulation results for PDSCH demodulation requirements.

# Discussion

We discuss open issues related to the PDSCH demodulation requirements in our accompanying paper [2].

## Simulation parameters

Proposed simulation parameters for demodulation requirements are shown in Table 1. These parameters are for HARQ disabled case. The parameters are assuming fixed SNR, NTN TDL-A, NTN TDL-C channel profiles and 100ns delay spread according to agreement in [3].

K-factor for line-of-sight channel has been discussed in previous meetings. In our previous RAN4 contribution [7] we had chosen k-factor corresponding to suburban scenario with 50 degrees elevation, which leads to 21.6dB. The worst-case elevation angle considered in RAN1 discussion and TR38.821 [4] Table 6.1.3.2-1 is 30 degrees for LEO, and thus k-factor value could be set to the mean k-factor value according to a specific scenario at this elevation from Table 6.7.2-x in TR38.811 [5]. For example in rural case, which is the most common scenario, K-factor is 8.05 dB at worst case elevation angle for LEO. In this contribution we have provided results with both for LOS channel.

In these simulations we have assumed UE velocity to be 3km/h as it is also in TR 38.821 [4] link simulation assumptions for S-band.

We have chosen to use two existing RMC:s with lowest SNR requirements for the simulations i.e. R.PDSCH.1-1.1 FDD (QPSK) and R.PDSCH.1-2.1 FDD (16QAM). We propose these to be used for demodulation requirements.

Proposal 1:  Use existing RMC:s R.PDSCH.1-1.1 FDD (QPSK) and R.PDSCH.1-2.1 FDD (16QAM) for NTN PDSCH demodulation requirements.

We propose the parameters provided to be taken into account when parameters for NTN PDSCH demodulation requirements are decided.

Proposal 2:  Use parameters in Table 1 for NTN PDSCH demodulation requirements.

We have discussed Doppler shift in our accompanying contribution [5].

Table 1 Proposed simulation parameters for PDSCH demodulation requirements

| Parameter |  | Value |
| --- | --- | --- |
| Number of SAN antennas/UE antennas |  | 1x2 |
| Carrier frequency |  | 2 GHz |
| Residual frequency offset |  | 0.1ppm (200Hz) |
| Channel model |  | NTN-TDLA 100 ns (NLOS) NTN-TDLC 100ns (LOS) |
| Channel model k-factor |  | 21.6 (corresponds to suburban scenario with 50 degrees elevation) 8.05 (corresponds to rural scenario with 30 degrees elevation) |
| UE velocity |  | 3 km/h |
| MCS |  | 4 (QPSK, 64QAM MCS table) 13 (16QAM, 64QAM MCS table) |
| Reference measurement channel |  | R.PDSCH.1-1.1 FDD (QPSK) R.PDSCH.1-2.1 FDD (16QAM) |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | N/A |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Depends on RMC |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | Ratio of PDSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=49, nSCID =0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 2 |
|  | Allocation length | 12 |
| Frequency domain resource assignment | RB assignment | Depends on RMC |

## Simulation results

Preliminary simulation results for QPSK/16QAM and NTN TDL-A/C are shown in Figures 1-4. Both BLER and normalized throughput plots are provided.

We are discussing principle of choosing K-factors for the simulations in the previous section. In this contribution we have provided results with two approaches for LOS channel. These are suburban scenario with 50 degrees elevation (21.6dB) and rural scenario with 30 degrees elevation (8.05dB). The performance difference between the two seems to be rather small.

Observation 1: Performance difference between the two K-factor approaches (21.6 vs. 8.05dB) is rather small.

As can be seen 10% BLER and 70% of the peak throughput can be achieved with reasonable SNR.

Figure 1 BLER and normalized throughput for QPSK and NTN-TDL-A 100ns

Figure 2 BLER and normalized throughput for QPSK and NTN-TDL-C 100ns

Figure 3 BLER and normalized throughput for 16QAM and NTN-TDL-A 100ns

Figure 4 BLER and normalized throughput for 16QAM and NTN-TDL-C 100ns

# Conclusion

This contribution discusses aspects related to NTN PDSCH demodulation requirements for NTN and has the following proposal and observations:

Proposal 1:  Use existing RMC:s R.PDSCH.1-1.1 FDD (QPSK) and R.PDSCH.1-2.1 FDD (16QAM) for NTN PDSCH demodulation requirements.

Proposal 2:  Use parameters in Table 1 for NTN PDSCH demodulation requirements.

Observation 1: Performance difference between the two K-factor approaches (21.6 vs. 8.05dB) is rather small.

# References

- RP-220208, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2213280, Discussion on PDSCH demodulation requirements for NTN, Nokia, Nokia Shanghai Bell

- R4-2210661, WF on NTN demodulation requirements - general and PDSCH, Qualcomm

- TR 38.821 v16.1.0, Solutions for NR to support non-terrestrial networks (NTN), 3GPP

- TR 38.811 v15.4.0, Study on New Radio (NR) to support non-terrestrial networks, 3GPP

- R4-2213279, Discussion on general issues for NTN demodulation requirements, Nokia, Nokia Shanghai Bell

- R4-2208880, Discussion on PDSCH demodulation requirements for NTN, Nokia, Nokia Shanghai Bell