3GPP TSG-RAN WG4 #104-e	R4-2213343

Electronic meeting, August 15 – August 26, 2022

Source: 	Ericsson

Title:  	Test case for L1-RSRP measurement requirement for HST FR2

Agenda Item:	9.7.3.5

Document for:	Discussion

# Introduction

Work on RRM requirements for train-mounted UE in high speed train scenario in FR2 [1] continued during the RAN4#103-e meeting, with outcome in terms of agreements and open issues on performance requirements captured in WFs [2].

In this contribution, we present our viewpoints and proposals on remaining issues in WFs [2].

# Disucssion

## Sub-Topic 1: Scope of RRM Performance Requirements

Issue 1-7: Necessity of Test Cases for L1-RSRP/SINR Measurement Requirement

Way Forward: - TC for L1 Measurement Requirement: - New test case for A.7.6.3.2, SSB based L1-RSRP measurement for PC6 UE. - FFS DRX cycle = 40 ms or Non-DRX configured, - SMTC period = 20 ms, - Propagation condition: AWGN with 19444 Hz frequency offset - FFS new test case for A.7.6.6.2, SSB based L1-SINR measurement for PC6 UE. - FFS DRX cycle = 40 ms or Non-DRX configured, - SMTC period = 20 ms, - Propagation condition: AWGN with 19444 Hz frequency offset - FFS the necessity of introducing TC for SSB based L1-SINR measurement: - To be decided in next RAN4 meeting.

Proposal 1: Tend to support DRX cycle = 40 ms.

Proposal 2: Don’t introduce L1-SINR test case.

# Conclustion

Proposal 1: Tend to support DRX cycle = 40 ms.

Proposal 2: Don’t introduce L1-SINR test case.

# References

- RP-202118 “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia

- R4-2210609 “WF on RRM performance requirement for FR2 HST”, Samsung