3GPP TSG-RAN WG4 #104-e	R4-2213345

Electronic meeting, August 15 – August 26, 2022

Source: 	Ericsson

Title:  	Test case for cell re-selection requirement for HST FR2

Agenda Item:	9.7.3.2

Document for:	Discussion

# Introduction

Work on RRM requirements for train-mounted UE in high speed train scenario in FR2 [1] continued during the RAN4#103-e meeting, with outcome in terms of agreements and open issues on performance requirements captured in WFs [2].

In this contribution, we present our viewpoints and proposals on cell re-selection issues in WFs [2].

# Disucssion

## Sub-Topic 1: Scope of RRM Performance Requirements

Issue 1-1: Necessity of Test Cases for Cell Re-selection Requirement

Way Forward: - TC for Cell Re-selection Requirement: - New test case for A.7.1.1.X, Cell reselection to FR2 intra-frequency NR case for PC6 UE configured with highSpeedMeasFlagFR2-r17, DRX cycle=0.32s: - Test case definition and applicability: - Option-1: 1 test case defined from either unidirectional or bi-directional: - FFS unidirectional or bi-directional: - Consider bi-directional as starting point - Option-2: 2 test cases defined for unidirectional and bi-directional respectively: - Applicability rule is introduced: UE needs to be tested with one of the tests in either unidirectional or bidirectional. - SMTC period = 20 ms, - Propagation condition: AWGN (serving cell) and AWGN with [9722] Hz frequency offset (neighbour cell) for unidirectional or [19444] Hz frequency offset (neighbour cell) for bi-directional case.

Proposal 1: Bi-directional test case shall be defined for cell re-selection requirement just since the coverage is greater than Un-directional, specially for cell mobility.

Issue 1-2: Necessity of Test Cases for Re-establishment Delay Requirement

Way Forward: - TC for Re-establishment Delay Requirement: - FFS new test case for A.7.3.2.1.X, Intra-frequency RRC Re-establishment in FR2 for PC6 UE configured with highSpeedMeasFlagFR2-r17. - Test case definition and applicability: - Option-1: 1 test case defined from either unidirectional or bi-directional: - FFS unidirectional or bi-directional: - Consider bi-directional as starting point - Option-2: 2 test cases defined for unidirectional and bi-directional respectively: - Applicability rule is introduced: UE needs to be tested with one of the tests in either unidirectional or bidirectional. - SMTC period = 20 ms, - Propagation condition: AWGN (serving cell) and AWGN with [9722] Hz frequency offset (neighbour cell) for unidirectional or [19444] Hz frequency offset (neighbour cell) for bi-directional case. - FFS the necessity of introducing TC for re-establishment delay requirement: - To be decided in next RAN4 meeting.

Proposal 2: Bi-directional test case shall be defined for cell re-selection requirement just since the coverage is greater than Un-directional, specially for cell mobility.

# Conclustion

Proposal 1: Bi-directional test case shall be defined for cell re-selection requirement just since the coverage is greater than Un-directional, specially for cell mobility.

Proposal 2: Bi-directional test case shall be defined for cell re-selection requirement just since the coverage is greater than Un-directional, specially for cell mobility.

# References

- RP-202118 “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia

- R4-2210609 “WF on RRM performance requirement for FR2 HST”, Samsung