3GPP TSG-RAN WG4 Meeting #104-e	R4-2213356

Electronic Meeting, August 15 – August 26, 2022

Source: 	Ericsson

Title:  	Test case for cell reselection for NTN

Agenda Item:	9.11.6.2

Document for:	Discussion

# Introduction

In RAN4# 103-e a WF on NTN RRM performance requirements was approved [1]. In this contribution we develop topics on performance requirements which were agreed to continue study in cell reselection part in the WF.

# Discussion

## Issue 3-2-2: NTN RRM requirements test

The agreements as follow list all cases on NTN requirements:

- Define the following TCs corresponding to existing TCs in clause A.6 for NTN RRM requirements - Cell reselection to intra- and inter-frequency neighbor cell - Including reselection measurement based on time and location trigger - Intra- and inter-frequency HO with known cell - Intra- and inter-frequency CHO - Including CHO with time or location condition - UE transmit timing - RLM and BFR - Intra-frequency measurement delay - Including 2 SMTCs per MO with and without collision - Including 2 satellites per SMTC - Inter-frequency measurement delay - Including 2 MGs - L1-RSRP measurement delay - RRM measurement accuracy

Tserivce and DdistanceThresh triggering cell reselection shall keep compliance with legacy test requirements as much as possible. Quote A.6.1.1.1.3 in TS38.133 in below:

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1

One possible time instant fulfilling Tserivce is after T3 and the time period between T3 and Tserivce shall be equal to or longer than measurement delay (Tdetect, NR_Intra  or Tevaluate, NR_ intra) for reselection. The setup aims to examining whether UE can start measurements before Tserivce, but not to examine the earliest time to start measurements.

One possible time instant fulfilling DdistanceThresh is at T3. The setup aims to examining whether UE can start measurements after DdistanceThresh, but not to examine the earliest time to start measurements.

Proposal 1: Regarding test cases for Cell reselection to intra- and inter-frequency neighbor cell without time or location trigger, test cases can refer to legacy TN test cases.

Proposal 2: Regarding test cases for reselection measurement based on time trigger, the time instant fulling Tserivce is after T3, time period between T3 (refer to A.6.1.1.1.3) and Tserivce shall be equal to or longer than measurement delay (Tdetect, NR_Intra or Tevaluate, NR_ intra  defined in A.6.1.1.1.3) for reselection.

Proposal 3: Regarding test cases for reselection measurement based on location trigger, the time instant fulling DdistanceThresh is at T3 (defined in A.6.1.1.1.3) to examine whether UE can start measurements after DdistanceThresh.

Proposal 4: Define test cases for relaxed measurement requirements on reselection in GEO scenario.

# Summary

Proposal 1: Regarding test cases for Cell reselection to intra- and inter-frequency neighbor cell without time or location trigger, test cases can refer to legacy TN test cases.

Proposal 2: Regarding test cases for reselection measurement based on time trigger, the time instant fulling Tserivce is after T3, time period between T3 (refer to A.6.1.1.1.3) and Tserivce shall be equal to or longer than measurement delay (Tdetect, NR_Intra or Tevaluate, NR_ intra  defined in A.6.1.1.1.3) for reselection.

Proposal 3: Regarding test cases for reselection measurement based on location trigger, the time instant fulling DdistanceThresh is at T3 (defined in A.6.1.1.1.3) to examine whether UE can start measurements after DdistanceThresh.

Proposal 4: Define test cases for relaxed measurement requirements on reselection in GEO scenario.

# References

- R4-2210610	WF on timing requirements and performance requirements for NR NTN	Qualcomm