3GPP TSG-RAN WG4 Meeting #104-e 	       R4-2213360

# Electronic Meeting, August 15 – August 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for NTN SAN demodulation

Agenda item:	9.11.7.2.1

Document for:	Discussion

# Introduction

The NTN WI is presented in [1], where the following RAN4 objectives are defined:

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite Note 2: The spectrum usage on the service link for HAPS might be a different spectrum allocation than for Satellite.

In this contribution we present our proposal for simulation parameters of PUSCH demodulation requirements for NTN, and some preliminary results.

# Discussion

We discuss open issues related to the SAN demodulation requirements in our accompanying paper [2].

# Simulation parameters

Proposed simulation parameters for demodulation requirements are shown in Table 1. The parameters are assuming NTN TDL-A, NTN TDL-C channel profiles, MCS4, and 0.1 ppm doppler error in service link according to agreement in [3,4]. Delay spread is 100ns according to agreement made in RAN4 #102-e.

We selected the k-factor line-of-sight TDL-C channel according to the worst case elevation angle and specific scenario. The worst case elevation angle for LEO is defined as 30 degrees in TR 38.811 [5] Table 6.1.3.2-1 and RAN1 discussions, and thus k-factor value could be set to the mean k-factor value according to a specific scenario at this elevation from Table 6.7.2-x in TR38.811 [5]. For example in rural case, which is the most common scenario, K-factor is 8.05 dB at worst case elevation angle for LEO.

In these simulations we have assumed UE velocity to be 3km/h as it is also in TR 38.821 [6] link simulation assumptions for S-band.

We propose the parameters provided to be taken into account when parameters for NTN PUSCH demodulation requirements are decided.

Proposal 1:  Use parameters in Table 1 for NTN PUSCH demodulation requirements.

Table 1 Proposed simulation parameters for PUSCH demodulation requirements

| Parameter |  | Value |
| --- | --- | --- |
| Number of UE antennas/SAN antennas |  | 1x1 1x2 |
| Channel model |  | NTN-TDL- A 100 ns (NLOS) NTN-TDL-C 100ns (LOS) |
| Channel model k-factor for LOS |  | 8.05 dB (corresponds to rural scenario with 30 degrees worst elevation angle for LEO) |
| UE speed |  | 3km/h |
| SCS/CBW |  | 15kHz SCS: largest supported CBW 20MHz, 30kHz SCS: largest supported CBW 20MHz |
| Fc |  | 2GHz |
| Residual frequency error |  | 0.1ppm (200Hz) |
| MCS |  | MCS4 |
| Transform precoding |  | Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A and B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 4PRBs centered |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

# Simulation results

Preliminary simulation results for PUSCH demodulation requirements are shown in the following Figures 1-4. The BLER and throughput percentage is provided for all considered configurations shown in Table 1. The results consider 0.1ppm (200Hz) residual frequency error, as well as both proposed SAN configuration (1 Rx and 2 Rx) and PUSCH mapping types in NTN-TDL-A and C.

Figure 1 BLER and normalized throughput for 15KHz SCS and NTN-TDL-A with Table 1 configuration.

Figure 2 BLER and normalized throughput for 30KHz SCS and NTN-TDL-A with Table 1 configuration.

Figure 3 BLER and normalized throughput for 15KHz SCS and NTN-TDL-C with Table 1 configuration.

Figure 4 BLER and normalized throughput for 30KHz SCS and NTN-TDL-C with Table 1 configuration.

Observation 1: Both PUSCH mapping types A and B have almost same performance.

Observation 2: 10% BLER and 70% of the peak throughput can be achieved with reasonable SNR especially with 2 SAN antennas.

# Conclusion

This contribution discusses aspects related to NTN SAN demodulation requirements and has the following proposal and observations:

Proposal 1:  Use parameters in Table 1 for NTN PUSCH demodulation requirements.

Observation 1: Both PUSCH mapping types A and B have almost same performance.

Observation 2: 10% BLER and 70% of the peak throughput can be achieved with reasonable SNR especially with 2 SAN antennas.

# References

- RP-220208, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2213337, Discussion on NTN SAN demodulation requirements, Nokia, Nokia Shanghai Bell

- R4-2210661, WF on NTN demodulation requirements - general and PDSCH, Qualcomm

- R4-2210662 WF on NTN SAN demodulation requirements

- TR 38.811 v15.4.0, Study on New Radio (NR) to support non-terrestrial networks, 3GPP

- TR 38.821 v16.1.0, Solutions for NR to support non-terrestrial networks (NTN), 3GPP