3GPP TSG-RAN WG4 Meeting #104-e                                         R4-2213370

Electronic Meeting, Aug 15-26, 2022

Title:	Reply LS on the minimum guard period between two SRS resources for antenna switching

Response to:	R1-2200796

Release:	Rel-17

Work Item:	NR_ext_to_71GHz

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact Person:

Name: 	Liu Ye

E-mail Address: 	leo.liuye@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

R4-1710048, LS Reply to LS related to SRS hopping

1. Overall Description:

RAN4 thanks RAN1 LS on the minimum guard period between two SRS resources for antenna switching. RAN4 has discussed it and would like to give feedback on the RAN4’s understanding.

In Rel-15 RAN4 had discussed on the antenna switching time required for sounding different antenna ports, and agreed on the value of 15us regardless of SCS type. For 480 and 960 kHz SCS in question, the same 15us antenna switching time applies, therefore the answer to question from RAN1 to RAN4 would be as below.

Question to RAN4: How many symbol(s) is/are needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz respectively, in FR2-2?

Answer to RAN1: The SRS antenna switching time is 15us for both 480 and 960 kHz in FR2-2. The minimum guard time needed to accommodate the required antenna switching time could be calculated as 7.2 symbols for 480kHz SCS and 14.4 symbols for 960kHz SCS, which are rounded up in below table. RAN1 could further discuss and determine on the final values.

Table 6.2.1.2-1: The minimum guard period between two SRS resources of an SRS resource set for antenna switching

|  |  | Y [symbol] |
| --- | --- | --- |
| 0 | 15 | 1 |
| 1 | 30 | 1 |
| 2 | 60 | 1 |
| 3 | 120 | 2 |
| 5 | 480 | 8 |
| 6 | 960 | 15 |

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take above consideration into account for the future work.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#104-bis-e		Oct 2021								Electronic Meeting

TSG-RAN4 Meeting#105		Nov 2021								Canada