3GPP TSG-RAN WG4 Meeting # 104-e												R4-2213371

Electronic Meeting, Aug 15-26 2022

Agenda item:	7.1

Source: 	Huawei

Title: 	On UE power class per band per band combination

Document for:	Approval

- Introduction

In RAN4 #103-e meeting, the UE feature of Power class per band per band combination was discussed and agreed by RAN4 as below. There is still a note within brackets. Current document shares our view on how to handle this note.

| Features | Index | exhaustive Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16. NR_RF_FR1_enh | 16-8 | UE power class per band per band combination | Per band per band combination power class |  | Yes | N/A | Per band power class inconsistent | Per band per BC | No | FR1 only | N/A | [It is not applicable to the case when UL-MIMO and intra-band UL CA are in operation at the same time.] | Optional with capability signalling |

- Discussion

The UE feature of Power class per band per band combination was introduced to solve the identified ambiguity of UE power class when UE works in inter-band CA mode [2, 3]. Furthermore this UE feature would reinforce the newly specified higher power limit feature.

The note under discussion is aiming to exclude the scenario of UL-MIMO and intra-band CA operation at the same time, however it’s not needed as pointed out by several companies [4]. The discussion of this UE feature in Rel-17 was focused on inter-band CA configuration all along. Based on the common understanding, the agreed CR [5] implemented the UE capability only in subclause for inter-band CA, thus already ruled out applying this UE capability to intra-band CA case. The note under discussion is redundant and could be removed.

On the other hand, with the note being left in specification, extra ambiguity might be introduced. Current note only excludes the case of UL-MIMO+intra-band CA, it seems implying that intra-band CA without UL-MIMO is applicable, which is not true at the moment. Similarly, if only UL-MIMO + intra-band CA is excluded, the status of TxD+intra-band CA is not fully clear. There might still be more examples. In order to avoid introducing extra ambiguity, either we try to exhaustively list all the inapplicable cases, or we could simply remove this statement. Considering the applicable scenario is already clearly limited to inter-band CA in 38.101-1, removing the statement would be an easier approach and won’t do any harm to the accuracy of the specification.

Based on above consideration, we propose to remove the note from the feature list.

Proposal 1: Removing the note from description of UE feature of Power class per band per band combination.

- References

[1] R4-2211189, Rel-17 RAN4 UE feature list for NR

[2] R4-2204082, On Power Class Ambiguity and New Power Limit for NR inter-band CA, Huawei, HiSilicon, RAN4#102-e

[3] R4-2209375, R17 Per band Per BC power class capability, OPPO, RAN4#103-e

[4] R4-2210468, Email discussion summary for [103-e][136] R17_feature_list, Moderator (CMCC), RAN4#103-e

[5] R4-2210767, Increasing the maximum power limit for inter-band UL CA, Qualcomm Incorporated, Verizon, Vodafone, Deutsche Telekom, US Cellular, T-Mobile USA, AT&T, China Unicom, NTT DOCOMO, INC., China Telecom, Nokia, Nokia Shanghai Bell, CableLabs, Charter Communications, Inc., Dish Network, Skyworks Solutions, Inc., ZTE, Huawei, HiSilicon, RAN4#103-e