3GPP TSG-RAN WG4 Meeting # 104-e		R4-2213379

# Electronic Meeting, August 15 – August 26, 2022

Source:	ZTE Corporation

Title:	SIB1 channel bandwidth configuration for utilizing irregular bandwidth

Agenda Item:	11.1.2

Document for:	Discussion

# Introduction

The SID on efficient utilization of irregular bandwidths was extended from Rel-17 to Rel-18 due to unexpected two issues[1], and RAN4 Chair suggested to focus on the first issue which relates to the understanding on SIB1 signaling behavior [2].

#### 2.4.2 Remaining Open issues - Evaluation on each method upon SI objectives of all irregular bandwidth approaches is still required. Consensus on SIB1 signaling behavior and for signaling the UE specific CBW is needed prior to method evaluations can be concluded. - Below is the chair’s decision from GTW relating to the SIB1 signaling aspects in relation to WF on the SI Chair =>: Based on the discussion, the issues listed in the draft of way forward R4-2210579 seem beyond the scope of this SI and realted to the exsiting specification and companies had different understanding. But they are thought of fundamental to address the other issues for this SI. To move forward, Chair suggested to put SI on hold and focus on those issues first. Whether to put SI on hold and how to organize the discussion for those fundamental issues need be discussed in the upcoming RAN plenary. In this RAN4 meeting, the TPs related to those issues can be noted, while the other TPs not related to the issues can be further discussed in the second round and approved if agreeable.

It is essential for the group to have an aligned understanding on the SIB1 signaling behavior, in particular, the aligned understanding on SIB1 broadcast channel bandwidth. In this contribution, we focus on the discussion on the downlink direction, but the similar discussion may be applicable to the UL direction as well.

# Discussion

First of all, at the downlink direction, SIB1 signals a carrier with a bandwidth which accommodates all possible DL BWPs allocated to different UEs in the cell during the initial access stage. More specifically, the SIB1 broadcast carrier bandwidth provides a reference channel bandwidth for a UE for the initial access as shown below [3]:

The UE has a freedom to choose a predefined channel bandwidth to access the cell, which is no larger than the SIB1 bandwidth but no less than the initial DL BWP bandwidth. And the predefined channel bandwidth should be specified in RAN4 specs [4][5] as shown in the following tables.

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A | N/A | N/A | N/A |
| 120 | 32 | 66 | 132 | 264 | N/A | N/A | N/A |
| 4801 | N/A | N/A | N/A | 66 | 124 | 248 | N/A |
| 9601 | N/A | N/A | N/A | 33 | 62 | 124 | 148 |
| Note 1: This SCS is optional in this release of the specification. |  |  |  |  |  |  |  |

Observation 1: A UE has to use a UE specific channel bandwidth to have an initial access a cell which is specified in RAN4 specs.

Observation 2: SIB1 broadcast bandwidth provides an upper limit on the UE specific channel bandwidth which a UE chooses for the initial access to the cell.

As also specified in RAN4 specs, a UE specific channel bandwidth shall fall within the BS channel bandwidth, and the UE does not need to know the placement of other UEs’ channel bandwidth. Since SIB1 bandwidth is broadcast toward all UEs in the cell, it may be inferred that the SIB1 bandwidth does not need to be the BS channel bandwidth [5], and it just indicates potential range of DL BWPs placement during the initial access procedure. After the initial access, the UE may be assigned with other DL BWPs outside the SIB1 bandwidth within the BS channel bandwidth.

Observation 3: SIB1 broadcast bandwidth does not need to be the BS channel bandwidth, and just indicates potential range of DL BWPs placement during the initial access procedure.

Observation 4: After the initial access, the UE may be assigned with other DL BWPs outside the SIB1 bandwidth within the BS channel bandwidth.

With the above observations, the number of PRBs of the SIB1 carrier bandwidth can be any value no more than the maximum value.

Proposal 1: RAN4 aligns the common understanding on the SIB1 bandwidth shown in the above 4 observations.

Proposal 2: SIB1 bandwidth can be configured with any value no more than the maximum value.

# Conclusion

In this paper, we have the following observations and proposals for understanding on the SIB1 signaling behavior:

Observation 1: A UE has to use a UE specific channel bandwidth to have an initial access a cell which is specified in RAN4 specs.

Observation 2: SIB1 broadcast bandwidth provides an upper limit on the UE specific channel bandwidth which a UE chooses for the initial access to the cell.

Observation 3: SIB1 broadcast bandwidth does not need to be the BS channel bandwidth, and just indicates potential range of DL BWPs placement during the initial access procedure.

Observation 4: After the initial access, the UE may be assigned with other DL BWPs outside the SIB1 bandwidth within the BS channel bandwidth.

Proposal 1: RAN4 aligns the common understanding on the SIB1 bandwidth shown in the above 4 observations.

Proposal 2: SIB1 bandwidth can be configured with any value no more than the maximum value.

# Reference

R4-2210579 WF on carrierBandwidth in SIB1, Ericsson

RP-221330 SR on Study on efficient utilization of lic. spectrum that is not aligned with existing NR channel BWs, RAN4

TS 38.331 v17.7.0

TS 38.104 v17.6.0

TS 38.101-1 v17.6.0