3GPP TSG-RAN WG4 Meeting # 104-e		R4-2213380

# Electronic Meeting, August 15 – August 26, 2022

Source:	ZTE Corporation

Title:	TP for the signaling and configuration aspects on the next larger channel bandwidth approach

Agenda Item:	11.1.1

Document for:	Approval

# Discussion

First of all, at the downlink direction, SIB1 signals a carrier with a bandwidth which accommodates all possible DL BWPs allocated to different UEs in the cell during the initial access stage. More specifically, the SIB1 broadcast carrier bandwidth provides a reference channel bandwidth for a UE for the initial access as shown below:

The UE has a freedom to choose a predefined standard channel bandwidth to access the cell, which is no larger than the SIB1 bandwidth but no less than the initial DL BWP bandwidth. If an initial DL BWP bandwidth is selected wrongly, it may happen that a UE cannot find a standard channel bandwidth for the initial access procedure thus the cell is not accessable.

However, the signalling and configuration aspects on this approach does not explicitly describe the restriction, though the example on 7MHz does reflect it:

### 6.1.2 Signalling and configuration aspects In this section we provide further signaling details on how to support irregular channels given the 7MHz allocation as an example. Two signaling methods are described. In 6.1.2.1 the next larger CBW used during initial access. In 6.1.2.2, the next smaller CBW is used during initial access, and then once RRC connection is established, the next larger CBW is used. #### 6.1.2.1 Method using Next larger channel bandwidth is broadcast in SIB1 The gNB broadcasts the DL carrier bandwidth and the bandwidth of the initial BWP (BWP#0) in SIB1. For the 7MHz allocation, SIB1 can indicate DL next larger standard channel bandwidth, i.e. 10 MHz, and that the initial DL BWP can be set to 5 MHz: - SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> frequencyInfoDL-> scs-SpecificCarrierList-> carrierBandwidth = 52 PRBs / subcarrierSpacing = 15 kHz - SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> initialDownlinkBWP-> genericParameters-> locationAndBandwidth = 25 PRBs Once the UE established the RRC connection, the gNB can account for the UE capabilities and re-configure the UE accordingly. At this point the gNB may override the carrier bandwidth value that the UE obtained from SIB1 and configure a dedicated BWP with a bandwidth that differs from the bandwidth of BWP#0. gNB may configure a larger bandwidth part that will cover the whole 7MHz allocation. - ServingCellConfig-> downlinkChannelBW-PerSCS-List-> carrierBandwidth = 52 PRBs, subcarrierSpacing = 15 kHz - ServingCellConfig-> downlinkBWP-ToAddModList-> bwp-Common-> genericParameters-> locationAndBandwidth = TBD PRBs Another example of interest to some operators is 5 MHz at the bottom of n12 being shared with 6 MHz at the bottom of n85. The details can be found in Annex A. NOTE: For uplinkChannelBW-PerSCS-List and scs-SpecificCarrierList symmetric operation bands with fixed duplex distance and asymmetric UL/DL channel bandwidth is band specific aspect which is defined in TS 38.101-1 and TS 38.101-2. Bands which utilize the wider channel bandwidth approach for the irregular bandwidths may need to be adopted for this aspect.

Since the method involves the selection of both SIB1 channel bandwidth and the initial BWP bandwidth for the next larger channel bandwidth approach, the restriction on the initial BWP bandwidth selection should be explicitly described. One example is shown in Annex.

Proposal 1: Update the signalling and configuration aspects on the next larger channel bandwidth approach as Annex.

# Annex

### 6.1.2 Signalling and configuration aspects In this section we provide further signaling details on how to support irregular channels given the 7MHz allocation as an example. Two signaling methods are described. In 6.1.2.1 the next larger CBW used during initial access. In 6.1.2.2, the next smaller CBW is used during initial access, and then once RRC connection is established, the next larger CBW is used. #### 6.1.2.1 Method using Next larger channel bandwidth is broadcast in SIB1 The gNB broadcasts the DL carrier bandwidth and the bandwidth of the initial BWP (BWP#0) in SIB1. The SIB1 DL channel bandwidth is set to the next larger channel bandwidth, and the selection of the bandwidth of the initial BWP should guarantee that there is at least one standard channel bandwidth which is no wider than the next larger channel bandwidth, and no less than the bandwidth of the initial BWP. For the 7MHz allocation, SIB1 can indicate DL next larger standard channel bandwidth, i.e. 10 MHz, and that the initial DL BWP can be set to 5 MHz: - SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> frequencyInfoDL-> scs-SpecificCarrierList-> carrierBandwidth = 52 PRBs / subcarrierSpacing = 15 kHz - SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> initialDownlinkBWP-> genericParameters-> locationAndBandwidth = 25 PRBs Once the UE established the RRC connection, the gNB can account for the UE capabilities and re-configure the UE accordingly. At this point the gNB may override the carrier bandwidth value that the UE obtained from SIB1 and configure a dedicated BWP with a bandwidth that differs from the bandwidth of BWP#0. gNB may configure a larger bandwidth part that will cover the whole 7MHz allocation. - ServingCellConfig-> downlinkChannelBW-PerSCS-List-> carrierBandwidth = 52 PRBs, subcarrierSpacing = 15 kHz - ServingCellConfig-> downlinkBWP-ToAddModList-> bwp-Common-> genericParameters-> locationAndBandwidth = TBD PRBs Another example of interest to some operators is 5 MHz at the bottom of n12 being shared with 6 MHz at the bottom of n85. The details can be found in Annex A. NOTE: For uplinkChannelBW-PerSCS-List and scs-SpecificCarrierList symmetric operation bands with fixed duplex distance and asymmetric UL/DL channel bandwidth is band specific aspect which is defined in TS 38.101-1 and TS 38.101-2. Bands which utilize the wider channel bandwidth approach for the irregular bandwidths may need to be adopted for this aspect.