3GPP TSG RAN WG4 Meeting #104-e	   R4-2213402

Electronic Meeting, 15th – 26th August 2022

Agenda Item:	9.12.1

Source: 	Ericsson

Title:	Discussions on UE power saving requirements

Document for:	Discussion

# Introduction

Good progress was reached on the remaining issues of UE power saving WI at last meeting. However, there are still some open issues that were captured in the WF in [1]. In this contribution we discuss and provide our view on those.

# Discussion

Following open issue remains [1]:

#### Issue 1-8: Clarifications for Low mobility criteria evaluation For NR-DC, the following proposals can be discussed in R4#104-e: - Proposal 1-1: For NR-DC, UE may also evaluate low mobility criteria in PSCell if network configures, and apply the evaluated low mobility state in the corresponding cell group. - Proposal 1-2: - In NR-DC, the UE configured with low mobility criteria evaluates the low mobility criterion only on SpCell. - In NR-DC, if the relaxed measurement criterion (low mobility criteria) is met on a SpCell then the UE assumes that the relaxation criterion is met on all serving cells in the CG of that SpCell - Proposal 1-3: keep the previous agreement that evaluate low mobility criteria on PCell. #### Issue 1-9: Introduce minimum requirement at transitions FFS: Define the minimum requirement at transitions between relaxed and non-relaxed RLM/BFD measurements for OOS evaluation.

We do not see much technical difference between proposal 1-1 and 1-2, but proposal 1-2 is clearer in our view as it is more specific on the type of cells and CG. For relaxed RLM and BFD, the UE needs to evaluate the low mobility criteria (if configured) at least on the SpCell. The radio conditions and thus the measurement performance is expected to be similar for cells that belong to same CG in NR-DC. Therefore, if the UE is configured with multiple cells (e.g. using CG), then the UE can be allowed to apply the outcome of low mobility criterion evaluation to all remaining serving cells in the same CG. This may further improve the power consumption in the UE as evaluating the criteria separately for each serving cell may require additional consumption. Although the criterion is configured per UE, as shown below, proposal 1-2 (if agreed) can be captured in RAN4 specification as it was done for intra-band CA. Thus, no impact on other WG is foreseen.

lowMobilityEvaluationConnected Indicates the criterion for a UE to detect low mobility in RRC_CONNECTED in an SpCell. The s-SearchDeltaP-Connected is the parameter "SSearchDeltaP-connected". And the t-SearchDeltaP-Connected is the parameter " TSearchDeltaP-Connected". Low mobility criterion is configured in NR Pcell for the case of NR SA/ NR CA/ NE-DC/NR-DC, and in the NR PSCell for the case of EN-DC

Proposal 1

    - In NR-DC, the UE configured with low mobility criteria evaluates the low mobility criterion only on SpCell.

    - In NR-DC, if the relaxed measurement criterion (low mobility criteria) is met on a SpCell then the UE assumes that the relaxation criterion is met on all serving cells in the CG of that SpCell.

Regarding the second FFS related to minimum requirements at transition between relaxed and non-relaxed RLM/BFD, we do not see strong motivation to introduce such requirements given that RAN4 already agreed on related entering and exiting criteria.

Proposal 2		No need to introduce transition requirements between relaxed and non-relaxed RLM/BFD measurements.

# Summary

In this contribution we have provided our view on the remaining issues of relaxed radio link monitoring and beam management requirements. Based on the discussions, we have made following proposals:

Proposal 1

    - In NR-DC, the UE configured with low mobility criteria evaluates the low mobility criterion only on SpCell.

    - In NR-DC, if the relaxed measurement criterion (low mobility criteria) is met on a SpCell then the UE assumes that the relaxation criterion is met on all serving cells in the CG of that SpCell.

Proposal 2		No need to introduce transition requirements between relaxed and non-relaxed RLM/BFD measurements.

# References

- R4-2210613, "WF on RLM/BFD relaxation for UE Power Saving enhancements ", Moderator (MediaTek inc.)

- TS 38.133 V17.0.0