3GPP TSG-RAN WG4 Meeting #104-e	R4-2213421

Electronic Meeting, 15 – 26 Aug, 2022

Title:	General view on FR2 OTA testing enhancement

Source:	OPPO

Agenda item:	11.5.1

Document for:	Approval

- Introduction

The new study item of NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements was approved in RAN #95-e meeting. As the objective of this study item, the study and discussion of FR2 test methodologies will be focused on UEs with multiple panels, and test methods will be defined for RF, RRM and demodulation.

This is the first meeting for the technical discussion on this SI. This contribution provides our view on the test methodologies.

- Discussion

The test method for FR2 has been discussed and defined in TR 38.810 and TR 38.884 with the condition of DUT with single-panel. The new SI will develop the test methodology using TR 38.810 and TR 38.884 as the baseline. [1]

In Clause 5.1.2 of TR 38.884, the antenna panel simulation assumption for DUT is that two antenna arrays are integrated in the UE, front side and back side separately.

The simulation assumption can be reused for defining multi-panel simultaneously transmission test method. With this, the antenna panel in the front of DUT and the antenna panel in the back will transmit with up to 4 DL layers.

Observation 1:        The simulation assumption of DUT antenna panels in Clause 5.1.2 of TR 38.884 can be reused when study multi-panel simultaneously transmission test method.

With the above antenna panel location assumption, taking the two panels simultaneously transmission into account, the test zone of the corresponding test method is required to enclose the whole DUT. The permitted test methods, including DFF, IFF, NFTF in TR 38.810 and CFFNF, CFFDNF, CFFdeltaNF in TR 38.884, can not fully satisfy with the test zone requirement for multi panel simultaneously transmission measurement. Also, the asymptotic expansion approach need to be further verified on its accuracy of deriving the measurement power when two panels transmit simultaneously. And the method of local search for the black-box approach to determine the NF test direction probably need to be improved for the case of dual-panel simultaneously transmission.

In summary, the test zone requirement of test method should be studied in RAN4.

Proposal 1:             The test zone requirement of test method should be studied in RAN4.

- Conclusions

This paper discussed about the antenna panel location assumption and the test zone of the test method for multi-panel simultaneously transmission, and got the following conclusion.

Proposal 1:             The test zone requirement of test method should be studied in RAN4.

- References

[1] RP-220988, New SI: Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements, Qualcomm Incorporated