3GPPRAN4#104-e                	R4-2213461

# Electronic Meeting, 15th – 26th Aug., 2022

Agenda item:		9.12.1

Source:	MediaTek Inc.

Title:	Discussion on Rel-17 RLM/BFD measurement relaxation

Document for:		Discussion

1	Introduction

In RAN4 #103e meeting, RAN4 had reached consensus on most of the issues for Rel-17 power saving, i.e., relaxation applicability¸ details of good serving cell quality criterion and low mobility criterions [2]. In this paper, we provide views on the remaining issues including: clarifications for Low mobility criteria evaluation, whether to introduce minimum requirement at transitions, and how to reply the RAN2 LS sent in last meeting [2].

2	RLM/BFD measurement relaxation

In last meeting, open issues for relaxation applicability are listed as follows [1]:

Issue 1-8: Clarifications for Low mobility criteria evaluation For NR-DC, the following proposals can be discussed in R4#104-e: - Proposal 1-1: For NR-DC, UE may also evaluate low mobility criteria in PSCell if network configures, and apply the evaluated low mobility state in the corresponding cell group. - Proposal 1-2: - In NR-DC, the UE configured with low mobility criteria evaluates the low mobility criterion only on SpCell. - In NR-DC, if the relaxed measurement criterion (low mobility criteria) is met on a SpCell then the UE assumes that the relaxation criterion is met on all serving cells in the CG of that SpCell - Proposal 1-3: keep the previous agreement that evaluate low mobility criteria on PCell. Issue 1-9: Introduce minimum requirement at transitions FFS: Define the minimum requirement at transitions between relaxed and non-relaxed RLM/BFD measurements for OOS evaluation.

For issue 1-8, considering that mobility state in PCell and PSCell should be very similar, we prefer to use simple design and keep the previous agreement. So we propose

Proposal 1: For NR-DC, keep the previous agreement that evaluate low mobility criteria on PCell

For issue 1-9, we suggest

Proposal 2: Transitions requirement between relaxed and non-relaxed RLM/BFD measurements are not specified

3	Other remaining issues

In last meeting, RAN2 LS was sent.

RAN2 would like to thank RAN4 for the Reply LS to RAN2 on RLM/BFD relaxation for ePowSav. Based on further RAN4 progress, RAN2 discussed the signaling for RLM/BFD relaxation, and achieved the following agreements: - Keep the current configuration for serving cell quality criterion as per-serving cell basis in RRC specification. More specifically, this means that the goodServingCell threshold for BFD can be configured per serving cell, and for RLM can be configured per SpCell.. RAN2 would like to check with RAN4 whether you have any concern. Besides, for low mobility criterion, RAN2 also made the following agreements: - For low mobility criterion, reuse the values of Rel-16 s-SearchDeltaP and t-SearchDeltaP for Rel-17 s-SearchDeltaP-Connected and t-SearchDeltaPConnected, respectively. - MN informs SN when low mobility criterion has been configured in NR PCell. How to capture it could be further discussed in CR (assume impact to TS38.331 and TS37.340) - For the R17 low mobility criterion, the UE considers the relaxed measurement criterion is fulfilled only when the defined criterion formula is fulfilled for a period of TSearchDeltaP-Connected. (Assume this is consistent with RAN4 decisions). - Capture TSearchDeltaP-Connected for low mobility criterion in RAN2 specification (as long as this does not contradict R4)..

For issue 1, it is reasonable that RLM is per SpCell configured and BFD is per serving cell configured. For issue 2. It has been solved in last RAN4 meeting and corresponding CR has been agreed, as shown below:

8.5.1.1 Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria For the UE supports bfd-Relaxation-r17 and configured with dedicated signaling goodServingCellEvaluationBFD, which is always configured to the UE when the network enables BFD relaxation for the UE as specified in TS 38.331[2], the relaxed requirements defined in clause 8.5.2.4 for SSB based beam failure detection and the relaxed requirements defined in clause 8.5.3.4 for CSI-RS based beam failure detection are allowed to apply to the relaxed BFD measuremetns on the serving cell after fulfilling the following conditions: - The good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] is fulfilled for any resource in the set of resources for beam failure detection, and - for the UE which is not performing intra-band carrier aggregation or for the UE which is performing intra-band carrier aggregation but not configured with SSB-based or CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell, when - the UE has fulfilled good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] for the serving cell configured with BFD-RS if the lowMobilityEvaluationConnected is not configured, or - the UE is also configured with lowMobilityEvaluationConnected and UE has fulfilled both low mobility criterion defined in clause 5.7.13.1 of TS 38.331 [2] for a period of TSearchDeltaP-Connected and good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] for the serving cell configured with BFD-RS.

So we propose to reply RAN2

Proposal 3: RAN4 to reply issues addressed in LS R2-2206675.

- RAN4 has no concern on goodServingCell criterion for BFD can be configured per serving cell, and for RLM can be configured per SpCell.

- RAN4 has already captured in the spec TS38.133 that UE shall consider the low mobility criterion is fulfilled only when the defined criterion formula is fulfilled for a period of TSearchDeltaP-Connected. There is no need to further modify the RAN2 spec.

4	Summary

In this contribution, we have following proposals:

Proposal 1: For NR-DC, keep the previous agreement that evaluate low mobility criteria on PCell

Proposal 2: Transitions requirement between relaxed and non-relaxed RLM/BFD measurements are not specified

Proposal 3: RAN4 to reply issues addressed in LS R2-2206675.

- RAN4 has no concern on goodServingCell criterion for BFD can be configured per serving cell, and for RLM can be configured per SpCell.

- RAN4 has already captured in the spec TS38.133 that UE shall consider the low mobility criterion is fulfilled only when the defined criterion formula is fulfilled for a period of TSearchDeltaP-Connected. There is no need to further modify the RAN2 spec.

Reference

- R4-2210613, “WF on RLM/BFD relaxation for UE Power Saving enhancements,” MediaTek, May, 2022.

- R2-2206675, “Reply LS to RAN4 on RLM/BFD relaxation,” VIVO, May, 2022.