3GPP TSG-RAN WG4 Meeting #104-e	R4-2213552

Electronic Meeting, 15 – 26 August, 2022

Title: 	On RRM requirements for PDC enhancements

Source: 	Huawei, HiSilicon

Agenda item:	9.21.1.1

Document for:	Discussion

Introduction

RRM requirements for PDC measurement are discussed in RAN4#103-e, and the outcomes are captured in WF [1]. Based on [1] the following issues needs to be further discussed:

- Applicable capability for PRS based PDC

- Number of samples for TRS based PDC

- Handling MG in PDC requirements

- DRX requirements

In this paper we will provide our views on above issues for PDC measurement.

Discussion

Applicable capability for PRS based PDC

Issue 1-2: RAN4 waits for RAN1 conclusion on the applicable capabilities for PRS based PDC? Agreement after 1st round: - RAN4 keep the impacted parameters in [] until RAN1 has reached agreement. RAN4 can additionally add an editor’s note: ‘Pending agreement on UE processing capability discussion’

For PRS based PDC, RAN4 has re-used the requirements from PRS measurement for positioning, and one remaining issue is whether the UE processing capability FG 13-1, which are reflected in the measurement for positioning, is applicable for PDC measurement.

| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | 1. Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS 2. Max number of DL PRS Resources in DL PRS Resource Set for PDC Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 3. Max number of DL PRS resources that UE can process in a slot. a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | [25-19, 2-53] |
| --- | --- | --- | --- | --- |

Based on the latest RAN1 feature list, we understand that processing capability {N,T} is not applicable for PDC, while the capability N’ (number of PRS resources per slot) is still applicable. Therefore, we suggest to make the following changes to PRS based PDC requirements.

Proposal 1: Update the PRS based PDC requirements as follow.

TUERxTx, PRS=NPRSslotN'Lavailable_PRSN*Nsample-1*TeffectPRS+Tlast

Number of samples for TRS based PDC

Issue 1-3: The number of samples assumed for deriving the accuracy requirements [1 or 4]? Way forward: further discuss - Option 1: samples assumed for deriving the accuracy requirements: 1 - Option 2: samples assumed for deriving the accuracy requirements: 4

One open issue from [1] is the sample number for TRS based PDC, i.e. 1 or 4. We suggestion is 4.

It was agreed that for PRS based PDC the sample number is 4, and in our view it is better to have aligned measurement period for two RS-es. Also, based on our simulation results, 4-sample would provide better accuracy especially for the small BW.

Proposal 2: Define measurement period for TRS based PDC based on 4 samples.

Handling MG in PDC requirements

Issue 1-4: Is there a need to introduce scaling factor Kgap when the PDC resources occasions collide with MG? Agreement in GTW May 18th: further discuss below options: - Option 1: Introduce scaling factor Kgap when the PDC resources occasions collide with legacy MG only - Option 3: No requirements defined for when PDC resources collide with gaps - Clarification on the requirements applicable conditions need to included into specification for this option

We do not have strong preference between the two options as both can resolve the collision between MG and PDC measurement. We slightly prefer option 1 as it allows more flexibility for the NW.

Proposal 3: Introduce a scaling factor Kgap to account for the PDC resources occasions dropped due to collision with MG.

DRX requirements

Issue 1-8: Is there a need to define DRX requirements for PDC measurement? Way forward/Agreement: further discuss - Option 1: Yes, define DRX requirements for PDC such that UE is assumed to take one sample per DRX cycle. - Option 2: Other Agreement in GTW may 18th: FFS whether define DRX requirements for PDC measurement

We prefer option 1 which is simply following the same principle as existing RRM or L1 measurement requirements, i.e. DRX is considered such that UE is assumed to take one sample per DRX cycle.

In last meeting there was a discussion about whether PDC measurement is time critical. In our understanding, PDC measurement is a periodic measurement configured by the gNB, so it is not time critical. If gNB considers a PDC measurement is urgently needed it can de-configure the DRX, but in case DRX is configured, having a periodic measurement performed at the RS periodicity will negatively impact UE power consumption and be contradicting with the motivation of DRX configuration.

Proposal 4: Define DRX requirements for PDC such that UE is assumed to take one sample per DRX cycle.

Conclusions

In this paper we provided our views on PDC measurement.

Proposal 1: Update the PRS based PDC requirements as follow.

TUERxTx, PRS=NPRSslotN'Lavailable_PRSN*Nsample-1*TeffectPRS+Tlast

Proposal 2: Define measurement period for TRS based PDC based on 4 samples.

Proposal 3: Introduce a scaling factor Kgap to account for the PDC resources occasions dropped due to collision with MG.

Proposal 4: Define DRX requirements for PDC such that UE is assumed to take one sample per DRX cycle.

Reference

- R4-2210618, WF on NR_IIOT_URLLC_enh, Nokia, Nokia Shanghai Bell