3GPP TSG-RAN WG4 Meeting # 104-e		R4-2213569

# Electronic Meeting, 15 – 26 August 2022

Source:	Sony

Title:	Views on switching time UL Tx switching across 3 or 4 bands in Rel-18

Agenda item:	11.15.2

Document for:	Discussion

- Background

In [1], an LS from RAN1 is sent to RAN4 with the aspects of switching time UL Tx switching across 3 or 4 bands:

- RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on potential increase of switching period and UE's complexity in case of UL Tx switching across 3 or 4 bands in comparison to 2 bands.

- RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands.

  - "When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period."

In this contribution, we share our views on switching time when the number of bands are increased.

- Discussion

In practical implementation, multiple factors affect the switching time, including the retuning in the RF part as well as the digital part (e.g., baseband bandwidth, filter, and data generation). In the RF domain, the switching time is highly dependent on the UE implementations. For example, the shortest switching time can be achieved if the UE has independent Tx chains for each band, while a longer switching time is required when the PLL and the PA are shared among multiple frequency bands.

The switching time between two carriers has been intensively discussed previously in RAN4, with some reference architecture and proposed switching time [2] [3]. Theoretically, the Tx switching time does not directly relate to the number of frequency bands that the UE needs to support. For example, if the UE is implemented with an entirely dedicated RF chain for each band, the switching time could remain the same when the number of frequency bands is increased. However, this leads to a more costly implementation and may not be the mainstream choice.

With an increased number of frequency bands to 3 or 4, it can be foreseen that some RF components, e.g., PLL and PA, need to be shared between multiple frequency bands, which will require additional retuning time. In addition, if the frequency separation also is enlarged along with the increased number of frequency bands, it could be foreseen that returning time of some RF components would also go up. For example, the PLL retuning time could be increased if the frequency separation between the two frequency bands is enlarged. Of course, this is still up to the PLL design and how the VCO in the PLL block are implemented. For example, if one wideband VCO is used to cover the 3 or 4 frequency bands that the UE needs to switch between, the tuning time is likely to be large due to the limited performance of the varactor diode. On the other hand, there are also possible implementations for faster retuning time, e.g., having separate VCOs for low, mid, and high bands.

Observation 1	The switching time does not directly depend on the number of frequency bands. However, the increased number of frequency bands may lead to a design where the RF chain is shared between bands to a larger extent and may also lead to a larger frequency separation between the bands. This is more challenging for shorter switching period implementation.

Meanwhile, it can be observed that different switching period has been agreed upon for switching between the two carriers [4]. It is our understanding that it is more challenging to maintain the switching period with an increased number of bands, but it may not go beyond the values that RAN4 previously agreed to.

Observation 2	The switching time defined by RAN4 for two carriers may be sufficient for the case of 3 or 4 bands.

- Conclusion

In this contribution, we have discussed the switching time for UL Tx switching across 3 or 4 bands in Rel-18 and the effecting factors. The following observations have been made:

Observation 1	The switching time does not directly depend on the number of frequency bands. However, the increased number of frequency bands may lead to a design where the RF chain is shared between bands to a larger extent and may also lead to a larger frequency separation between the bands. This is more challenging for shorter switching period implementation.

Observation 2	The switching time defined by RAN4 for two carriers may be sufficient for the case of 3 or 4 bands.

- Reference

- R1-2205642,		“LS on UL Segmented Transmission for UL synchronization for IoT NTN”, RAN1

- R4-1910751,		"Switching times and reference architectures", Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #92-Bis, Chongqing, China, 14 – 18 Oct 2019

- R4-1914164, 	"Discussion on the switching between 1Tx carrier and 2Tx carrier", Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #93, Reno, NV, USA, 14-18 Nov 2019

- R4-2002816,		“LS on Tx switching between two uplink carriers”, RAN4