3GPP TSG-RAN WG4 Meeting #104-e                                                            R4-2213599

# Electronic Meeting, August 15 – August 26, 2022

Source:	ZTE Corporation

Title:	Simplification on multiple UL CA configurations in CA configuration table

Agenda Item:	11.3.3

Document for:	Approval

- 1	Introduction

In the configuration tables of EN-DC in TS 38.101-3, for the sake of brevity and to reduce the size of DC configuration tables, the downlink configurations are grouped for the configurations having the same component frequency bands. Regarding to the UL DC configurations, if multiple UL DC configurations are listed for multiple DL DC configurations, valid uplink configurations are such that uplink does not have more carriers than downlink. For example, for the configuration DC_1A_n257J, the UL DC configurations can be in the list of DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I and DC_1A_n257J, while DC_1A_n257K, DC_1A_n257L and DC_1A_n257M are the invalid uplink configurations for this case.

Example 1 (EN-DC configuration):

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n257A DC_1A_n257D DC_1A_n257E DC_1A_n257F DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M |

However, for the CA configuration tables in TS 38.101-2, all the possible uplink CA configurations are listed in the table, which does not use the same rule as in TS 38.101-3 and results in the redundancy of the configuration table.

Example 2 (Intra-band CA within FR2):

In this paper, we propose to analyse how to simplify the UL CA configurations in CA configuration tables.

- 2	Discussion

As discussed above, the intra-band CA configurations and inter-band CA configurations within FR2 in TS 38.101-2 list all possible UL CA configurations. Furthermore, if we look at inter-band CA configuration between FR1 and FR2 in TS 38.101-3, we see that all possible UL CA configurations are listed in the CA configuration table.

Example 3 (Inter-band CA between FR1 and FR2):

Observation 1:	 The uplink CA configurations within FR2 in TS 38.101-2 and the uplink CA configurations between FR1 and FR2 in TS 38.101-3 list all possible UL configurations in the CA configuration table, which make the table quite redundant and inconsistent with rules for DC configurations.

Here we propose to apply the same rule in CA configuration table for uplink as in DC configuration table.

Proposal 1:	 For uplink CA configuration, it is suggested that the same rules as in DC configuration table be applied to CA configuration table, i.e. if multiple UL CA configurations are listed for multiple DL CA configurations, valid uplink CA configurations are such that uplink does not have more carriers than downlink.

Based on the Proposal 1, we have the following revised uplink CA configurations for the above examples.

Example 2a (Intra-band CA within FR2 – Revised UL Configurations):

Example 3a (Inter-band CA between FR1 and FR2 – Revised UL Configurations)

Proposal 2:	 It is proposed to apply the rules in Proposal 1 for uplink CA band combination request.

- 4	Conclusion

In this paper, we analyse how to simplify the UL CA configurations in CA configuration tables. The following observation and proposals are proposed.

Observation 1:	 The uplink CA configurations within FR2 in TS 38.101-2 and the uplink CA configurations between FR1 and FR2 in TS 38.101-3 list all possible UL configurations in the CA configuration table, which make the table quite redundant and inconsistent with rules for DC configurations.

Proposal 1:	 For uplink CA configuration, it is suggested that the same rules as in DC configuration table be applied to CA configuration table, i.e. if multiple UL CA configurations are listed for multiple DL CA configurations, valid uplink CA configurations are such that uplink does not have more carriers than downlink.

Proposal 2:	 It is proposed to apply the rules in Proposal 1 for uplink CA band combination request.

- 5	References

[1] TS 38.101-2, NR User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone.

[2] TS 38.101-3, NR User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios.