3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213617

# Electronic meeting, August 15 - August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0748 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-3 on corrections to MSD test points for DC_1A-20A_n8A, DC_1A-20A_n38A and DC_1A-20A_n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, DC_R17_2BLTE_1BNR_3DL2UL-Core |  |  |  |  |  | Date: |  |  | 2022-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MSD test points for DC_1A-20A_n8A and DC_1A-20A_n38A are incorrectly specified as DC_1A-20A_n78A. The MSD test points for DC_1A-20A_n78(2A) should be added as the same as DC_1A-20A_n78A. Furthermore, some editorial regrouping and reordering should be made for the MSD test points for some configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the MSD test points for DC_1A_20A_n8A, DC_1A-20A_n38A and DC_1A-20A_n78A. - Regroup and reorder the MSD test points for DC_1A-3A_n28A, DC_1A_n3A-n28A, DC_1A_n3A-n41A, DC_1A_n3A-n77A, DC_1A_n8A-n40A, DC_1A_n8A-n78A, DC_1A-11A_n77A, DC_1A-11A_n78A, DC_1A-11A_n79A, DC_1A-19A_n77A, DC_1A-19A_n79A, DC_1A-28A_n3A, DC_1A-28A_n7A, DC_1A-28A_n77A, DC_1A-28A_n79A, DC_1A_n28A-n40A, DC_1A-38A_n78A, DC_1A-41A_n77A, DC_1A-41A_n78A and DC_1A_n75A-n78A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The MSD test points for DC_1A_20A_n8A, DC_1A-20A_n38A and DC_1A-20A_n78A will be incorrect. - The MSD test points for the mentioned DC configurations will be in disorder. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-3 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  |  | 3 |  | 1723.5 |  | 5 |  | 25 |  | 1818.5 |  | 4.0 |  | IMD5 |  |
|  |  | n28 |  | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1949 |  | 5 |  | 25 |  | 2139 |  | 11.0 |  | IMD4 |  |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 | 1960 | 5 | 25 | 2150 | 5 | IMD4 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  |  | n71 |  | 675 |  | 5 |  | 25 |  | 629 |  | N/A |  | N/A |  |
| DC_1A_n3A-n28A |  | 1 |  | 1975 |  | 5 |  | 25 |  | 2165 |  | N/A |  | N/A |  |
|  |  | n3 |  | 1723.5 |  | 5 |  | 25 |  | 1818.5 |  | 4.0 |  | IMD5 |  |
|  |  | n28 |  | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |
| DC_1A_n3A-n41A |  | 1 |  | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | N/A |  | N/A |  |
|  |  | n3 |  | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | N/A |  | N/A |  |
|  | n41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |
| DC_1A_n3A-n75A | n75 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| DC_1A_n3A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n79 | 4950 | 40 | 216 | 4950 | 4.7 | IMD5 |
| DC_1A-7A_n28A DC_1A-7C_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |
| DC_1A-7A_n40A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_1A_n8A-n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
| DC_1A_n8A-n77(2A) | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | 1.5 | IMD5 |
| DC_1A_n8A-n77A | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
| DC_1A_n8A-n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | 8.8 | IMD3 |
| DC_1A_n8A-n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
| DC_1A_n8A-n77(2A) | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | 3.3 | IMD5 |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A_n8A-n77(2A) | 8 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-3A_n77A DC_1A-3A_n77(2A) DC_1A-3A_n77(3A) DC_1A-3C_n77A DC_1A-3A_n77C DC_1A-3C_n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_1A-3A_n78A DC_1A-3C_n78A DC_1A-3A_n78C DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  |  | n78 |  | 3725 |  | 10 |  | 50 |  | 3725 |  | N/A |  | N/A |  |
| DC_1A_n3A-n77A DC_1A_n3A-n77(2A) |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | n3 |  | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3700 |  | 10 |  | 50 |  | 3700 |  | 28.4 |  | IMD2 |  |
|  |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | n3 |  | 1770 |  | 5 |  | 25 |  | 1865 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3360 |  | 10 |  | 50 |  | 3360 |  | 11.2 |  | IMD4 |  |
|  |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | n3 |  | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | 31.5 |  | IMD2 |  |
|  |  | n77 |  | 3757.5 |  | 10 |  | 50 |  | 3757.5 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | n3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 8.5 |  | IMD4 |  |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
| DC_1A-5A_n77A DC_1A-5A_n77(2A) | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n77 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-5A_n78A DC_1A-5A_n78C | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-7A_n77A DC_1A-7A_n77(2A) DC_1A-7A-7A_n77A DC_1A-7A-7A_n77(2A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD44 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A-7A_n78A DC_1A-7C_n78A DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) DC_1A-7A_n78C DC_1A-1A-7A_n78A DC_1A-7A-7A_n78C | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A DC_1A_n7A-n78(2A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |
| DC_1A-5A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 8.9 | IMD4 |
|  | n79 | 4907.5 | 40 | 216 | 4907.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 5 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-8A_n28A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 3.3 | IMD5 |
| DC_1A-8A_n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 8.0 | IMD4 |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | 1 | 1945 | 5 | 25 | 2135 | 5.3 | IMD5 |
|  | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 3.3 | IMD5 |
| DC_1A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 14.4 | IMD3 |
| DC_1A-8A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n79 | 4815 | 40 | 216 | 4815 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 15.8 | IMD3 |
| DC_1A-8A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | N/A |
|  |  | 1 |  | 1955 |  | 5 |  | 25 |  | 2145 |  | 8.2 |  | IMD4 |  |
| DC_1A_n8A-n40A |  | 1 |  | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |
|  |  | n8 |  | 885 |  | 5 |  | 25 |  | 930 |  | 8.0 |  | IMD4 |  |
|  |  | n40 |  | 2395 |  | 5 |  | 25 |  | 2395 |  | N/A |  | N/A |  |
| DC_1A_n8A-n78A |  | 1 |  | 1945 |  | 5 |  | 25 |  | 2135 |  | N/A |  | N/A |  |
|  |  | n8 |  | 900 |  | 5 |  | 25 |  | 945 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3745 |  | 10 |  | 50 |  | 3745 |  | 14.9 |  | IMD3 |  |
|  |  | 1 |  | 1940 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |  |
|  |  | n8 |  | 895 |  | 5 |  | 25 |  | 940 |  | 3.3 |  | IMD5 |  |
|  | n78 | 3380 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-11A_n3A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 11 | 1432 | 5 | 25 | 1480 | 15.2 | IMD3 |
| DC_1A-11A_n28A | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | 1 | 1960 | 5 | 25 | 2150 | 28.3 | IMD21 |
| DC_1A-11A_n41A | 11 | 1442 | 5 | 25 | 1490 | N/A | N/A |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | N/A |
|  | 1 | 1966 | 5 | 25 | 2156 | 10.2 | IMD4 |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
|  | 11 | 1442 | 5 | 25 | 1490 | 10.6 | IMD4 |
| DC_1A-11A_n77A DC_1A-11A_n77(2A) DC_1A-11A_n77(3A) | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  |  | 11 |  | 1438 |  | 5 |  | 25 |  | 1486 |  | 31.4 |  | IMD2 |  |
|  |  | n77 |  | 3441 |  | 10 |  | 50 |  | 3441 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 30.8 |  | IMD2 |  |
|  |  | 11 |  | 1438 |  | 5 |  | 25 |  | 1486 |  | N/A |  | N/A |  |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | N/A |
| DC_1A-11A_n78A DC_1A-11A_n78(2A) | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  |  | 11 |  | 1438 |  | 5 |  | 25 |  | 1486 |  | 31.4 |  | IMD2 |  |
|  |  | n78 |  | 3441 |  | 10 |  | 50 |  | 3441 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 30.8 |  | IMD2 |  |
|  |  | 11 |  | 1438 |  | 5 |  | 25 |  | 1486 |  | N/A |  | N/A |  |
|  | n78 | 3578 | 10 | 50 | 3578 | N/A | N/A |
| DC_1A-11A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  |  | 11 |  | 1435 |  | 5 |  | 25 |  | 1483 |  | 10.2 |  | IMD4 |  |
|  |  | n79 |  | 4427 |  | 40 |  | 216 |  | 4427 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1928 |  | 5 |  | 25 |  | 2118 |  | 15.6 |  | IMD3 |  |
|  |  | 11 |  | 1431 |  | 5 |  | 25 |  | 1479 |  | N/A |  | N/A |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
| DC_1A-18A_n77A DC_1A-18A_n77(2A) | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| DC_1A-18A_n78A DC_1A-18A_n78(2A) | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |
|  |  | n79 |  | 4592.5 |  | 40 |  | 216 |  | 4592.5 |  | N/A |  | N/A |  |
| DC_1A-19A_n77A |  | 1 |  | 1940 |  | 5 |  | 25 |  | 2130 |  | 17.8 |  | IMD3 |  |
|  |  | 19 |  | 832.5 |  | 5 |  | 25 |  | 877.5 |  | N/A |  | N/A |  |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | n78 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |
| DC_1A-19A_n79A |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  |  | 19 |  | 837.5 |  | 5 |  | 25 |  | 882.5 |  | 18.3 |  | IMD3 |  |
|  |  | n79 |  | 4782.5 |  | 40 |  | 216 |  | 4782.5 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 8.1 |  | IMD4 |  |
|  |  | 19 |  | 837.5 |  | 5 |  | 25 |  | 882.5 |  | N/A |  | N/A |  |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A_n28A-n41A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n41 | 2653 | 10 | 50 | 2653 | 30.1 | IMD2 |
|  | 1 | 1923 | 5 | 25 | 2113 | N/A | N/A |
|  | n28 | 707 | 5 | 25 | 762 | 29.3 | IMD2 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 730 | 10 | 50 | 785 | 4.5 | IMD5 |
|  | n41 | 2510 | 10 | 50 | 2510 | N/A | N/A |
| DC_1A-20A_n7A | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
| DC_1A-20A_n8A |  | 1 |  | 1925 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |
|  |  | 20 |  | 846 |  | 5 |  | 25 |  | 805 |  | 11.5 |  | IMD4 |  |
|  |  | n8 |  | 910 |  | 5 |  | 25 |  | 955 |  | N/A |  | N/A |  |
| DC_1A-20A_n38A |  | 1 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 20 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |
| DC_1A-20A_n78(2A) |  | 20 |  | 835 |  | 5 |  | 25 |  | 794 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-21A_n28A10 | 1 | 1975.3 | 5 | 25 | 2165.3 | 16.1 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  |  | n79 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| DC_1A-28A_n3A |  | 1 |  | 1949 |  | 5 |  | 25 |  | 2139 |  | 11.0 |  | IMD4 |  |
|  |  | 28 |  | 710.5 |  | 5 |  | 25 |  | 765.5 |  | N/A |  | N/A |  |
|  |  | n3 |  | 1780 |  | 5 |  | 25 |  | 1875 |  | N/A |  | N/A |  |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B |  | 1 |  | 1935 |  | 5 |  | 25 |  | 2125 |  | N/A |  | N/A |  |
|  |  | 28 |  | 730 |  | 10 |  | 50 |  | 785 |  | 4.5 |  | IMD5 |  |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
| DC_1A-28A_n40A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 8.9 | IMD4 |
|  |  | n40 |  | 2340 |  | 5 |  | 25 |  | 2340 |  | N/A |  | N/A |  |
| DC_1A-28A_n77A |  | 1 |  | 1960 |  | 5 |  | 25 |  | 2150 |  | 15.7 |  | IMD3 |  |
|  |  | 28 |  | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3630 |  | 10 |  | 50 |  | 3630 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |
|  |  | 28 |  | 739 |  | 5 |  | 25 |  | 794 |  | 4.2 |  | IMD5 |  |
|  | n77 | 3352 | 10 | 50 | 3352 | N/A | N/A |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  |  | 28 |  | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3630 |  | 10 |  | 50 |  | 3630 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |
|  |  | n78 |  | 3352 |  | 10 |  | 50 |  | 3352 |  | N/A |  | N/A |  |
| DC_1A-28A_n79A |  | 1 |  | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |
|  |  | 28 |  | 733 |  | 5 |  | 25 |  | 788 |  | 15.2 |  | IMD3 |  |
|  |  | n79 |  | 4648 |  | 40 |  | 216 |  | 4648 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1925 |  | 5 |  | 25 |  | 2115 |  | N/A |  | N/A |  |
|  |  | 28 |  | 740 |  | 5 |  | 25 |  | 795 |  | 10.0 |  | IMD4 |  |
|  |  | n79 |  | 4980 |  | 40 |  | 216 |  | 4980 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1977.5 |  | 5 |  | 25 |  | 2167.5 |  | 1.2 |  | IMD4 |  |
|  |  | 28 |  | 745.5 |  | 5 |  | 25 |  | 800.5 |  | N/A |  | N/A |  |
|  |  | n79 |  | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A |  | N/A |  |
|  |  | 1 |  | 1935 |  | 5 |  | 25 |  | 2125 |  | 4.5 |  | IMD5 |  |
|  |  | 28 |  | 718 |  | 5 |  | 25 |  | 773 |  | N/A |  | N/A |  |
|  |  | n79 |  | 4807 |  | 40 |  | 216 |  | 4807 |  | N/A |  | N/A |  |
| DC_1A_n28A-n40A |  | 1 |  | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |
|  |  | n28 |  | 743 |  | 5 |  | 25 |  | 798 |  | N/A |  | N/A |  |
|  |  | n40 |  | 2374 |  | 5 |  | 25 |  | 2374 |  | 10.1 |  | IMD4 |  |
|  |  | 1 |  | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |
|  |  | n28 |  | 713 |  | 5 |  | 25 |  | 768 |  | 8.6 |  | IMD4 |  |
|  | n40 | 2314 | 5 | 25 | 2314 | N/A | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 4.2 | IMD5 |
| DC_1A_n28A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | 15.2 | IMD39 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n79 | 4630 | 40 | 216 | 4630 | 14.9 | IMD34 |
| DC_1A-32A_n3A | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| DC_1A-32A_n78A DC_1A-32A_n78C DC_1A-32A_n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 31.8 | IMD2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 0 | IMD5 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-38A_n78A DC_1A-38A_n78(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 38 | 2590 | 5 | 25 | 2590 | 12.7 | IMD4 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_1A_n38A-n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n38 | 2590 | 5 | 25 | 2590 | 12.7 | IMD4 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 40 | 2340 | 5 | 25 | 2340 | 10.6 | IMD4 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.1 | IMD4 |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n78 | 3430 | 10 | 50 | 3430 | N/A | N/A |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | IMD4 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
| DC_1A-41A_n77A DC_1A-41C_n77A DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) |  | 1 |  | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |
|  |  | 41 |  | 2510 |  | 5 |  | 25 |  | 2510 |  | N/A |  | IMD4 |  |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  |  | 1 |  | 1930 |  | 5 |  | 25 |  | 2120 |  | 11.0 |  | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | IMD5 |
|  |  | n77 |  | 4150 |  | 10 |  | 50 |  | 4150 |  | N/A |  |  |  |
| DC_1A-41A_n78A DC_1A-41C_n78A DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 41 | 2515 | 5 | 25 | 2515 | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
| DC_1A_n41A-n77A DC_1A_n41A-n77(2A) DC_1A_n41A-n78A DC_1A_n41A-n78(2A) | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n41 | 2515 | 10 | 50 | 2515 | 11.5 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  |  | n78 |  | 3330 |  | 10 |  | 50 |  | 3330 |  | 19.6 |  | IMD3 |  |
| DC_1A-41A_n79A |  | 1 |  | 1970 |  | 5 |  | 25 |  | 2160 |  | N/A |  | N/A |  |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | IMD2 |
|  |  | n79 |  | 4500 |  | 40 |  | 216 |  | 4500 |  | N/A |  | N/A |  |
| DC_1A-42A_n3A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | 42 | 3425 | 5 | 25 | 3425 | 13.0 | IMD4 |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | 42 | 3416 | 5 | 25 | 3416 | 15.7 | IMD3 |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | n28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | 1 | 1944 | 5 | 25 | 2134 | 15.7 | IMD3 |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_1A_SUL_n77A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
| DC_1A_SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  |  | n78 |  | 3425 |  | 10 |  | 50 |  | 3425 |  | 13.0 |  | IMD4 |  |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) |  | 1 |  | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | N/A |  |
|  |  | n75 |  | - |  | - |  | - |  | 1470 |  | 30.4 |  | IMD2 |  |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |
| DC_1A_SUL_n78A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| << Unchanged configurations omitted >> |  |  |  |  |  |  |  |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 2: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified NOTE 3: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_2A-46A_n77A; DC_2A_n77A; fc_2A + fc_n77A; BW_2A + BW_n77A. DC_2A-46A_n77A; DC_2A_n77A; -fc_2A + 2*fc_n77A; -BW_2A + 2*BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 2*fc_13A + fc_n77A; 2*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 3*fc_13A + fc_n77A; 3*BW_13A + BW_n77A. DC_13A-46A_n2A; DC_13A_n2A; 2*fc_n2A + 2*fc_13A; 2*BW_n2A+2*BW_13A. DC_13A-46A_n77A; DC_13A_n77A; -3*fc_13A + 2*fc_n77A; -3*BW_13A + 2*BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; fc_66A + fc_n77A; BW_66A + BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; -fc_66A + 2*fc_n77A; -BW_66A + 2*BW_n77A. DC_13A-46A_n66A; DC_13A_n66A; 3*fc_13A + fc_n66A; BW_13A + 2*BW_n66A. DC_13A-46A_n66A; DC_13A_n66A; 2*fc_13A + 3*fc_n66A; BW_13A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; fc_48A + fc_n66A; BW_48A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; 2*fc_48A + fc_n66A; 2*BW_48A + BW_n66A. DC_2A-46_n5A; DC_2A_n5A; 2*fc_2A + 2*fc_n5A; BW_2A + 2*BW_n5A. DC_2A-46_n5A; DC_2A_n5A; fc_2A + 4*fc_n5A; BW_2*2A + BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + fc_n5A; BW_48A + 2*BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + 2*fc_n5A; BW_2*48A + BW_n5A. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. NOTE 8: Band 5 is also affected by IMD5 from UL DC_2A_n12A, but MSD value is not specified as there is only partial overlap of IMD5 with DL carrier. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 10: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2 fall in B1 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped.NOTE 12: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 13: For the DC band combination, simultaneous Rx/Tx capability is allowed between n78 and n79 NOTE 14: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |

==============================================================

<< End of changes >>