3GPP TSG-RAN WG4 Meeting #104-e                                                    	R4-2213633

Electronic Meeting, 15 – 26 August, 2022

Source:	Huawei, Hisilicon

Title:	TP for TR 37.718-31-11 DC_3C-7A-28A_n1A with DL_3C-28A_n1A_UL_3A_n1A, DL_3C-28A_n1A_UL_3C_n1A and  DL_3C-28A_n1A_UL_28A_n1A

Agenda item:			10.5.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-31-11 to include DC_3A-7A-38A_n78A and DC_3C-7A-38A_n78A.

# 2. Text Proposal

<Start of Text Proposal>

5.1.x	DC_3-7-28_n1

5.1.x.1	 Configurations for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3C-7A-28A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_28A_n1A |

5.1.x.2	 ∆TIB and ∆RIB values

Table 5.1.x.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.5 |
|  | n1 | 0.6 |

Table 5.1.x.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n1 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

5.1.x.3		Reference sensitivity exceptions

No additional MSD requirement needs to be defined for this dual connectivity configuration.

<End of Text Proposal>