3GPP TSG-RAN WG4 Meeting # 104-e 	R4-2213645

Electronic Meeting, August 15th - 26th, 2022

Agenda Item: 9.18.3.1.4

Source: MediaTek Inc.

Title: Discussion on signaling characteristic in RedCap

Document for: Discussion

# Introduction

In this contribution paper, we discuss the remaining open issues on signalling characteristic captured in the way forward (WF) [1].

# Discussion on BFD open issues

The open issues from the WF are given below:

Whether to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17 Candidate options: - Option 1 (QC): RAN4 to define further relaxations (e.g., larger K values) to RLM/BFD measurements for a Rel-17 RedCap UE that is configured with and satisfies the stationary criterion. - Option 2 (HW, Apple, CMCC, vivo, OPPO): Relaxed RLM/BFD based on Rel-17 UE power saving WI is out of RedCap WI scope. - Option 3 (E///, MTK, Xiaomi, MTK): FFS under maintenance If further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI is defined for RedCap in Rel-17 Candidate options: - Option 1 (QC): RAN4 to consider defining the applicability of power-saving enhancements to longer DRX cycles (up-to 160ms) to allow stationary UEs save significant power.

During RAN4 discussion on RedCap rel-17, companies agreed to include small data transmission (SDT) capability in RedCap due to its advantage in enhancing the INACTIVE mode for RedCap. Now, in a similar manner, the capability of rlm/bfd-Relaxation-r17 can be combined in RedCap rel-17 due to its power saving advantages in RLM/BFD. Besides, the current requirements for rlm/bfd-Relaxation-r17 are already finalised, therefore, the effort of having this new power saving relaxation in RLM/BFD RedCap is not high. Thus, we support to define further relaxation to RLM/BFD measurements for rel-17 RedCap when the UE satisfy the relaxed measurement criteria.

- RAN4 has already supported a new rel-17 capability (namely SDT) with RedCap, hence, in the same manner RAN4 can further relax RLM/BFD based on RLM/BFD power saving relaxation.

- RAN4 to define further relaxations (e.g., introduce K values) to RLM/BFD measurements for a Rel-17 RedCap UE that is configured with and satisfies RLM/BFD relaxation criteria.

# Summary

In this contribution, discussion on general and RRM requirement in RedCap UEs are provided and we have the following observation:

Observation 1: RAN4 has already supported a new rel-17 capability (namely SDT) with RedCap, hence, in the same manner RAN4 can further relax RLM/BFD based on RLM/BFD power saving relaxation.

Also, we have the following proposal:

Proposal 1: RAN4 to define further relaxations (e.g., introduce K values) to RLM/BFD measurements for a Rel-17 RedCap UE that is configured with and satisfies RLM/BFD relaxation criteria.

# References

- R4-2210592, “WF on RedCap RRM requirements”, Ericsson, May 2022.