3GPP TSG-RAN WG4 Meeting # 104-e 	R4-2213648

Electronic Meeting, August 15th - 26th, 2022

Agenda Item: 9.18.3.3

Source: MediaTek Inc.

Title: RRM measurements relaxation for stationary criterion

Document for: Discussion

# Introduction

In this contribution paper, the remaining issues of the RRM measurement relaxation for reduced capability (RedCap) on the Rel-17 specification are discussed. Where the agreements and open issues from the previous meeting are captured in the way forward (WF) in [1].

# Discussion on the remaining open issues

In this section, the RRM measurement relaxations are discussed for: (i) general aspects for RRM measurement relaxation for RedCap; (ii) RRM measurement relaxation for Redcap at Idle/Inactive state; and (iii) RRM measurement relaxation for Redcap at CONNECTED state.

# Discussion on general aspects for RRM measurement relaxation for RedCap

The remaining open issues from the previous meeting are the following:

Issue 2-1-1: Scenario to be considered for Rel-17 RRM relaxation for Redcap UE when both Rel-16 and Rel-17 criteria are configured No; Rel-16 relaxation criterion; Rel-17 relaxation criterion; Applicability. 7; Rel-16 low mobility; Rel-17 stationary; Allowed. 8; Rel-16 not-at-cell-edge; Rel-17 stationary; NO. 9; Rel-16 low mobility & Rel-16 not-at-cell-edge; Rel-17 stationary; Allowed. 10; Rel-16 low-mobility; Rel-17 stationary & Rel-17 not-at-cell-edge; Allowed. 11; Rel-16 not-at-cell-edge; Rel-17 stationary & Rel-17 not-at-cell-edge; Allowed. 12; Rel-16 low mobility & Rel-16 not-at-cell-edge; Rel-17 stationary & Rel-17 not-at-cell-edge; Allowed. Agreements from 2nd round: case 8 is discussed at the next meeting based on RAN2’s input

For Issue 2-1-1, case 8 is the only case that is left open for discussion and hence we provide our view for case 8 only. In previous meeting, RAN4 sent an LS to RAN2 to ask whether case 8 and 9 are allowed. RAN2 sent an LS response with the following answer:

Regarding the coexistence cases of Rel-17 and Rel-17 RRM relaxation: - Simultaneous configuration of R16 not-at-cell-edge criterion and R17 stationary criterion for idle/inactive mode is a valid configuration from the network’s point of view, where the network supports RRM relaxation for both R16 and R17 UEs in idle/inactive mode. - From signalling’s point of view, any R16 RRM relaxation criterion and any R17 RRM relaxation criterion for idle/inactive mode can be configured in a same cell at a same time, as independent criteria (i.e., without requiring a UE to fulfil both the R16 and the R17 criteria in order to relax its RRM measurements). - If combined with a not-at-cell-edge criterion (i.e. for Rel-17 stationary & Rel-17 not-at-cell-edge), the R17 stationary criterion can only be combined with the R17 not-at-cell-edge criterion, not with the R16 one. In this way, RAN2 kindly request RAN4 to consider support cases#8 and case #9 in the previous LS [R2-2204487/R4-2207109]. But it is up to RAN4 to make the final decision on whether support case#8 and case#9, for example, considering other reasons from RAN4 (if any).

To our understanding, rule 3 is to prevent mixing of rel-17 stationary criterion with rel-16 not-at-cell edge criterion in the same requirements. However, rule 3 doesn't prevent the NW from configuring rel-16 relaxation criterion with rel-17 relaxation criterion at the same time, which is the same understanding of RAN2. Hence, case 8 shall be allowed.

- Rel-16 low not-at-cell-edge and Rel-17 stationary (i.e. case 8) are allowed to be configured together.

# Discussion on the RRM measurement relaxation for Redcap at Idle/Inactive state

The remaining open issues from the previous meeting are the following:

Issue 2-2-2: Principle on RRM relaxation for eDRX with PTW - Proposals - Option 1: No measurement relaxation when UE is configured with eDRX cycles above 10.24 seconds (Huawei CMCC xiaomi oppo) - Option 2: Introduce intra-/inter-frequency and inter-RAT E-UTRAN neighbour cell measurement relaxations for eDRX cycles with PTW up to eDRX cycle length of 163.84 s, whereby the UE applies the scaling factor (6) based on measurement occasions across PTWs but does not measure across PTW’s. (Ericsson Nokia) - Option 3: No need to support “The relaxed RRM measurement period for PHY filtering shall not cross different PTW windows” (Ericsson MTK) - Option 4: If Rel-16/17 relaxed measurement requirements can be applied with eDRX cycles greater than 10.24 seconds (with PTW), the relaxed RRM measurement period for PHY filtering shall not cross different PTW windows (Apple MTK) - Option 5: The UE is allowed to apply the agreed relaxation factor (6) for eDRX cycle up to [20.48 sec] while the UE does measurement without relaxation within PTW. When the UE has met two relaxation criteria while configured with eDRX cycles ≥ 20.48, the UE is allowed to meet the relaxed measurement requirements which allows the UE to skip the measurement up to 4 hours (Ericsson MTK) - Option 6: Based on principle of option 4, relax RRM measurement using a scaling factor 6 providing the PTW length is sufficient; otherwise no relaxation; when both criteria are satisfied, skipping measurement for 4 hours. (vivo) Agreement (GTW): - Relax RRM requirements for eDRX with PTW Agreement (GTW): - For eDRX with PTW - For the single relaxation criterion, if Rel-16/17 relaxed measurement requirements can be applied with eDRX cycles greater than 10.24 seconds (with PTW), the relaxed RRM measurement period for PHY filtering shall not be cross different PTW windows - Scaling factor is 6 - FFS on the conditions that the scaling factor applies. - For two relaxation criteria, when the UE has met two relaxation criteria, the UE is allowed to reuse the fixed long measurement period of 4 hours. - For eDRX without PTW - For two relaxation criteria, when the UE has met two relaxation criteria, the UE is allowed to reuse the fixed long measurement period of 4 hours. The following options are available after 2nd round discussion - Option 1: Introduce intra-/inter-frequency and inter-RAT E-UTRAN neighbour cell measurement relaxations for eDRX cycles with PTW up to eDRX cycle length of 163.84 s, whereby the UE applies the scaling factor (6) based on measurement occasions across PTWs but does not measure across PTW’s. (Ericsson Nokia) - Option 2: Based on principle of option 4 (the relaxed RRM measurement period for PHY filtering shall not cross different PTW windows), relax RRM measurement using a scaling factor 6 providing the PTW length is sufficient. (vivo) - Option 2a: The scaling factor applies only when the relaxed evaluation/measurement time with such scaling factor on one carrier is not greater than single PTW window length. (Apple) - Option 3: For FR1, do not need a condition. For FR2, option 2a (MTK). Agreement (2nd round): No consensus on the conditions when the scaling factor 6 applies Issue 2-2-3: Higher priority inter-frequency measurement Relaxation when only Rel-17 stationarity criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ - Proposals: - Option 1: The inter-frequency measurement relaxation could use measurement period of 4 hours, when only Rel-17 stationarity criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. (Apple Xiaomi vivo oppo) - Option 2a: measurement at least every K*Thigher_priority_search where Thigher_priority_search is described in clause 4.2B.2.7 and, K = 60*2 = 120 regardless of whether 1 Rx or 2 Rx RedCap is configured with eDRX_IDLE cycle or not () - Option 2b: when only Rel-17 stationarity criterion is satisfied and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ or both Rel-17 criteria are satisfied, RRM relaxation for higher priority frequency could be based on the same methodology used by Rel-16 UE power saving, i.e., based on K4*Thigher_priority_search where K4 = 4*K2 = 240. (vivo) - Option 3: When only Rel-17 stationary criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE measures higher priority inter-frequency/inter-RAT layers at least every 1 hour. When both Rel-17 criteria are satisfied and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE measures high priority inter-frequency/inter-RAT layers at least every 2 hours. (CMCC) - Option 4: LS response from RAN2 on the Inter-frequency measurement Relaxation when only Rel-17 stationarity criterion is met shall be captured in RAN4 specification. (Nokia MTK) - Option 5: Case 1: only stationary relaxation criterion is fulfilled, For this case, we support option 2b, 3 and 5. Case 2: multiple relaxation criteria are fulfilled, For this approach we support option 3 or fixed period of 4 hours (as agreed for lower/equal priority frequencies). (Ericsson) - Option 6: There is no specification impact due to measurement on higher priority inter-frequency layers as the same relaxation method applies as for equal and lower priority inter-frequency layers. (Nokia MTK) - Option 7: The requirement shall be 60s*Nlayer. (the conclusion in RAN2 LS [R2-2203555], i.e., RRM relaxation for higher priority frequency would be performed similar to equal/lower priority frequency, is not aligned with RAN4 R16) (Huawei) Agreement (2nd round): No consensus

For Issue 2-2-2,

The new eDRX requirements are up to 10485.76 s, which is very relaxed compared to both general 5G NR DRX requirements and relaxed DRX requirements for Rel-16/17 RRM relaxation. Hence, designing new eDRX requirements for these criteria may not be necessary. However, the new requirements for eDRX starts from 2.56 s and the requirements for the 2.56 s are expected to be the same as the general requirements for DRX = 2.56 s. Furthermore, the relaxed DRX requirements for low mobility and not-at-cell-edge criteria for DRX = 2.56 s are relaxed by a factor K1 = 3 and stationary for K2 = 6, hence the requirements of DRX cycle = 2.56 s for Rel-16/17 RRM relaxation are more relaxed compared to the low eDRX cycles = 2.56 s and 5.12 s. Therefore, in order to have the RAN4 requirement consistent with all the DRX and eDRX requirements there should be a new eDRX requirements for the Rel-16/17 RRM relaxation for low eDRX cycles with PTW. Yet, the new relaxed eDRX should not be higher than the next eDRX cycle, for example, the relaxed eDRX cycle =20.96 s should not be more relaxed than the eDRX cycle = 41.92 s. In order to ensure this, different scaling factors can be introduced for different eDRX with PTW, where for larger the eDRX with PTW have smaller scaling factor. For example, eDRX = 20.48s can apply scaling factor = 6 for the stationary criterion, while for 40.96s the scaling factor can equal to 5 for the stationary criterion, so on. Thus, the RRM relaxation is applied for the lower eDRX values while the largest ones are kept the same.

- The existing relaxed DRX cycle = 2.56 s for low mobility and not-at-cell edge criteria can be more relaxed compared to the new eDRX requirements, hence there should be new relaxed eDRX to resolve that issue.

- The new eDRX requirements are up to 10485.76 s (i.e. already very relaxed) hence there is no need for further relax the high values of eDRX with RRM relaxation.

- Support the design of new relaxed eDRX for Rel-16/17 RRM relaxation for low eDRX cycles with PTW.

- The scaling factor applies only when the relaxed evaluation/measurement time with such scaling factor on one carrier is not greater than single PTW window length.

- Different scaling factor can be applied for different eDRX with PTW, where the larger the eDRX with PTW the smaller the scaling factor.

For Issue 2-2-3, From RAN4 Rel-16 specification: if the following condition is satisfied ‘When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ’ then the action is dependent on what criteria have been fulfilled. For low mobility criterion, the UE measures for high priority every K2*Thigher_priority_search, where K2 = 60 based on whether the UE is configured with highPriorityMeasRelax or not. However, for not-at-cell-edge criterion, and low mobility and not-at-cell-edge criterion the UE measures for high priority every K2*Thigher_priority_search, where K2 = 60 regardless of whether the UE is configured with highPriorityMeasRelax or not. Now, for Rel-17 RedCap requirements for stationary criterion and stationery and not-at-cell-edge criterion RAN4 sent an LS to RAN2 for their feedback and RAN2 has responded to RAN4 LS as:

Regarding higher priority inter-frequency measurement relaxation when only Rel-17 stationarity criterion is satisfied and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ or both Rel-17 criteria are satisfied, RAN4 kindly ask RAN2’s guidance on what is RAN2’s methodology designed for this scenario, if there is any. RAN2 concluded there is no need to introduce an indication, similar as highPriorityMeasRelax in Rel-16, to control the RRM relaxation of higher priority frequency. When the criteria are satisfied, e.g. only Rel-17 stationarity criterion is satisfied and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ or both Rel-17 criteria are satisfied, RRM relaxation for higher priority frequency would be performed similar to equal/lower priority frequency. The detailed relaxation methods are up to RAN4.

From RAN2 response, it is clear that for any fulfilled criterion the requirements shall be regardless of the priority flag (highPriorityMeasRelax). Therefore, the requirements for both stationary criterion and stationary and not-at-cell edge criterion when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, shall be measured for the high priority at all times. In addition, the factor K2= 60 from Rel-16 is selected so that the minimum time for measurements is in every 1 hour. Now, given that in Rel-17 the agreement is to have a minimum of 4 hours, hence, RAN4 shall follow the same methodology and update the factor to be K4 = 240.

- For inter-frequency measurement relaxation RAN4 shall define the following for both stationary criterion and stationary and not-at-cell edge criterion:

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-frequency layers of higher priority at least every K4*Thigher_priority_search where Thigher_priority_search is described in clause 4.2B.2.7 and K4=240.

# Discussion on RRM measurement relaxation for RedCap at CONNECTED mode

The remaining open issues from the previous meeting are the following:

Issue 2-3-4 RRM measurement relaxation in SDT - Proposals - Option 1: RRM relaxation should not be applied during SDT period in inactive mode (Apple) - Option 2: RRM relaxation is not excluded for SDT (Ericsson, Nokia, CMCC, MTK) Agreement (2nd round): No consensus

For Issue 2-3-4, Regarding the requirements for the SDT for RedCap with RRM relaxation we refer to the formula of timing advanced validation requirements in SDT for RedCap, which is (min(640ms, M1*TDRX)). From the formula it is clear that if we substitute K3*TDRX (where K3 = 3 for low mobility and K3 = 6 for stationary) instead of TDRX the output will always be equal to 640ms. Therefore, the motivation to include a new formula with K3*TDRX is not clear to us. However, from RAN4 requirements perspective it should be fine to support RRM relaxation with SDT for RedCap with TA validation requirements is given as a constant value = 640ms.

- The TA validation requirements for SDT for RedCap with RRM relaxation is always equal to 640ms.

- RAN4 can define TA validation requirement as a single value = 640ms for SDT in RedCap with RRM relaxation.

# Summary

In this contribution, the RRM measurement relaxation for RedCap are discussed and hence we have the following observation and proposals:

Proposal 1: Rel-16 low not-at-cell-edge and Rel-17 stationary (i.e. case 8) are allowed to be configured together.

Observation 1: The existing relaxed DRX cycle = 2.56 s for low mobility and not-at-cell edge criteria can be more relaxed compared to the new eDRX requirements, hence there should be new relaxed eDRX to resolve that issue.

Proposal 2: The new eDRX requirements are up to 10485.76 s (i.e. already very relaxed) hence there is no need for further relax the high values of eDRX with RRM relaxation.

Proposal 3: Support the design of new relaxed eDRX for Rel-16/17 RRM relaxation for low eDRX cycles with PTW.

Proposal 4: The scaling factor applies only when the relaxed evaluation/measurement time with such scaling factor on one carrier is not greater than single PTW window length.

Proposal 5: Different scaling factor can be applied for different eDRX with PTW, where the larger the eDRX with PTW the smaller the scaling factor.

Proposal 6: For inter-frequency measurement relaxation RAN4 shall define the following for both stationary criterion and stationary and not-at-cell edge criterion:

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-frequency layers of higher priority at least every K4*Thigher_priority_search where Thigher_priority_search is described in clause 4.2B.2.7 and K4=240.

Observation 2: The TA validation requirements for SDT for RedCap with RRM relaxation is always equal to 640ms.

Proposal 7: RAN4 can define TA validation requirement as a single value = 640ms for SDT in RedCap with RRM relaxation.

# References

- R4-2210596, “WF on eDRX and RRM measurement relaxations requirements for Redcap UE”, vivo.