3GPP TSG-RAN WG4 Meeting #104-e 	R4-2213650

Electronic Meeting, August 15th - 26th, 2022

Agenda Item: 13.1.4

Source: MediaTek inc.

Title: BWP operation without BW restrictions for Non-RedCap UEs

Document for: Discussion and decision

# Introduction

In the previous meeting, RAN4 received an LS document from RAN2 [1], in which, RAN2 asked RAN4 two questions on the BWP operation without bandwidth restriction, as given below:

Question 1: Whether it is a valid scenario in the standard to support the operation of BWP without SSB where the UE does not perform BM/RLM/BFD due to the lack of necessary reference signal (SSB and CSI-RS) in the active BWP. Question 2: If the answer to question 1 is that this is not valid, how should the UE perform BM/RLM/BFD when the active BWP does not contain SSB.

The above questions were discussed in Meeting#103-e and tentative agreement for question 1 is captured below [2]:

Answer to Q1 that the concerned scenario is invalid from RAN4 requirements point of view.

While the agreement on question 2 is given in the way forward (WF) as [3]:

Agreement: - RAN4 is to inform RAN2 that RAN4 requirements are defined only for the case when the target RS (SSB or CSI-RS) to perform BM/RLM/BFD is contained within the UE active BWP - FFS whether it would be feasible for the UE to perform BM/RLM/BFD on RSs configured outside the active BWP - Companies should provide analysis on the feasibility of performing BM/RLM/BFD on RSs that are not contained within the active BWP - RAN4 feasibility study focused on the response to RAN2 LS without any update on RAN4 specifications

Furthermore, from the previous RAN plenary meeting (Meeting#96) the following was agreed [4]:

To task the relevant Working Groups (RAN 1, 2, 4) to make progress on their discussions related to the RAN 2 LS in R2-2204009, aim to ensure that Feature Group 6-1a “bwp-WithoutRestriction” works in an early implementable form in R18, or, possibly R17, and report progress to RAN #97.

Therefore, in this contribution paper, we provide analysis and discussion on the feasibility of performing BM/RLM/BFD on reference signals that are not contained within the active BWP. Then, the LS response to RAN2 LS if given at the end of this document, hence RAN4 can send the LS in a timely manner and to report the progress to RAN#97.

It should be noted that the discussion, analysis and solution in this contribution paper are for non-RedCap UEs, where RedCap UEs is out of scope of this LS discussion.

- RAN4 discussion, analysis, solution, and LS response shall focus solely on non-RedCap UEs where RedCap UEs discussion is out of scope.

# Discussion

In order to assess the feasibility of performing measurements without SSB in the active BWP, we assess the available reference signals, which are CSI-RS within active BWP and the SSB outside the active BWP. Then, discussion on the UE implementation is provided. After that, discussion and solution on what existing requirements can be reused is given.

# Measurements based on CSI-RS

Based on the requirements in RAN4 specification (TS 38.133), the BM/RLM/BFD measurements can be performed based on SSB and CSI-RS contained within the BWP. Now, given that the discussion is for BWP without SSB, hence, we focus first on other reference signals within the active BWP, which is CSI-RS. For BM/RLM/BFD based CSI-RS in FR1 the existing requirements in RAN4 can be directly applied with no additional changes is needed to perform the required measurements for the case of active BWP without SSB.

- RAN4 requirements for BM/RLM/BFD based CSI-RS in FR1 can be directly applied for BWP without SSB.

While for BM/RLM/BFD based CSI-RS in FR2 the existing requirements in RAN4 is dependent on the existing of SSB in the active BWP to provide the QCL information. This can be observed from the CSI-RS FR2 requirements where beam sweeping scaling factor (N) is equal to 1, which is not the case for the BM/RLM/BFD based SSB requirements where N = 8. Thus, it is clear that the QCL information has to be obtained using SSB signals in FR2 before using CSI-RS in FR2.

- RAN4 requirements for BM/RLM/BFD based CSI-RS in FR1 can not be directly applied for BWP without SSB because it is dependent on SSB signals to acquire QCL information.

- RAN4 shall not rely on CSI-RS for BM/RLM/BFD in FR2 for the BWP operation without SSB.

# Measurements based on SSB outside the active BWP

The existing RAN4 requirements specify that the UE is not required to perform BM/RLM/BFD outside the active BWP. Hence, in order to perform BM/RLM/BFD outside the active BWP RAN4 need to discuss the possible UE implementation to perform measurements outside BWP. Based on our analysis, two possible UE implementations can be used to perform measurement outside active BWP, which are: (i) the UE always operate in wideband bandwidth to cover both the active BWP and the SSB outside the active BWP; (ii) using interruption (i.e., small gap) to enlarge the UE BW to cover both active BWP and SSB outside active BWP at the time of performing BM/RLM/BFD (i.e., measuring SSB symbols).

# Using large BW at all times

One straight forward UE implementation is to use large BW that cover both the active BWP and the SSB outside that BWP, hence, the UE can perform BM/RLM/BFD without any interruption or gaps. Yet, this means the UE BW can be up to 100MHz for FR1 regardless of the active BWP size. Therefore, using large BW at all time comes at the cost of high-power consumption, which results in having UE with least power efficiency compared to the other implementation described in 2.2.2.

- One possible UE implementation to perform BM/RLM/BFD outside the active BWP is using large RF and baseband bandwidth to cover both the active BWP and the SSB outside that BWP.

- Using large BW to perform BM/RLM/BFD outside the active BWP comes at the cost of high-power consumption (i.e., reduced UE power efficiency).

# Using small gap to allow the UE to enlarge the BW

Another UE implementation to perform BM/RLM/BFD outside the active BWP is to enlarge the BW of the UE at the time of performing SSB measurement outside the active BWP (i.e., the UE operates in narrow band, which is the BWP size, for the non-SSB symbols then enlarge the BWP at the time when measuring SSB during the SSB symbols). Given that enlarging the UE BW require enlarging the UE RF BW, hence, some interruption is needed before and after the UE BW change. In other words, a small gap is needed for the UE to reconfigure the RF/BB BW. The main advantage of this UE implementation is having a UE with high power efficiency compared to the scenario of using large BW at all time (i.e., the UE implementation in previous subsection 2.2.1). Nevertheless, existing RAN4 requirements do not cover the UE to perform BM/RLM/BFD outside the active BWP with small gaps, yet. Therefore, RAN shall introduce a new optional capability to allow the UE to perform BM/RLM/BFD based on the SSB outside the active BWP with a small gap. Also, RAN4 needs to decide which existing requirements can be used as a baseline for the small gap, hence, we provide our analysis and conclusion in the next subsection.

- With a small gap, UE can perform RF/BB reconfiguration and achieve most UE power saving to measure SSBs outside its active BWP.

- For non-RedCap UEs, introduce a new optional Rel-18 UE capability for SSB-based RLM/BFD/CBD/BM using SSBs outside active BWP with a small gap.

In addition, for the scenario of interest, i.e. narrow BWPs being configured for UE power saving, UE is likely to be in a low traffic mode without SCells or with dormant SCells. Where, the dormant BWP in a dormant SCell can be configured to contain CD-SSB. Hence, no need to apply the new UE capability to SCells.

- When UE is configured with narrow BWP for power saving on PCell/PSCell, its traffic load is very likely to be low and its SCells, if any, are likely to be deactivated or dormant.

- The support for SSB-based L1 measurements using SSBs outside active BWP is applicable to PCell/PSCell only.

# Which requirements to reuse for the small gap

The existing capabilities that their requirements could be reused to perform BM/RLM/BFD based on SSB outside the active BWP are (i) Rel-16 NeedForGap, and (ii) Rel-17 NCSG. In here, we assess these two capabilities based on the completeness of their existing requirements. Both capabilities have discussed the scenario where the UE perform RRM measurements using SSB outside its active BWP. Nevertheless, it is ambiguous whether interruption is needed for SSB-based RRM measurements without gaps in R16 NeedForGap. This is clear from the following WID objective specified in RAN4 Rel-18 WID [5]:

(2) Define RRM requirements for measurement without gaps for the following cases - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed - Define related requirements, such as CSSF, measurement period, scheduling restriction etc.

This is a clear indication that RAN4 specification is not completed yet for Rel-16 NeedForGap and there will be a long discussion before reaching consensus between all companies, which leads to some uncertainty to close the current BWP without SSB issue on time. Besides, as we discussed earlier in the previous subsection, interruption (in the form of small gap) is needed at the time when the UE enlarge the RF BW to measure the SSB symbols. Therefore, RAN4 can not proceed with Rel-16 NeedForGap due to its current uncertainties.

- Specification of R16 NeedForGap is incomplete and remaining work is to be discussed in Rel-18 RAN4 WI.

- Coupling Rel-16 NeedforGap with L1 measurement for BWP without restriction requires long and controversial discussion in RAN4.

On the other hand, Rel-17 NCSG already allows UE to report whether interruptions are needed, hence, there is no need to discuss the issue of interruption if we leverage the use of Rel-17 NCSG. In addition, the L3 measurement requirements for NCSG have been already completed in Rel-17 and there is already a foundation for RAN4 to further consider L1 measurements. Therefore, RAN4 can leverage Rel-17 NCSG to design a new non-RedCap UE capability to perform BM/RLM/BFD measurements based on SSB outside of the UE active BWP.

- Specification changes are minimized, and controversial discussions can be avoided, if we leverage R17 NCSG to support L1 measurement for BWP without restriction.

- For non-RedCap UEs, RAN4 shall introduce a new optional Rel-18 UE capability to indicate the support for SSB-based RLM/BFD/BM on SpCell (i.e. PCell or PSCell) using SSBs outside active BWP with a small gap.

- Reuse the UE capability reporting framework from Rel-17 NCSG for the new capability of SSB-based RLM/BFD/BM using SSBs outside active BWP.

# LS response to LS R2-2204009

- RAN4 shall respond to LS R2-2204009 with the following response:

1. Overall Description: RAN4 has discussed the LS R2-2204009, and have reached the following answer for the RAN2’s questions: Question 1: Whether it is a valid scenario in the standard to support the operation of BWP without SSB where the UE does not perform BM/RLM/BFD due to the lack of necessary reference signal (SSB and CSI-RS) in the active BWP. RAN4 Reply: From a RAN4 specification point of view, the scenario in which UE does not perform BM/RLM/BFD is not valid. Question 2: If the answer to question 1 is that this is not valid, how should the UE perform BM/RLM/BFD when the active BWP does not contain SSB. RAN4 Reply: RAN4 requirements are defined only for the case when the target RS (SSB or CSI-RS) to perform BM/RLM/BFD is contained within the UE active BWP. From a UE implementation point of view, it would be possible for the UE to perform BM/RLM/BFD outside the active BWP if a small gap is allowed for non-RedCap UEs. Therefore, RAN4 has the following agreements: Agreement 1: For non-RedCap UEs, RAN4 shall introduce a new optional Rel-18 UE capability to indicate the support for SSB-based RLM/BFD/BM on SpCell (i.e. PCell or PSCell) using SSBs outside active BWP with a small gap. Agreement 2: Reuse the UE capability reporting framework from Rel-17 NCSG for the new capability of SSB-based RLM/BFD/BM using SSBs outside active BWP. 2. To RAN WG2 group. ACTION: RAN4 kindly ask RAN2 to take the above into consideration.

# Summary

In this contribution, discussion on the response to RAN1 and RAN2 LS are provided, and we have the following observations and proposals:

Proposal 1: RAN4 discussion, analysis, solution, and LS response shall focus solely on non-RedCap UEs where RedCap UEs discussion is out of scope.

Observation 1: RAN4 requirements for BM/RLM/BFD based CSI-RS in FR1 can be directly applied for BWP without SSB.

Observation 2: RAN4 requirements for BM/RLM/BFD based CSI-RS in FR1 can not be directly applied for BWP without SSB because it is dependent on SSB signals to acquire QCL information.

Proposal 2: RAN4 shall not rely on CSI-RS for BM/RLM/BFD in FR2 for the BWP operation without SSB.

Observation 3: One possible UE implementation to perform BM/RLM/BFD outside the active BWP is using large RF and baseband bandwidth to cover both the active BWP and the SSB outside that BWP.

Observation 4: Using large BW to perform BM/RLM/BFD outside the active BWP comes at the cost of high-power consumption (i.e., reduced UE power efficiency).

Observation 5: With a small gap, UE can perform RF/BB reconfiguration and achieve most UE power saving to measure SSBs outside its active BWP.

Proposal 3: For non-RedCap UEs, introduce a new optional Rel-18 UE capability for SSB-based RLM/BFD/CBD/BM using SSBs outside active BWP with a small gap.

Observation 6: When UE is configured with narrow BWP for power saving on PCell/PSCell, its traffic load is very likely to be low and its SCells, if any, are likely to be deactivated or dormant.

Proposal 4: The support for SSB-based L1 measurements using SSBs outside active BWP is applicable to PCell/PSCell only.

Observation 7: Specification of R16 NeedForGap is incomplete and remaining work is to be discussed in Rel-18 RAN4 WI.

Observation 8: Coupling Rel-16 NeedforGap with L1 measurement for BWP without restriction requires long and controversial discussion in RAN4.

Observation 9: Specification changes are minimized, and controversial discussions can be avoided, if we leverage R17 NCSG to support L1 measurement for BWP without restriction.

Proposal 5: For non-RedCap UEs, RAN4 shall introduce a new optional Rel-18 UE capability to indicate the support for SSB-based RLM/BFD/BM on SpCell (i.e. PCell or PSCell) using SSBs outside active BWP with a small gap.

Proposal 6: Reuse the UE capability reporting framework from Rel-17 NCSG for the new capability of SSB-based RLM/BFD/BM using SSBs outside active BWP.

Proposal 7: RAN4 shall respond to LS R2-2204009 with the following response:

1. Overall Description: RAN4 has discussed the LS R2-2204009, and have reached the following answer for the RAN2’s questions: Question 1: Whether it is a valid scenario in the standard to support the operation of BWP without SSB where the UE does not perform BM/RLM/BFD due to the lack of necessary reference signal (SSB and CSI-RS) in the active BWP. RAN4 Reply: From a RAN4 specification point of view, the scenario in which UE does not perform BM/RLM/BFD is not valid. Question 2: If the answer to question 1 is that this is not valid, how should the UE perform BM/RLM/BFD when the active BWP does not contain SSB. RAN4 Reply: RAN4 requirements are defined only for the case when the target RS (SSB or CSI-RS) to perform BM/RLM/BFD is contained within the UE active BWP. From a UE implementation point of view, it would be possible for the UE to perform BM/RLM/BFD outside the active BWP if a small gap is allowed for non-RedCap UEs. Therefore, RAN4 has the following agreements: Agreement 1: For non-RedCap UEs, RAN4 shall introduce a new optional Rel-18 UE capability to indicate the support for SSB-based RLM/BFD/BM on SpCell (i.e. PCell or PSCell) using SSBs outside active BWP with a small gap. Agreement 2: Reuse the UE capability reporting framework from Rel-17 NCSG for the new capability of SSB-based RLM/BFD/BM using SSBs outside active BWP. 2. To RAN WG2 group. ACTION: RAN4 kindly ask RAN2 to take the above into consideration.

# References

- R2-2204009, “LS on BWP operation without bandwidth restriction”.

- R4-2210503, “Email discussion summary for [103-e][234] LS_reply”.

- R4-2211219, “WF on R17 gap coordination and BWP operation without BW restriction”.

- RP-221870, “NCD-SSB for all devices: proposed Way Forward”.

- RP-221018, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”.