3GPP TSG-RAN WG4 Meeting #104-e             	R4-2213662

Electrical Meeting, 15th Aug- 26th Aug, 2022

Agenda item:	9.24.6.1.2

Source:	Samsung

Title:	Simulation results for Rel-17 NB-IoT

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, it was agreed to introduce UL 16QAM requirement for NPUSCH in Rel-17 NB-IoT. The detail simulation assumption was discussed to facilitate the requirement definition. The related agreement was captured into the WF [1] as

- Number of allocated subcarriers - 12 tones - Number of scheduled RUs and TBS - (IRU, ITBS) = (0,15), TBS=280 bits - RV - 0,2,0,2 - Maximum number of HARQ transmission: 4 - Simulation assumption for NPUSCH format 1 (just for information) Parameter; Value. Number of antennas; 1T2R. Propagation condition; ETU 1Hz Low. SCS; 15KHz. Frequency Resource; 12 tones. Modulation order; 16QAM. Number of Repetition; 1. TB Scheduling; single-TB scheduling. Noise Estimation; Practical. Channel Estimation; Practical. Frequency offset; 0. Time offset; 0. (IRU, ITBS); (0,15). RV; {0,2,0,2}. Max number of HARQ transmission; 4.

In this contribution, the ideal and impairment simulation are provided for requirement derivation.

# 2	Simulation Results

In this subsection, we provide the ideal and impairment results as illustrated in table 1.

Table 1: simulation results for UL NPUSCH1 with 16QAM

| Antenna configuration | Propagation conditions | (IRU, ITBS) | Number of allocated subcarriers | SNR@70 of max TP (ideal) | SNR@70% of max TP (impairment) |
| --- | --- | --- | --- | --- | --- |
| 1T2R | ETU1 low | (0, 15) | 12 | 7.57 | 9.57 |

The related FRC for 16QAM is proposed as table 2

Table 2: FRC table for 16QAM

| Reference channel | A16-5 |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Number of allocated subcarriers | 12 |
| Diversity | No |
| Modulation | 16QAM |
| ITBS / IRU | 15 / 0 |
| Payload size (bits) | 280 |
| Allocated resource unit | 1 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.528 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 576 |
| Total symbols per resource unit | 144 |
| Channel estimation length (ms) Note 1 | 2 |

Proposal 1: RAN4 applies the following FRC for UL 16 QAM requirement.

| Reference channel | A16-x |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Number of allocated subcarriers | 12 |
| Diversity | No |
| Modulation | 16QAM |
| ITBS / IRU | 15 / 0 |
| Payload size (bits) | 280 |
| Allocated resource unit | 1 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.528 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 576 |
| Total symbols per resource unit | 144 |
| Channel estimation length (ms) Note 1 | 2 |

# 3	Conclusion

In this contribution, the simulation results with ideal and impairment are provided for requirement derivation.

Proposal 1: RAN4 applies the following FRC for UL 16 QAM requirement.

| Reference channel | A16-x |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Number of allocated subcarriers | 12 |
| Diversity | No |
| Modulation | 16QAM |
| ITBS / IRU | 15 / 0 |
| Payload size (bits) | 280 |
| Allocated resource unit | 1 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.528 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 576 |
| Total symbols per resource unit | 144 |
| Channel estimation length (ms) Note 1 | 2 |

# References

R4-2210674, WF on Rel-17 NB-IoT and eMTC performance requirements