3GPP TSG-RAN WG4 Meeting #104-e              	R4-2213665

Electrical Meeting, 15th Aug-26th Aug, 2022

Agenda item:	9.16.2.2

Source:	Samsung

Title:	Discussion and simulation results on PUCCH demodulation requirement for Rel-17 coverage enhancement

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, it was agreed to introduce PUCCH demodulation requirement with JCE. The detail simulation assumptions for each requirement were discussed to facilitate the requirement definition. The related agreement was captured into the WF [1]

In this contribution, the view on the remaining issue of test setup of PUCCH requirement for Rel-17 coverage enhancement is provided, and also the initial simulation result for PUCCH is provided.

# 2	PUCCH enhancements

In this section, the view on the remaining issue for PUCCH requirement is provided

DMRS configuration for PUCCH format 3

The following is the agreement in the last meeting

- Option 1: only cover without additional DMRS - Option 2: Cover both with and without additional DMRS - Make decision on this issue in the next meeting - Encourage companies to bring simulation results for both with and without additional DMRS for next meeting

PUCCH format 3 can support both with and without additional DMRS configuration. For Rel-15, PUCCH requirement with two configurations were specified.  In general, in our understanding, PUCCH is not the bottleneck of uplink, considering the payload is small with low modulation order. With additional DMRS configuration, the channel estimation performance is not the limit factor of PUCCH performance.

The purpose of JCE is to improve the channel estimation performance, while considering the multiple repetition gain, the PUCCH performance improvement due to JCE is minor. Therefore, we think it is reasonable to not configure additional DMRS for PUCCH with JCE. To reduce the test effort, we prefer to define PUCCH format 3 without additional DMRS configuration

Proposal 1: RAN4 defines PUCCH format 3 requirements with JCE only covering without additional DMRS

# 3	Initial simulation results

In this section, based on the agreed simulation assumption in the last meeting, the initial simulation results are provided for simulation alignment purpose.

Table 1. Ideal simulation results for PUCCH format1 with JCE

| Case | SCS | slot | Format | BW | PRB | Payload | Tx/Rx | Channel | Metric | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz SCS FDD | 8 | 1 | 5MHz | 1 | 2bits | 1T2R | TDLA30-10 low | ACK missed detection | -10.80 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -10.56 |
| 2 | 30KHz SCS FDD | 8 | 1 | 10MHz | 1 | 2bits | 1T2R | TDLA30-10 low | ACK missed detection | -10.67 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -9.93 |
| 3 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 | 1 | 5MHz | 1 | 2bits | 1T2R | TDLA30-10 low | ACK missed detection | -6.09 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -5.24 |
| 4 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 | 1 | 10MHz | 1 | 2bits | 1T2R | TDLA30-10 low | ACK missed detection | -6.25 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -4.85 |
| 5 | 60KHz SCS TDD (DDSUU, S=10D:2G:2U) | 2 | 1 | 50MHz | 1 | 2bits | 1T2R | TDLA30-75 low | ACK missed detection | -6.16 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -5.40 |
| 6 | 120KHz SCS TDD (DDSUU, S=10D:2G:2U) | 2 | 1 | 50MHz | 1 | 2bits | 1T2R | TDLA30-75 low | ACK missed detection | -6.21 |
|  |  |  |  |  |  |  |  |  | NACK to ACK detection | -5.81 |

Table 2. Ideal simulation results for PUCCH format3 with JCE

| Case | SCS | slot | Format | BW | PRB | Payload | Length | Tx/Rx | Channel | DMRS | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz SCS FDD | 8 | 3 | 5MHz | 1 | 16bits | 14 | 1T2R | TDLA30-10 low | Without additional DMRS | -4.93 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -5.07 |
| 2 | 30KHz SCS FDD | 8 | 3 | 10MHz | 1 | 16bits | 14 | 1T2R | TDLA30-10 low | Without additional DMRS | -4.91 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -4.96 |
| 3 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 | 3 | 5MHz | 1 | 16bits | 14 | 1T2R | TDLA30-10 low | Without additional DMRS | -0.51 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -0.66 |
| 4 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 | 3 | 10MHz | 1 | 16bits | 14 | 1T2R | TDLA30-10 low | Without additional DMRS | -1.22 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -1.29 |
| 5 | 60KHz SCS TDD (DDSUU, S=10D:2G:2U) | 2 | 3 | 50MHz | 1 | 16bits | 14 | 1T2R | TDLA30-75 low | Without additional DMRS | -1.09 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -2.35 |
| 6 | 120KHz SCS TDD (DDSUU, S=10D:2G:2U) | 2 | 3 | 50MHz | 1 | 16bits | 14 | 1T2R | TDLA30-75 low | Without additional DMRS | -0.44 |
|  |  |  |  |  |  |  |  |  |  | With additional DMRS | -1.05 |

# 4	Conclusion

In this contribution, the view on the remaining issue for test setup of PUCCH with JCE requirement was provided. Meanwhile, the initial simulation results are provided for alignment.

Proposal 1: RAN4 defines PUCCH format 3 requirements with JCE only covering without additional DMRS

# References

R4-2210667, WF on PUCCH demodulation requirement, Nokia