3GPP TSG-RAN WG4 Meeting #104-e              	R4-2213670

Electrical Meeting, 15th Aug-26th Aug, 2022

Agenda item:	9.21.3.1

Source:	Samsung

Title:	View on PUCCH demodulation requirement for Enhanced IIOT and URLLC support

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, the performance part of Enhanced IIoT and URLLC WI was under discussion. The general issue for test scope of BS demodulation was captured into the WF [1] as

- Do not introduce PUSCH requirements - Develop PUCCH requirements for PF0, with sub slot repetition and inter sub-slot hopping, but the condition introduction in the specification on the availability of at least 2 simulation inputs

In this contribution, the view on the requirement of PUCCH sub slot repetition is provided, and also the initial simulation result for PUCCH format 0 is provided.

# 2	PUCCH with sub slot repetition

In this section, the view on test setup for PUCCH requirement with sub-slot repetition is provided.

SubslotLengthfor PUCCH

This parameter is to indicate the sub-slot length for sub-slot based PUCCH feedback in number of symbols, where 2 or 7 symbols are available for normal CP scenario. sub-slot length with 7 is preferred

Proposal 1: RAN4 defines PUCCH sub-slot repetition requirement with subslotlenght as 7

Number of repetitions

Regarding the number of repetitions, it can be supported up to 8. From slot combination perspective, 2 repetition can fulfill the test purpose. Similar with multi-slot PUCCH requirement, we prefer to apply 2 repetitions for PUCCH requirement with sub-slot repetition.

Proposal 2: RAN4 defines PUCCH sub-slot repetition requirement with sub slot repetition as 2

Inter sub-slot hopping and First symbol

As agreed in RAN1, a PUCCH transmission in each of the repetition slots has a same first symbol, as provided by starting symbol index. Similar with Rel-15 PUCCH format 0 with 2 symbols cases, we prefer to 5 as the starting symbol index for each sub-slot as Figure 1.

Proposal 3: RAN4 defines PUCCH sub-slot repetition requirement with inter-slot hopping enable

- 5 as the starting symbol index for first sub-slot repetition

- 12 as the starting symbol index for second sub-slot repetition

Figure 1: inter sub-slot hopping and starting symbol index

Other simulation assumption

For other simulation assumption, we propose to reuse the assumption for Rel-15 PUCCH format 0 requirement with 2 symbols

Proposal 4: Reuse existing Rel-15 PUCCH format 0 requirement with 2 symbols for PUCCH requirement with sub-slot repetition.

# 3	Initial simulation results

In this section, the preferred simulation assumption is provided, and the initial simulation result is provided for simulation alignment purpose.

Table 1 simulation assumption for PF0 with sub slot repetition

| Parameter | Test |
| --- | --- |
| SCS/BW | 15KHz SCS/10MHz |
| Number of UCI information bits | 1 |
| SubslotLengthfor PUCCH | 7 |
| Number of repetition | 2 |
| Inter sub-slot hopping | Enabled |
| First PRB prior to frequency hopping | 0 |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of symbols | 2 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 5 for first sub-slot repetition 12 for second sub-slot repetition |

Table 2 Initial simulation results for PF0 with sub-slot repetition

| Number of Tx | Number of Rx | Format | Cyclic Prefix | Channel | Channel bandwidth | Metric | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 0 | Normal | TDLC-300-100 low | 15KHz 10MHz | ACK missed detection | 0.2dB |

# 4	Conclusion

In this contribution, the view on test setup for PUCCH requirement with sub-slot repetition.

Proposal 1: RAN4 defines PUCCH sub-slot repetition requirement with subslotlenght as 7

Proposal 2: RAN4 defines PUCCH sub-slot repetition requirement with sub slot repetition as 2

Proposal 3: RAN4 defines PUCCH sub-slot repetition requirement with inter-slot hopping enable

- 5 as the startingsymbol index for first sub-slot repetition

- 12 as the startingsymbol index for second sub-slot repetition

Proposal 4: Reuse existing Rel-15 PUCCH format 0 requirement with 2 symbols for PUCCH requirement with sub-slot repetition.

# References

R4-2210673, WF on Enhanced IIoT and URLLC support demodulation and CSI requirements, Nokia