3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213686

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	11.12.2

Source: 	ZTE Corporation

Title: 	Discussion on coexistence evaluation for ATG network

Document for:	Discussion

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. In this contribution, we want to share some initial views on coexistence study for ATG coexistence.

- Specify features to core specifications for ATG BS and UE [RAN4]

  - Scenario:

    - BS on the ground, and the CPE type of UE mounted in the aircraft

    - A direct radio link between BS on the ground and CPE type of UE mounted in the aircraft

    - Note: The deployment characteristics described in the justification section shall be taken as a basis for the technical discussion.

  - Specify core requirements for coexistence between ATG and IMT terrestrial network

    - Example bands include n1, n78 and n79.

    - Perform FR1 co-existence evaluation for ATG network (e.g. ACLR, ACS)

    - Identify key characteristics where it is necessary to differentiate ATG ground-based BS and UEs from conventional ground based BS and UEs

      - Aim to reuse existing requirements for BS and UE where possible, e.g.,

        - Reuse TN BS requirements for ATG BS

    - Specify RF requirements for ATG UE/BS

      - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects.

      - Taking into account identified differences between ATG and fully ground based systems

      - Consider BS type 1-C/1-H/1-O and specify the requirements

      - Consider conductive requirements for UE

    - Specify RRM core requirements for ATG UE

      - Taking into account identified differences between ATG and fully ground based systems

      - Considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects

    - Specify new UE/BS type(s) for ATG network if necessary

Discussion

## 2.1. Coexistence scenario

As indicated in the objective, there are three example bands requested for ATG network standardization in Rel-18 and its coexistence scenarios could be found in following Figure 2.1-1 for band n1, Figure 2.1-2 for band n78 and Figure 2.1-3 for band n79.

Figure 2.1-1. coexistence scenario for band n1

Figure 2.1.-2. coexistence scenario for band n78

Figure 2.1-3. coexistence scenario for band n79

Based on the above adjacent spectrum for ATG network deployment, then the coexistence scenario could be summarized as following.The aggressor and victim combinations are listed in Table 2.1-1.

Table 2.1-1: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with ATG | ATG DL | TN DL | Applicable for n1, n78 and n79 | Phase 1 |
| 2 | TN with ATG | ATG UL | TN UL | Applicable for n1, n78 and n79 | Phase 1 |
| 3 | TN with ATG | TN DL | ATG DL | Applicable for n1, n78 and n79 | Phase 1 |
| 4 | TN with ATG | TN UL | ATG UL | Applicable for n1, n78 and n79 | Phase 1 |
| 5 | TN with ATG | ATG UL | TN DL | Applicable for n1 coexisting with n39 and n33 | Phase 1 |
| 6 | TN with ATG | TN DL | ATG UL | Applicable for n1 coexisting with n39 and n33 | Phase 1 |

The frequency and bandwidth for ATG coexistence study are listed in Table 2.1-2.

Table 2.1-2: Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Rural | 4.9 GHz | 100MHz | TDD | 1 |
| TN Rural | 3.5 GHz | 100 MHz | TDD | 1 |
| TN Rural | 2.1GHz | 20 MHz | FDD | 1 |
| NOTE 1: Since aircraft route is mostly in the rural scenarios, therefore we propose to focus on the rural scenario for ATG coexistence study. |  |  |  |  |

Proposal 1: to consider the coexistence scenario in Table 2.1-1 and proposed frequency and bandwidth configuration in Table 2.1-2 for ATG coexistence study.

## 2.2. System parameters

### 2.2.1	ATG parameters

The general parameters for ATG network coexistence study are listed in the following Table 2.2.1-1.

Table 2.2.1-1: the general system parameters for ATG network

| ATG CPE altitude | 7km, 12 Km | 7km, 12 Km . | 7km, 12 Km |
| --- | --- | --- | --- |
| Carrier frequency | 4.9 GHz | 3.5 GHz | 2.1GHz |
| Frequency reuse factor | 1 | 1 | 1 |
| Duplex scheme | TDD | TDD | FDD |
| Channel bandwidth | 100 MHz | 100MHz | 20MHz |
| Subcarrier spacing (SCS) | 30 KHz | 30 KHz | 15 KHz |
| Number of cells | 1 | 1 | 1 |
| Frequency reuse | 1 | 1 | 1 |
| Environment1 | Rural | Rural | Rural |
| UE distribution | Single UE per ATG cell | Single UE per ATG cell | Single UE per ATG cell |
| Indoor UE percentage | 0% | 0% | 0% |
| Number of DL scheduled UEs per cell | 1 | 1 | 1 |
| Number of UL scheduled UEs per cell | 1 | 1 | 1 |
| DL scheduled bandwidth per UE | 100 MHz | 100 MHz | 20MHz |
| UL scheduled bandwidth per UE | 100 MHz | 100 MHz | 20MHz |
| DL power control | No | No | No |
| UL power control | See section 2.6.2 | See section 2.6.2 | See section 2.6.2 |
| ATG BS antenna | See section 2.3.1 | See section 2.3.1 | See section 2.3.1 |
| ATG BS noise figure | 5 dB | 5 dB | 5 dB |
| ATG BS to UE pathloss model | See section 2.5.1 | See section 2.5.1 | See section 2.5.1 |
| ATG UE assumption | See section 2.2.2 | See section 2.2.2 | See section 2.2.2 |
| NOTE 1: ATG BS is assumed to serve UEs in the rural environment. |  |  |  |

As shown in the following figure, only single ATG UE is supposed to be served by the ATG BS. And for the simplification of coexistence study of ATG network, ATG UE is supposed to be moving in the line of ATG BS deployment as shown in following Figure 2.2.1-1.

Figure 2.2.1-1 ATG network coexisting with terrestrial network [Rural area]

### 2.2.2	ATG UE parameters

ATG UE parameters are given in Table 2.2.2-1

Table 2.2.2-1: ATG UE characteristics for system level simulations

| Characteristics | CPE | CPE |
| --- | --- | --- |
| Frequency band | 2.1GHz | 3.5GHz and 4.9GHz |
| Antenna type and configuration | Omini antenna assumption | Antenna array assumption : (Note 1) (8x2x2) or (16x1x2) |
| Polarisation | Linear: +/-90°X-pol | Linear: +/-90°X-pol |
| Rx Antenna gain | 0dBi | 4-5dBi； |
| Antenna temperature | 290 K | 290 K |
| Noise figure | 9 dB | 9 dB |
| Tx transmit power | 33dBm per polarization; 36dBm for two polarization; | 26dBm per polarization |
| Tx antenna gain | 0dbi | 4-5dBi； |
| NOTE 1: 8 or 16 antenna elements are placed along the aircraft moving direction for the ease of the installation. |  |  |

Table 2.2.2-2: antenna array assumption for ATG UE at 3.5GHz and 4.9GHz

| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H [54º] for V /[60-70] |
| --- | --- | --- |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30dBc |
| 1.5 | Antenna polarization | Linear ±90º |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | (8x2x2) or (16x1x2) |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |

### 2.2.3	TN parameters

TN parameters for co-existence study are given in Table 2.2.3-1, 2.2.3-2 and 2.2.3-3.

Table 2.2.3-1: Simulation assumptions of TN respectively based on NR

|  | NR | NR |
| --- | --- | --- |
| Carrier frequency in GHz | 2.1GHz | 3.5GHz and 4.9GHz |
| Size of each nominal channel BW in MHz | 20 | 100 |
| Transmission bandwidth in MHz | N/A | N/A |
| Environment | Rural | Rural |
| Network layout | 19-sites 57 sectors with wrap-around | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | 7500m | 7500m |
| System loading and activity | NA | NA |
| Network location | See Figure 2.3.3-1 | See Figure 2.3.3-1 |
| DL subcarrier spacing | 15kHz | 30kHz |
| UL | OFDMA | OFDMA |
| DL power control | No | No |
| UL power control | TR 36.942[8] | TR 36.942[8] |
| Frequency reuse | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 |
| Number of scheduled UE per cell (UL) | 1 | 1 |
| UE antenna height in meter | 1.5m | 1.5m |
| UE TX power in dBm | -40 to 23 | -33 to 23 |
| UE antenna gain in dBi | 0 | 0 |
| Building penetration loss | In pathloss model, TR 38.901[10] | In pathloss model, TR 38.901[10] |
| Cell selection margin in dB | 3 | 3 |
| BS-MS min distance in meters | 35 | 35 |
| BS noise figure in dB | 5 | 5 |
| UE noise figure in dB | 9 | 9 |
| BS-UE path-loss model | TR 38.901[10] | TR 38.901[10] |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901[10] | Deployment scenario related, referring to TR 38.901[10] |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 | Inter-cell 0.5 Intra-cell 1 |
| Link-level performance model | See Section 6.2.8 Throughtput-SINR mapping | See Section 6.2.8 Throughtput-SINR mapping |
| UE distribution | Uniform | Uniform |
| Evaluation metrics | See Section 6.2.8 Throughtput or SNR loss criteria | See Section 6.2.8 Throughtput or SNR loss criteria |

Table 2.2.3-2: Deployment-related parameters of TN

|  | Urban Macro | Rural Macro | Remarks |
| --- | --- | --- | --- |
| ISD in meters | NA | 7500 | ITU-R Report M.2292[11] |
| BS Antenna height in meters | NA | 30 |  |
| UE Outdoor/indoor | 100% Outdoor |  |  |
| UE height in meter | NA | 1.5 | RP-200559 [12] 3GPP LS to ITU-R WP5D and ITU-R WP5D [IMT_Parameters] [13] |

Table 2.2.3-3: ACLR/ACS for TN

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | ACS | 33 |

As shown in the following figure 2.3.3-1, since ISD of ATG BS is relatively large and only one ATG CPE is active in one time. Therefore we only need to consider the TN BS around ATG BS as shown in the following Figure.

Figure 2.3.3-1. ATG network coexisting with terrestrial network [Rural area]

### 2.2.4	TN UE parameters

TN UE parameters are given in Table 2.2.4-1

Table 2.2.4-1: TN UE characteristics for system level simulations

| Characteristics | UE |
| --- | --- |
| Frequency band | 2.1GHz, 3.5GHz and 4.9GHz |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 9 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

## 2.3.  Antenna and beamforming pattern modelling

### 2.3.1	ATG BS and UE antenna and beam forming pattern modelling

For AAS antenna for ATG BS, it refers to the pattern in TR 38.921.

Table 2.3.1-1: AAS antenna parameters for ATG BS at 2.1GHz, 3.5GHz and 4.9GHz

|  |  | Rural |  |
| --- | --- | --- | --- |
| 1 | ATG BS Antenna Characteristics |  |  |
| 1.1 | Antenna pattern | TR 38.921[14] |  |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |  |
| 1.5 | Antenna polarization | Linear ±45º |  |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements |  |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |  |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 |  |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 |  |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 |  |
| 1.12 | Mechanical up-tilt (degrees) | 20 degree 50-70km ISD 5.7 degree 100km ISD TBD degree for 200km ISD |  |

### 2.3.2	TN BS and UE antenna and beam forming pattern modelling

For AAS antenna of TN BS, it refers to the pattern in TR 38.921.

Table 2.3.2-1: AAS antenna parameters for TN BS at 2.1GHz, 3.5GHz and 4.9GHz

|  |  | Rural |  |
| --- | --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |  |
| 1.1 | Antenna pattern | TR 38.921[14] |  |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |  |
| 1.5 | Antenna polarization | Linear ±45º |  |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements |  |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |  |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 |  |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 |  |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 |  |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 |  |
| 1.12 | Mechanical downtilt (degrees) | 3 |  |

For non-AAS antenna of TN BS, the parameters in Table 2.3.2-2 are used for TN BS antenna pattern in the ATG coexistence simulation.

Table 2.3.2-2: FR1 BS Non-AAS antenna pattern for TN BS at 2.1GHz, 3.5GHz and 4.9GHz

| Parameter for BS | Values |  |
| --- | --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=10°,SLAV=20dB |  |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |  |
| Combining method for 3D antenna pattern (dB) | A“θ",φ”=-min-AE,Vθ"+AE,Hφ”, Am |  |
| Maximum directional gain of an antenna GE,max | 17 dBi |  |
| Conducted power | 46 dBm |  |
| Mechanical downtilt in degrees | Rural | 3 |
|  | Urban | NA |

## 2.4. ACIR model

ACLR modelling for TN and ATG co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline.

## 2.5. Propagation model

### 2.5.1 Propagation model between ATG BS and UE

Propagation model between ATG BS and UE should reference to section 6.6 in TR 38.811[5].

### 2.5.2	Propagation model between TN BS and UE

Propagation model between TN BS and UE should reference to section 7.4 in TR 38.901[10].

### 2.5.3	Propagation model between ATG BS and TN BS

Propagation model between ATG BS and TN BS should reference to TS 38.811[5] which is used for DL-UL cross link interference for band n1.

## 2.6. Transmission power control model

### 2.6.1	TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942[8] is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = TBD dB,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

### 2.6.2	ATG UL TPC

For the coexistence study, the same TPC model of TN for ATG UL scenarios is adopted

### 2.6.3	ATG DL TPC

For downlink scenario, no power control scheme is applied.

## 2.7. Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

## 2.8. Performance metric

For NR, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For ATG, the average throughput loss and 5%-ile throughput loss should be less than 5%.

## 2.9. Throughput ~ SNR mapping

The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Where:

S(SNIR)	Shannon bound, S(SNIR) =log2(1+SNIR) [bps/Hz]
		Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX 	Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 2 represent a baseline case, which assumes:

- 1:1 antenna configurations

- AWGN channel model

- Link Adaptation (see table 2 for details of the highest and lowest rate codes)

- No HARQ

Table 2: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Conclusions

In this contribution, we shared some initial views on coexistence study for ATG coexistence and proposals are made as following:

Proposal 1: to consider the coexistence scenario in Table 2.1-1 and proposed frequency and bandwidth configuration in Table 2.1-2 for ATG coexistence study.

Proposal 2: to use the simulation assumptions in this contribution as baseline for further coexistence study.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.